- BSNL TTA Question Paper-Communication Specialization 2007

- (1) The most common modulation system used for telegraphy is

- (a) FSK

- (b) PSK.

- (c) PCM

- (d) single tone modulation

- (e) Two tone modulation

- (2)VSB is an abbreviation of vestigal sideband, is derived by filtering

- (a) DSB

- (b) AM

- (c) either (a) or (b)

- (d) PM

- (3) The Hartely law states that

- (a) The maximum rate of information transmission depends on the depth of modulation

- (b) The maximum rate of information depends on the channel bandwidth.

- (c) Only binary codes may be used

- (d) Redundancy is essential

- (4) The FM signal with, a modulation index mf is passed through a frequency tripler. The wave in the output of the tripler will have a modulation index of

- (a) mf/9

- (b) mf/3

- (c) mf

- (d) 3 mf

- (e) 9mf

- (5) In high power AM transmission, modulation is done at

- (a) Buffer stage

- (b) Oscillator stage

- (c) RF power stage

- (d) If stage

- (6) Companding is used

- (a) In delta modulator to combat noise

- (b) To limit amplitude in PCM transmitters.

- (c) In PWM for working it with TDM

- (d) To protect small signals in PCM from quantizing distortion

- (e) In PCM to reduce the SNR

- (7) In PCM system the SNR of the output signal increases

- (a) Inversely with bandwidth

- (b) Exponential with bandwidth

- (c) With rate of sampling

- (d) At low frequencies only

- (8) Armstrong modulator generates

- (a) Phase modulated signal

- (b) Frequency modulated signal

- (c) Both of these

- (d) Pulse code modulated signal

- (e) AM and PCM signals

- (9) A Klystron is a cavity acting as buncher and catcher is used as microwave tube for

- (a) Guiding waves

- (b) Velocity modulation

- (c) frequency modulation

- (d) impedance matching

- (e) All of these

- (10) Easily adjustable tuning component in a waveguide is

- (a) plunger

- (b) plunger and stub

- (c) screw

- (d) both (a) and (c)

- (e) both (b) and (c)

- (11) A ferrite is

- (a) A non-conductor with magnetic properties

- (b) A conductor with magnetic properties

- (c) A semiconductors

- (d) An insulator which attenuates magnetic fields

- (e) A compound with good conductivity.

- (12) Vacuum tubes eventually fail at microwave frequencies because of their

- (a) Inter electrode capacitance

- (b) Small series inductance

- (c) Large shunt capacitance

- (d) Short transit time

- (e) Increased noise figure

- (13) The biggest disadvantage the IMP ATT diode has is its

- (a) Low Efficiency

- (b) high noise

- (c) Low BW

- (d) inability to provide pulse operation

- (e) low power handling ability

- (14) In AM transmission the frequency, which is not transmitted is

- (a) carrier frequency

- (b) audio frequency

- (c) upper side frequency

- (d) lower side frequency

- (15) FM broadcast band lies in

- (a) VHF

- (b) UHF

- (c) SHF

- (d) HF

- (16) Automatic gain control is used

- (a) to maintain the tuning correct

- (b) to reduce the voltage of loud passage of music

- (c) to maintain the same amount of output, when stations of different strength are received

- (d) to increase the amplification at high frequencies

- (17) The modulation system inherently is most noise resistant in

- (a) SSB suppressed carrier

- (b) FM

- (c) PPM

- (d) PCM

- (18) In practical waveguide act as

- (a) low pass filter

- (b) high pass filter

- (c)band pass filter

- (d)band stop filter

- (19) The antenna efficiencies achieved in practice depend upon

- (a) wave length

- (b) impedance

- (c) frequency

- (d) none of above

- (20) The process of compressing the digital codes at the transmitter and then expanding them back to their original form at receiver is known as

- (a) Quantizing

- (b) companding

- (c) step sizing

- (d) modulation

- (21) Digital transmission efficiency is given by

- (a) information bits/total bits

- (b) total bits/information bits

- (c) redundant bits/information bits

- (d) none of the above

- (22) The speed of BRI ISDN interface is

- (a) 2B + D

- (b) 2D + B

- (c) 30B + D

- (d) 30D + B

- (23) Which of the following is not a microwave generation source?

- (a) Klystron

- (b) Magnetron

- (c) TWT

- (d) Diode

- (24) A signal of maximum frequency of 1 KHz is sampled at Nyquist Rate. The interval between two successive samples is:

- (a) 50 micro seconds

- (b) 100 micro seconds

- (c) 500 micro seconds

- (d) 1000 micro seconds

- (25) In order to get back the original signal, it is necessary to use:

- (a) low pass filter

- (b) high pass filter

- (c) band pass filter

- (d) band reject filter

- (26) Man made noise id caused by:

- (a) Solar eruptions

- (b) Distant Stars

- (c) Lightning Discharges

- (d) Arc discharge in electric machines

- (27) At microwave frequencies, the size of antenna becomes

- (a) very large

- (b) large

- (c) small

- (d) very small

- (28) Due to curvature of earth, microwave repeaters are placed at distance of about

- (a) l0 Km

- (b) 50 Km

- (c) 200 Km

- (d) 500 Km

- (29) For handling large microwave power, the best medium is

- (a) coaxial line

- (b) rectangular waveguide

- (c) stripline

- (d) circular wave guide

- (30) An attenuator is used with TWT to

- (a) prevent oscillations

- (b) increase gain

- (c) prevent saturation

- (d) help bunching

- (3l) TWT is basically

- (a) an oscillator

- (b) tuned amplifier

- (c) wideband amplifier

- (d) an audio amplifier

- (32) The negative resistance in Gunn diode is due to

- (a) electron transfer to a less mobile energy level

- (b) high reverse bias

- (c) electron domain formation at the junction

- (d) tunneling across the junction

- (33) Which of the following sinusoidal oscillator is preferred for microwave frequencies?

- (a) resonant circuit oscillator

- (b) RC phase shift oscillators

- (c) negative resistance oscillators

- (d) all of the above

- (34) When electromagnetic waves are propagated in a waveguide

- (a) they travel along the walls of the waveguide

- (b) they travel through the dielectric without touching the walls

- (c) they are reflected from the walls but do not travel along the walls

- (d) none of above

- (35) Wave guides are generally made of

- (a) Cast iron or steel

- (b) White metal or gun metal

- (c) bronze or aluminium

- (d) plastic or bakelite

- (36) The cut off frequency of a wave guide means

- (a) lower frequencies will not be propagated

- (b) it determines the dimensions of the wave guide

- (c) frequency at which zero transmission takes place

- (d) None of above

- (37) In case of matched load

- (a) Transmission is zero

- (b) reflection is zero

- (c) reflection is unity

- (d) transmission is equivalent to reflection

- (38) In an AM wave with 100% modulation, the carrier is suppressed. The percentage of power saving will be

- (a) 100%

- (b) 50%

- (c) 25%

- (d) 66.7%

- (39) The function of AM detector circuit is

- (a) to rectify the input signal

- (b) to discard the carrier

- (c) to provide the audio signal

- (d) All of the above

- (40) In FM, the noise can be further decreased by

- (a) decreasing deviation

- (b) increasing deviation

- (c) keeping deviation constant

- (d) none of these

- (41) In PPM, message resides in

- (a) Pulses

- (b) time location of pulse edges

- (c) none of these

- (42) Which of the following pulse systems is preferred for communication in presence of noise?

- (a) PAM

- (b) PDM

- (c) PPM

- (d) none of above

- (43) Which of the following pulse systems requires higher bandwidth

- (a) PAM

- (b)PDM

- (c)PPM

- (d)none of these

- (44) The audio frequency range lies between

- (a) 20 to 20,000 Hz

- (b) 20 to 20,000 KHz

- (c) 400 to 8,000Hz

- (d) 500 to 5,000 Hz

- (45) Maximum undistorted power output of a transmitter is obtained when its modulation is:

- (a) more than 100%

- (b) 100%

- (c) less than 100%

- (d) 50%

- (46) The AGC voltage in a radio receiver is proportional to

- (a) the amount of modulation

- (b) the amplitude of audio signal

- (c) the amplitude of IF carrier

- (d) none of these

- (47) An FM transmitter has maximum frequency deviation of 75 KHz and reproduces audio signal up to 15 KHz. Minimum channel width required, in KHz is

- (a) 180

- (b) 120

- (c) 90

- (d) 60

- (48) With 100% modulation, ratio of side band power to total power transmitted in an amplitude modulated wave is

- (a) 2/3

- (b) l/3

- (c) l/2

- (d) l/4

- (49) To increase the Q factor of an induction, it wound with

- (a) thicker wire

- (b) thinner wire

- (c) longer wire

- (d) wire with heavy insulation

- (50) Power factor of a purely resistive circuit is:

- (a) zero

- (b) one

- (c) 0.5

- (d) infinity