- BSNL TTA Question Paper Control Systems Specialization 2007

- 1. An open loop control system has its

- (a) control action independent of the output or desired quantity

- (b) controlling action, depending upon human judgment

- (c) internal system changes automatically taken care of

- (d) both (a) and (b)

- (e) all (a),(b) and ©

- 2. A servo system must have

- (a) feedback system

- (b) power amplifier to amplify error

- (c) capacity to control position or its derivative

- (d) all of these

- (e) none of these

- 3. The major disadvantage of a feedback system may be

- (a) Inaccuracy

- (b) inefficiency

- (c) Unreliability

- (d) instability

- (e) Insensitivity

- 4. Properties of a transfer function

- (a) It is ratio of two polynomials is S and assumes zero initial conditions

- (b) It depends on system elements and not input and output of the system

- (c) Coefficients of the powers of S in denominator and numerator are all real constant. The order of

- denominator is usually greater than or equal to the order of numerator

- (d) All of these

- (e) It is a function which transfer one physical system into another physical system.

- 5. The classical analogous of a simple lever is

- (a) Capacitor bridge

- (b) transformer

- (c) mutual inductor

- (d) either of these

- 6. Two blocks G1(s) and G2(s) can be cascaded to get resultant transfer function as

- (a) G1(s) + G2(s)

- (b) G1(s) / G2(s)

- (c) G1(s) G2(s)

- (d) 1+G1(s) G2(s)

- (e) 1-G1(s)G2(s)

- (f) two blocks cannot be cascaded

- 7. The principles of homogeneity and super position can be applied to

- (a) linear time invariant system

- (b) non-linear time invariant system

- (c) digital control system

- (d) both (a) and (b)

- 8. Pick up the nonlinear system

- (a) automatic voltage regulator

- (b) d.c. servomotor with high field excitation

- (c) temperature control of a furnaces using thermistor

- (d) speed control using SCR

- (e) all of these

- 9. Signal flow graph (SFG) is a

- (a) polar graph

- (b) semi log graph

- (c) log log graph

- (d) a special type of graph for analyzing modem control system

- (e) a topological representation of a set of differential equations

- 10. Disadvantages of magnetic amplifier

- (a) time lag, less flexible, non-sinusoidal waveform

- (b) low power consumption and isolation of the active circuit

- (c) saturation of the core

- (d) all of these

- 11. Pick up false statement regarding magnetic amplifiers

- (a) The gate coil of an ideal magnetic amplifier has either zero or infinite inductance

- (b) Resistance of control and gate winding is very small

- (c) Magnetic amplifier gas dropping load characteristics

- (d) Magnetic amplifiers are not used to control the speed of d.c. shunt motor

- (e) Magnetic amplifiers can be used in automatic control of electric drivers of higher rating.

- 12. High power amplification is achieved by using

- (a) push pull amplifier

- (b) amplidyne

- (c) magnetic amplifier

- (d) DC amplifier

- (e) D.C. Generator

- 13. Pick up false statement regarding servomotors

- (a) The d.c. servomotors are lighter than equivalent a.c. servomotors

- (b) The d.c. servomotors develops higher starting and reversing torque than equivalent a.c. servomotor.

- (c) A drag cup a.c. servomotor has one windings on stator and other on rotor

- (d) Output power of servomotors varies from 1/20 W to 100 W

- 14. To reduce steady state error

- (a) decrease natural frequency

- (b) decrease damping

- (c) increase damped frequency

- (d) increase time constant

- (f) increase gain constant of the system

- 15. A good factor for Mp should be

- (a) less than 1

- (b) lying between 1.1 and 1.5

- (c) more than 2.2

- (d) zero

- (e) infinity

- 16. Pick up false statement. Routh-Hurwitz criterion

- (a) is used for determining stability of a system

- (b) is an algebraic procedure

- (c) gives the exact location of roots of the characteristic equation

- (d) does not indicate relative degree of stability or instability

- 17. Which of the following is the time domain method of determining stability of a control system

- (a) Bode plot

- (b) Nyquist plot

- (c) Nicholos chart

- (d) Routh-Hurwitz array

- (e) Constant M and (fy) locus

- (f) Root locus technique

- 18. The technique which gives transient response quickly as well as stability information is

- (a) Nyquist plot

- (b) Routh-Hurwitz criteria

- (c) Bode plot

- (d) Root locus plot

- (e) Nichols plot

- 19. The bandwidth can be increased by use of

- (a) phase lag network

- (b) phase lead network

- (c) both (a) and (b) in cascade

- (d) both (a) and (b) in parallel

- (e) none of these

- 20. Nyquist plot is drawn on

- (a) semi log graph paper

- (b) log log graph paper

- (c) polar graph paper

- (d) centimeter graph paper

- 21. If the gain margin is positive and the phase margin is negative the system is

- (a) stable

- (b) unstable

- (c) indeterminist

- 22. The Bode plot is applicable to

- (a) all phase network

- (b) minimum phase network

- (c) maximum phase network

- (d) lag lead network

- (e) none of these

- 23. The valid relation between setting time ts and rise time tr is

- (a) tr>ts

- (b) ts>tr

- (c) ts=tr

- (d) none of these

- 24. As a root moves further away from imaginary axis the stability

- (a) increases

- (b) decreases

- (c) not affected

- (d) none of these

- 25. Flat frequency response means that the magnitude ratio of output to input over the

- bandwidth is

- (a) variable

- (b) zero

- (c) constant

- (d) none of above

- 26. How many octaves are between 200 Hz and 800 Hz

- (a) Two octave

- (b) One octave

- (c) Four octave

- (d) None of above

- 27. Human system can be considered as

- (a) open loop system

- (b) close loop system with single feedback

- (c) close loop system with multi variable feedback

- (d) none of these

- 28. In a feedback system the transient response

- (a) Decays at constant rate

- (b) gets magnified

- (c) decays slowly

- (d) decays more quickly

- 29. Transfer function of a system is used to calculate

- (a) the steady state gain

- (b) the main constant

- (c) the order of system

- (d) the output for any given input

- (e) all of the above

- 30. Transfer function of a system is defined as the ratio of output to input in

- (a) Laplace transform

- (b) Z-transform

- (c) Fourier transform

- (d) Simple algebraic form

- 31. Introduction of feedback decreases the effect of

- (a) disturbances

- (b) noise signals

- (c) error signals

- (d) all the above

- 32. The system response of a system can be best tested with

- (a) unit impulse input signal

- (b) ramp input signal

- (c) sinusoidal input signal

- (d) exponentially decaying input signal

- 33. Which of the following is a closed loop system

- (a) electric switch

- (b) car starter

- (c) de generator

- (d) auto-pilot for an aircraft

- 34. Which of the following is used as an error detector

- (a) potentiometer

- (b) field controlled ac motor

- (c) amplidyne

- (d) armature controlled ac motor

- 35. The break away point of root loci are

- (a) open loop poles

- (b) closed loop poles

- (c) open loop zeros

- (d) closed loop zeros

- 36. Noise in a control system can be kept low by

- (a) reducing the bandwidth

- (b) attenuating such frequencies at which external signals get coupled into the system

- (c) both (a) and (b)

- (d) none of these

- 37. Main cause of absolute instability in the control system is

- (a) parameters of controlling system

- (b) parameters of controlled system

- (c) parameters of feedback system

- (d) error detector where the two signals are compared

- 38. Basically a controller is

- (a) a amplifier

- (b) a clipper

- (c) a comparator

- (d) a summer

- 39. A system with gain margin close to unity or a phase margin close to zero is

- (a) highly stable

- (b) highly oscillatory

- (c) relatively stable

- (d) none of these

- 40. Which of following elements is not used in an automatic control system

- (a) sensor

- (b) error detector

- (c) oscillator

- (d) final control element

- 41. AC systems are usually preferred to the DC systems in control applications because

- (a) AC systems are cheaper

- (b) AC systems are more stable

- (c) AC systems have better performance characteristics and smaller in size

- (d) all of these

- 42. A system has the transfer function (1-s)/(1+s); It is known as

- (a) low pass system

- (b) high pass system

- (c) all pass system

- (d) none of the above

- 43. In control systems, excessive bandwidth should be avoided because

- (a) noise is proportional to bandwidth

- (b) it leads to low relative stability

- (c) it leads to slow speed of response

- (d) none of these

- 44. In most systems, an increase in gain leads to

- (a) larger damping ratio

- (b) smaller damping ratio

- (c) constant damping ratio

- (d) none of these

- 45. A step function is applied to the input of a system and output is of the form y = t, the system is

- (a) stable

- (b) unstable

- (c) not necessarily stable

- (d) conditionally stable

- 46. Which of the following can be magnified by magnetic amplifier

- (a) voltage

- (b) current

- (c) power

- (d) none of above

- 47. The inductance is not used in lag network because of

- (a) big size

- (b) time delay and hysteresis losses

- (c) high reactance

- (d) none of these

- 48. Saturation in a stable control system can cause

- (a) conditional stability

- (b) over damping

- (c) low level oscillations

- (d) high level oscillations

- 49. Excessive noise in control systems can cause

- (a) reduction in bandwidth

- (b) reduction in gain

- (c) saturation in amplifying stages

- (d) oscillations

- 50. The type-0 system has

- (a) net pole at the origin

- (b) no pole at the origin

- (c) simple at one origin

- (d) two poles at the origin