- BSNL TTA Question Paper-Electrical Specialization 2007

- 1. In a D.C. generator, if the brushes are given a small amount of forward shift, the effect of armature is

- a. Totally demagnetizing

- b. Totally magnetizing

- c. Partly demagnetizing and partly cross magnetizing

- d. Totally cross magnetizing

- 2. The air gap between stator and armature of an electric motor is kept as small as possible

- a. To get a stronger magnetic field

- b. to improve the air circulation

- c. To reach the higher speed of rotation

- d. To make the rotation easier.

- 3. Two series motors are coupled. One motor runs as generator and other as motor. The friction losses of the two machines will be equal when

- a. Both operates at same voltage

- b. Both have same back emf

- c. Both have same speed

- d. both have same excitation

- 4. Plugging of D.C. motor is normally executed by

- a. Reversing the field polarity

- b. Reversing the armature polarity

- c. Reversing both the armature and field polarity

- d. Connecting a resistance across the armature.

- 5. Transformer oil transformer provides

- a. Insulation and cooling

- b. B. Cooling and lubrication

- c. Lubrication and insulation

- d. Insulation, cooling and lubrication

- 6. Leakage fluxes of transformer may be minimized by

- a. Reducing the magnetizing current to the minimum

- b. Reducing the reluctance of the iron core to the minimum

- c. Reducing the number of primary and secondary turn to the minimum

- d. Sectionalizing and interleaving the primary and secondary windings

- 7. Electric power is transformed upon one coil to other coil in a transformer

- a. Electrically

- b. Electro Magnetically

- c. Magnetically

- d. Physically

- 8. The most suitable and economical connection for small high voltage transformer isa.

- Star- Delta connection

- b. Delta- Delta connection

- c. Delta- Star connection

- d. Star- Star connection

- 9. An alternator is said to be over excited when it is operating at

- a. Unity power factor

- b. Leading power factor

- c. Lagging power factor

- d. Either lagging or leading power factor

- 10. In an A.C. machine, the armature winding is kept stationary while the field winding is kept rotating for the following reason

- a. Armature handles very large currents and high voltages

- b. Armature friction involving deep slots to accommodate large coils is easy if armature is kept

- stationary

- c. Ease of cooling the stator than rotor

- d. None of the above.

- 11. In a synchronous motor, the torque angle is the

- a. Angle between the rotating stator flux and rotor poles

- b. Angle between the magnetizing current and back emf

- c. Angle between the supply voltage and back emf

- d. None of the above

- 12. A 3-phase synchronous motor is said to be "floating" when it operates

- a. On no load and without loss

- b. On constantly varying load

- c. On pulsating load

- d. On high load and variable supply voltage

- 13. Speed of synchronous motor depends upon

- a. Number of poles

- b. Supply frequency

- c. Both (a) and (b)

- d. Neither (a) nor (b)

- 14. Imbalance in the shaft of an induction motor occurs due to

- a. Slip rings

- b. Overheating of winding

- c. Non uniform of air gap

- d. Rigid construction

- 15. Squirrel cage induction motor has

- a. Zero starting torque

- b. Very small starting torque

- c. Medium starting torque

- d. Very high starting torque

- 16. The principle of operation of a 3-phase induction motor is similar to that of a

- a. Synchronous motor

- b. Repulsion - start induction motor

- c. Transformer with a shorted secondary

- d. Capacitor - start, induction - run motor

- 17. The speed/load characteristics of a universal motor are similar to those

- a. D.C. shunt motor

- b. D.C. series motor

- c. A.C. motor

- d. None of the above

- 18. Single phase A.C. motor generally used for vacuum cleaners is

- a. Universal motor

- b. Repulsion motor

- c. Hysteresis motor

- d. Reluctance motor

- 19. Buchholz relay is used for the protection of

- a. Switch yard

- b. Transformers

- c. Alternators

- d. Transmission lines

- 20. The type of braking used in traction system is

- a. Mechanical braking

- b. Electro - pneumatic braking

- c. Vacuum braking system

- d. All the above

- 21. The function of processing zenger diode in a UJT circuit used for triggering of SCRs is to

- a. Expedite the generation of triggering pulses

- b. Delay the generation of triggering pulses

- c. Provide a constant voltage to UJT to prevent erratic firing

- d. Provide a variable voltage to UJT as the source voltage changes

- 22. The frequency of a ripple in the output voltage of a 3 - phase semi converter depends upon

- a. Firing angle and load resistance

- b. Firing angle and supply frequency

- c. Firing angle and load inductance

- d. Only on load circuit parameters

- 23. The SCR is turned off when the anode currents falls below

- a. Forward current rating

- b. Break - over voltage

- c. Holding current

- d. Latching

- 24. V4 characteristics of emitter of a UJT is

- a. Similar to CE with linear and saturation region

- b. Similar to FET with a linear and pinch of region

- c. Similar to tunnel diode in some respects

- d. Linear between the peak point and valley point

- 25. A transformer works on

- a. DC

- b. AC

- c. AC & DC both

- d. Neither AC not DC

- 26. Which of the following device is used in transformer?

- a. Tube light

- b. Electric heater

- c. Mobile phone

- d. Rectifier module

- 27. Earth electrodes can be in the form of

- a. rods or piper

- b. stripes

- c. plates

- d. any of above

- 28. Carbone or metal brushes are used in

- a. DC generators only

- b. AC generators only

- c. Both AC & DC generation

- d. None of above

- 29. Energy is lost due to Joule's heating effects in winding of transformer. This is called

- a. Copper loss

- b. Eddy current loss

- c. Flux loss

- d. None

- 30. In refrigeration cycle heat is lost in

- a. Cooling coil

- b. Condenser

- c. Compressor

- d. Expansion valve

- 31. The power factor of AC circuit is

- a. R/X

- b. R/Z

- c. Z/R

- d. Zero

- 32. Silicon controlled output is good if ripple factor is

- a. Switch

- b. Transformer

- c. Amplifier

- d. None of above

- 33. The rectifier output is good if ripple factor is\

- a. More

- b. Less

- c. Constant

- d. None of above

- 34. Protective relays can monitor large AC current by means of

- a. Current transformer

- b. Potential transformer

- c. Micro transformer

- d. None of above

- 35. The combines AM of two similar batteries connected in parallel is:

- a. halved

- b. doubled

- c. remain constant

- d. none of above

- 36. The current in circuit having 5 V EMI source and 10 Ohm resistance is:

- a. 2 Amp

- b. 50 Amp

- c. 5 Amp

- d. ½ Amp

- 37. The chopper is a device to change

- a. Voltage

- b. Current

- c. Frequency

- d. None of these

- 38. The power consumption, in case of centrifugal loads (like pump, fan, blower etc) is

- proportional to:

- a. speed

- b. square of speed

- c. cube of speed

- d. none of these

- 39. Which of these need to be measured after rewinding the motor:

- a. no load current

- b. air gap

- c. winding resistance

- d. all of the above

- 40. Five percent increase in supply frequency will change the synchronous speed of motor by:

- a. -5%

- b. +5%

- c. -10%

- d. +10%

- 41. Which of the following is the best inverter?

- a. square wave inverter

- b. sine wave inverter

- c. pure sine wave inverter

- d. triangular wave inverter

- 42. For driving a motor in a tape recorder or record player, the motor used is generally:

- a. a synchronous motor

- b. a hydraulic motor

- c. an induction motor

- d. a dc series motor

- 43. The DC motor starter used with a constant speed shunt motor is:

- a. 2 point starter

- b. 3 point starter

- c. 4 point starter

- d. 5 point starter

- 44. A commutator in a DC motor converts

- a. AC to DC

- b. DC to AC

- c. Both AC to DC and DC to AC

- d. None of these

- 45. Two transformers running in parallel will share the load according to their:

- a. leakage reactance

- b. pu impedance

- c. efficiency

- d. rating

- 46. The size of the Earth Wire is determined by:

- a. the ampere capacity of the service wires

- b. the atmospheric conditions

- c. the voltage of service wires

- d. none of these

- 47. The function of lightning arrester is:

- a. to limit the short circuit fault current

- b. to provide path to high voltage surge to earth

- c. to reduce arcing

- d. none of these

- 48. Surge protector provide:

- a. high impedance to normal voltage

- b. low impedance to surge

- c. both (a) and (b)

- d. none of these

- 49. Earthing is necessary to give protection against

- a. voltage fluctuation

- b. overloading

- c. danger of electric shock

- d. high temperature of conductors

- 50. The primary function of fuse is to

- a. protect the appliance

- b. open the circuit

- c. prevent excessive current

- d. protect the line