- BSNL TTA Question Paper-Instruments and Measurement

- Specialization 2007

- (1) Instrument is a device for determining

- (a) the magnitude of a quantity

- (b) the physics of a variable

- (c) either of the above

- (d) both (a) and (b)

- (2) Electronic instruments are preferred because they have

- (a) no indicating part

- (b) low resistance in parallel circuit

- (c) very fast response

- (d) high resistance in series circuit

- (e) no passive elements.

- (3) A DC wattmeter essentially consist of

- (a) two ammeters

- (b) two voltmeters

- (c) a voltmeter and an ammeter

- (d) a current and potential transformer

- (4) Decibel is a unit of

- (a) power

- (b) impedance

- (c) frequency

- (d) power ratio

- (5) A dc voltmeter may be used directly to measure

- (a) frequency

- (b) polarity

- (c) power factor

- (d)power

- (6) An accurate voltmeter must have an internal impedance of

- (a) very low value

- (b) low value

- (c) medium value

- (d) very high value

- (7) The insulation resistance of a transformer winding can be easily measured with

- (a) Wheatstone bridge

- (b) megger

- (c) Kelvin bridge

- (d) voltmeter

- (8) A 100 V voltmeter has full-scale accuracy of 5%. At its reading of 50 V it will give an error of

- (a) 10%

- (b) 5%

- (c) 2.5%

- (d) 1.25%

- (9) You are required to check the p. f. of an electric load. No p.f. meter is available. You would use:

- (a) a wattmeter

- (b) a ammeter, a voltmeter and a wattmeter

- (c) a voltmeter and a ammeter

- (d) a kWh meter

- (10) The resistance of a field coil may be correctly measured by using

- (a) a voltmeter and an ammeter

- (b) Schering bridge

- (c) a Kelvin double bridge

- (d) a Maxwell bridge

- (11) An analog instrument has output

- (a) Pulsating in nature

- (b) Sinusoidal in nature

- (c) Which is continuous function of time and bears a constant relation to its input

- (d) Independent of the input quantity

- (12) Basic charge measuring instrument is

- (a) Duddel's oscillograph

- (b) Cathode ray oscillograph

- (c) Vibration Galvanometer

- (d) Bailastic Galvanometer

- (e) Battery Charging equipment

- (13) A.C. voltage can be measured (using a d.c. instrument) as a value obtained

- (a) by subtracting the d.c. reading from it's a.c. reading.

- (b) Using the output function of the multimeter

- (c) By using a suitable inductor in series with it

- (d) By using a parallel capacitor with it

- (e) None of the above

- (14) A moving coil permanent magnet ammeter can be used to measure

- (a) D. C. current only

- (b) A. C. current only

- (c) A. C. and D. C. currents

- (d) voltage by incorporating a shunt resistance

- (e) none of these

- (15) Select the wrong statement

- (a) the internal resistance of the voltmeter must be high

- (b) the internal resistance of ammeter must be low

- (c) the poor overload capacity is the main disadvantage of hot wires instrument

- (d) the check continuity with multimeter, the highest range should be used.

- (e) In moving iron voltmeter, frequency compensation is achieved by connecting a capacitor across its

- fixed coil.

- (16) Which of the following instrument is suitable for measuring both a.c. and d.c.

- quantities.

- (a) permanent magnet moving coil ammeter.

- (b) Induction type ammeter.

- (c) Quadrant electrometer.

- (d) Moving iron repulsion type ammeter.

- (e) Moving iron attraction type voltmeter.

- (17) Swamping resistance is used in moving coil instruments to reduce error due to

- (a) thermal EMF

- (b) temperature

- (c) power taken by the instrument

- (d) galvanometer sensitivity.

- (18) A power factor meter is based on the principle of

- (a) electrostatic instrument

- (b) Electrodynamometer instrument

- (c) Electro thermo type instrument

- (d) Rectifier type instrument.

- (19) A potentiometer recorder is used for

- (a) AC singles

- (b) DC signals

- (c) both (a) and (b)

- (d) time varying signals

- (e) none of these.

- (20) Transformers used in conjunction with measuring instruments for measuring purposes are called

- (a) Measuring transformers

- (b) transformer meters

- (c) power transformers

- (d) instrument transformers

- (e) pulse transformers.

- (21) Leakage flux in an electrical machine is measure by

- (a) Ballistic galvanometer

- (b) Flux meter

- (c) Either (a) or (b)

- (d) Vibration galvanometer

- (e) CRO

- (22) A C.R.O. is used to indicate

- (a) supply waveform

- (b) magnitude of the applied voltage

- (c) B.H. loop

- (d) all of these

- (e) Magnitude of the current flowing in it.

- (23) An oscillator is a

- (a) an amplifier having feedback network

- (b) a high gain amplifier

- (c) a wide band amplifier

- (d) a untuned amplifier

- (e) None of these

- (24) Distortion can be measured by

- (a) Wave meter

- (b) Digital filters

- (c) Wein bridge circuit

- (d) Bridge T filter circuit

- (25) Series connected Q- meter is preferable for measurement of components having

- (a) high impedance

- (b) low impedance

- (c) both (a) and (b)

- (d) high frequency

- (e) low capacitance

- (26) A potentiometer is

- (a) an active transducer

- (b) a passive transducer

- (c) a secondary transducer

- (d) a digital transducer

- (e) a current sensing transducer

- (27) The basic components of a digital voltmeter are:

- (a) A/D converter and a counter

- (b) A/D converted and a rectifier

- (c) D/A converter and a counter

- (d) Ramp generator and counter

- (e) Comparator

- (28) Which of the following electrical equipment cannot convert ac into dc

- (a) diode

- (b) converter

- (c) transformer

- (d) mercury arc rectifier

- (29) Voltage measurement are often taken by using either a voltmeter or

- (a) an ammeter

- (b) an ohmmeter

- (c) an oscillator

- (d) a watt-meter

- (30)The electric device which blocks DC but allows AC is called:

- (a) capacitance

- (b) inductor

- (c) an oscilloscope

- (d) a watt-meter

- (31 ) The range of an ammeter can be extended by using a

- (a) shunt in series

- (b) shunt in parallel

- (c) multiplier in series

- (d) multiplier in parallel

- (32)A device that changes one form of energy to another is called

- (a) rheostat

- (b) oscillator

- (c) transducer

- (d) varicap

- (33) Aquadag is used in CRO to collect

- (a) primary electron

- (b) secondary electron

- (c) both primary and secondary

- (d) none of above

- (34) A vertical amplifier for CRO can be designed for

- (a) only a high gain

- (b) only a broad bandwidth

- (c) a constant gain time bandwidth product

- (d)all of the above

- (35) One of the following is active transducer

- (a) Strain gauge

- (b) Selsyn

- (c) Photovoltic cell

- (d) Photo emissive cell

- (36) The dynamic characteristics of capacitive transducer are similar to those of

- (a) low pass filter

- (b) high pass filter

- (c) band pass filter

- (d) band stop filter

- (37) Thermocouples are

- (a) passive transducers

- (b) active transducers

- (c) both active and passive transducers

- (d) output transducers

- (38) The size of air cored transducers as compare to iron core counter part are

- (a) bigger

- (b) smaller

- (c) same

- (39) From the point of view of safety, the resistance of earthing electrode should be:

- (a) low

- (b) high

- (c) medium

- (d) the value of resistance of electrode does not effect the safety

- (40) In CRT the focusing anode is located

- (a) between pre accelerating and accelerating anodes

- (b) after accelerating anodes

- (c) before pre accelerating anodes

- (d) none of above

- (41) Which transducer converts heat energy into electrical energy

- (a) I. V. D. T.

- (b) thermocouple

- (c) photoconductor

- (d) none of the above

- (42) Which of photoelectric transducer is used for production of electric energy by converting solar energy

- (a) photo emission cell

- (b) photo diode

- (c) photo transistor

- (d) both (b) and (c)

- (43) Which of the following instruments consumes maximum power during measurement?

- (a) induction instruments

- (b) hot wire instruments

- (c) thermocouple instruments

- (d) electrodynamometer instruments

- (44) Which of the following meters has the best accurancy

- (a) moving iron meter

- (b) moving coil meter

- (c) rectifier type meter

- (d) thermocouple meter

- (45) The function of the safety resistor in ohm meter is to

- (a) limit the current in the coil

- (b) increase the voltage drop across the coil

- (c) increase the current in the coil

- (d) protect the battery

- (46) Which of the following instruments is free from hysteresis and eddy current losses?

- (a) M.l. instrument

- (b) electrostatic instrument

- (c) electrodynamometer type instrument

- (d)all of these

- (47) The dielectric loss of a capacitance can be measured by

- (a) Wien bridge

- (b) Owen bridge

- (c) Schering bridge

- (d) Maxwell bridge

- (48) Reed frequency meter is essentially a

- (a) recording system

- (b) deflection measuring system

- (c) vibration measuring system

- (d) oscillatory measuring system

- (49) In measurements made using a Q meter, high impedance elements should preferably be connected in

- (a) star

- (b) delta

- (c) series

- (d) parallel

- (50) A digital voltmeter measures

- (a) peak value

- (b) peak-to-peak value

- (c) rms value

- (d) average value