- BSNL TTA Question Paper-Network/ Transmission 2007

- 1. Pick up wrong statement

- (a) A group of interconnected individual components known as circuit elements is called a network.

- (b) A humped network is an arrangement of physically separate resistors, inductors and capacitors.

- (c) Distributed network is one, which the resistive, inductive and capacitive effects are inseparable for

- network analyses.

- (d) A branch is a network having four elements.

- 2. Kirchoff's laws for networks are:

- (a) The algebraic sum of branch currents meeting at any node is zero.

- (b) The algebric sum of voltage drops in any set of branches forming a closed circuit or loop must be

- equal to zero.

- (c) Both (a) and (b)

- (d) Neither (a) and (b)

- 3. Mutually coupled circuit is a circuit which is:

- (a) Bilateral

- (b) Unilateral

- (c) None of these

- (d) Either (a) or (b)

- 4. Duality is a

- (a) Transformation in which current and voltages are interchanged

- (b) Active sources become passive sources

- (c) Passive sources become active sources

- (d) Both (b) and (c)

- 5. Combined inductance of two inductors L1 and L2 connected and voltages are interchanged

- (a) L1 + L2

- (b) (L1 + L2)/ L1

- (c) (L1 + L2) / (L1 X L2)

- (d) (L1 X L2) / (L1 + L2)

- 6. Normal analysis techniques are based on

- (a) Thevenin's theorem

- (b) Tellegan's theorem

- (c) Superposition theorem

- (d) Kirchoff's Law

- 7. Two voltage sources can be connected in parallel when they are equal in

- (a) Magnitude

- (b) Frequency

- (c) Phase

- (d) All the above

- 8. The kirchoff's law fail in

- (a) Linear circuits

- (b) Non-linear circuits

- (c) Lumped parameter circuits

- (d) Distributed parameter circuits

- 9. Which of the following is a nonreciprocal network ?

- (a) A network consisting of all resistances

- (b) A network consisting of all capacitances

- (c) A network consisting of all inductances

- (d) A transistor model

- 10. When two systems obey equations of the same form the systems are said to be

- (a) Similar system

- (b) Identical system

- (c) Analogous system

- (d) Digital system

- 11. For a highly selective circuit

- (a) It must have large value of Q

- (b) It must have high value of capacitance to produce resonance at fixed frequency

- (c) Either (a) or (b)

- (d) Neither (a) nor (b)

- 12. A network consisting of four terminals is called a

- (a) One port network

- (b) Two port network

- (c) Four port network

- (d) None of the above

- 13. Driving point of a network is

- (a) A port where voltage or current source is connected

- (b) A terminal where load is connected

- (c) A port where load is connected

- (d) None of the above

- 14. Ceramic filters are similar in construction to

- (a) Crystal filters

- (b) Crystal ladder filters

- (c) Crystal lattice

- (d) Mechanical filters

- 15. When two port networks are connected in parallel the resultant

- (a) Z parameters are the some of individual parameters

- (b) Y- parameters are the some of individual parameters

- (c) Both (a) and (b)

- (d) Neither (a) nor (b)

- 16. Electric wave filters

- (a) Allow electric signals with specified frequency range

- (b) Suppress signals outside a specified range

- (c) Both (a) and (b) occurs simultaneously

- (d) Either (a) or (b) occur at a time

- 17. A cascade connection of low pass filter and high pass filter is called

- (a) Band pass filter

- (b) Band elimination filter

- (c) Neither (a) nor (b)

- (d) Both (a) and (b)

- 18. The response of a network is decided by the location of

- (a) Its poles

- (b) Its zeros

- (c) Either (a) nor (b)

- (d) Both (a) and (b)

- 19. Example of two port network is

- (a) Transformer

- (b) Transmission line

- (c) Bridge circuit and transistor circuit

- (d) All of the above

- 20. The circuit whose properties are same in either direction is called

- (a) Universal circuit

- (b) Reversible circuit

- (c) Unilateral circuit

- (d) Bilateral circuit

- 21. Distortion in transmission line is due to

- (a) Delay distortion

- (b) Phase distortion

- (c) Frequency distortion

- (d) All the above

- 22. The general parameters distributed along a transmission line are

- (a) R&L only

- (b) L&C only

- (c) C&G only

- (d) R, L, C&G

- 23. Phase distortion is prominently caused by

- (a) circuit transients

- (b) non linear characteristics

- (c) linearity

- (d) none

- 24. The voltage or current from the receiving end towards the sending end, decreasing in amplitude with increasing distance from the load is called

- (a) incident wave

- (b) medium wave

- (c) reflected wave

- (d) none of above

- 25. E.M. Waves of UHF is propagated efficiently via

- (a) parallel wire transmission lines

- (b) open wire transmission lines

- (c) wave guides

- (d) coaxial cables

- 26. Norton theorem is valid for network containing only

- (a) linear elements

- (b) no linear elements

- (c) resistance

- (d) reactance

- 27. The maximum power is absorbed by one network from other, joined to it at two terminals when the impedance of one is

- (a) complex conjugate of other

- (b) square root of other

- (c) same as other

- (d) none of above

- 28. The decrease in effective conductor cross section at high frequencies

- (a) decrease the conductor resistance

- (b) increase the conductor resistance

- (c) no change in conductor resistance

- (d) none of above

- 29. Voltage standing wave ratio lies in the range

- (a) 0 to 1

- (b) 1 to infinity

- (c) 0 to infinity

- (d) -1 to +1

- 30. Attenuators have applications

- (a) in AC circuits only

- (b) in DC circuits only

- (c) in AC as well DC circuits

- (d) in low frequency circuits only

- 31. In an network

- (a) the number of tree branches is equal to the number of links

- (b) the number of tree branches cannot be equal to the number of links

- (c) the number of tree branches has no relation with the number of links branches

- (d) none of these

- 32. In open line transmission systems, attenuation is more at

- (a) lower frequencies

- (b) medium frequencies

- (c) higher frequencies

- (d) remains constant

- 33. a power ratio 100 is equivalent to

- (a) 10 dB

- (b) 20 dB

- (c) 50 dB

- (d) 100 dB

- 34. The velocity factor for small widely spaced conductors such as open wire line in air is very

- nearly

- (a) 0.66

- (b) 0.98

- (c) 0.82

- (d) 0.76

- 35. Transmission of power to a load over a transmission line achieves optimum value when standing wave ratio (SWR) becomes

- (a) 2 : 1

- (b) 1 : 2

- (c) 1 : 1

- (d) 1 : 10

- 36. The VSWR in a short circuited loss less transmission line equals

- (a) infinity

- (b) unity

- (c) zero

- (d) none of above

- 37. The velocity factor of a transmission line

- (a) is always greater than unity

- (b) depend upon the permitivity of the surrounding medium

- (c) is lease for air medium

- (d) is governed by skin effect

- 38. Which of the following is not correct

- (a) voltage source is an active element

- (b) current source is a passive element

- (c) resistance is a passive element

- (d) conductance is a passive element

- 39. A network is said to be nonlinear if it does not satisfy

- (a) superposition condition

- (b) homogeneity condition

- (c) both superposition and homogeneity conditions

- (d) associative condition

- 40. An capacitor with zero initial condition at t = 0+ act as a

- (a) short circuit

- (b) open circuit

- (c) current source

- (d) voltage source

- 41. An inductor stores energy in

- (a) electrostatic field

- (b) electromagnetic field

- (c) magnetic field

- (d) core

- 42. In series LCR circuits, at resonance,

- (a) current is maximum, power factor is zero

- (b) current is maximum, power factor is unity

- (c) current is minimum, power factor is unity

- (d) none of above

- 43. In an RCL series circuit, during resonance, the impedance will be

- (a) zero

- (b) minimum

- (c) maximum

- (d) none of above

- 44. When a source is delivering maximum power to load, the efficiency of the circuit is always

- (a) 50%

- (b) 75%

- (c) 100%

- (d) None of above

- 45. In a linear network, when the ac input is doubled, the ac output becomes

- (a) two times

- (b) four times

- (c) half

- (d) one forth

- 46. A passive network has

- (a) current sources but no voltage sources

- (b) voltage sources but no current sources

- (c) both current and voltage sources

- (d) no voltage or current sources

- 47. Two resistances are connected in parallel and each dissipates 50 waits. The total power supplied by the source is

- (a) 25 watts

- (b) 50 watts

- (c) 100 watts

- (d) 200 watts

- 48. Three bulbs of 60 watts each are connected is parallel across 220v, 50 Hz supply. If one bulb burns out

- (a) only remaining two will operate

- (b) remaining two will not operate

- (c) all of three will operate

- (d) there will be heavy current from the supply

- 49. The amplitude of an audio signal is 10 and that of carrier wave is 50. Percentage modulation is:

- (a) 0.2

- (b) 20

- (c) 5

- (d) 60

- 50. The main advantage of PCM system is:

- (a) lower bandwidth

- (b) lower power

- (c) lower noise