- BSNL JTO- Telecom Model Exam Paper 2009

- Q.1 A house served by a 220V supply light, is protected by a 9-Ampere fuse. The maximum number of

- 60 W bulbs in parallel that can be turned on is

- (a) 11 (b) 33

- (c) 22 (d) 44

- Q.2 An n-channel JFET has IDSS = 1 μA & VP =–5V. The maximum transconductance is

- (a) gm = 0.4 milli mho (b) gm = 0.04milli mho

- (c) gm = 0.04 mho (d) gm = 0.4 millimilli mho

- Q.3 The base to base resistance of a UJT is 6 K Ohm when the emitter current is zero. If RB1 = 3.6 K

- Ohm, the intrinsic stand off ratio is

- (a) n = 0.66 (b) n = 0.6

- (c) n = 6.0 (d) n = 3.6

- Q.4 A capacitor of 1μF initially charged to 10 V is connected across an ideal inductor of 0.1 mH. The

- maximum current in the circuit is

- (a) 0.5 A (b) 1 A

- (c) 1.5 A (d) 2 A

- Q.5 Silicon has a preference in IC technology because

- (a) it is an indirect semiconductor

- (b) it is a covalent semiconductor

- (c) it is an elemental semiconductor

- (d) of the availability of nature oxide SiO

- Q.6 Three equal resistance of magnitude 5 Ohm each are connected in delta. The resistance between

- any two pair of terminals of the delta will be

- (a) 5 Ohm (b) 5/3 Ohm

- (c) 10/3 Ohm (d) 3/5 Ohm

- Q.7 The R.M.S. value of a half wave rectified sinusoidal alternating current with peak value Im is

- (a) Im/1 (b) mI / 2

- (c) mI /2 (d) mI 3

- Q.8 For a series resonant circuit, at the half power points,which of the following is true?

- (a) Current is half of the current at resonance.

- (b) Resistance is equal to the reactance.

- (c) The impedance is half the impedance at the resonance.

- (d) None of the above

- Q.9 The lossless line of characteristics impedance 300 Ohm is terminated in a pure resistance of 200

- Ohm. The value of the standing wave ratio is

- (a) 1.5 (b) 0.67

- (c) 1.0 (d) 1.25

- Q.10 The transient current in lossless L-C circuit when excited from an AC source is, sine wave

- (a) Critically damped (b) Underdamped

- (c) Overdamped (d) Undamped

- Q.11 The values of L and C for a low pass filter with cut off frequency of 2.5 KHz and operating with a

- terminated load resistance of 450 Ohm are given by

- (a) 57.3 mH; 0.283 μF (b) 28.66 μH; 0.14 μF

- (c) 114.64 mH; 0.566 mF (d) 50.23 mH; 0.632 mF

- Q.12 The driving point impedance with poles at ? = 0(zero) and ? = 8 (infinity) must have the

- (a) s term in the denominator and an excess term in the numerator

- (b) s term in the numerator and an excess term in the denominator

- (c) s term in the numerator and equal number of terms in the

- numerator and denominator

- (d) s term in the denominator and equal number of terms in the

- numerator and denominator

- Q.13 A transmission line is terminated at its characteristic impedance. The reflection coefficient is

- (a) 1 (b) –1

- (c) 0 (d) 8

- Q.14 In the circuit shown below, the current through the 3/11O resistance between terminals A & B is

- (a) 4 Amps (b) 1 Amp

- (c) 2 Amps (d) 5 Amps

- Q.15 In a series RLC circuit operating below the resonant frequency the current

- (a) I leads Vs (b) I lags Vs

- (c) I is in Phase with Vs (d) None of these

- Q.16 An antenna has maximum radiation intensity Umax = 10 Watt/Sr and average radiation intensity

- Uavg = 4.5 Watt/Sr. If the efficiency of the antenna is given as ?r = 95%, the input power of the

- antenna is

- (a) 2.222 Watt (b) 12.11 Watt

- (c) 55.55 Watt (d) 59.52 Watt

- Q.17 In an airport, a receiving, antenna has a maximum dimension of 3 meters and operates at 100

- MHz. An aircraft approaching the airport is 1/2 Km away from the antenna . The aircraft is in the

- region of the antenna.

- (a) far-field (b) near-field

- (c) close-field (d) Out of reach

- Q.18 A lossless transmission line with characteristic impedance 500 Ohms is excited by a signal of

- voltage 10?0° volts at 1.2 MHz. If the line is terminated by ZL at a distance 1 Km, the input impedance

- s of the line for ZL = 8 (infinity) and ZL = 0 (zero) in Ohm, respectively are

- (a) +j8, 0 (b) –j8, 0

- (c) 0, – j8 (d) 0, +j8

- Q.19 If the electric field of a plane wave is represented by E = 10 yˆ cos (109 t+30z) volt/m, assuming

- e is the dielectric constant, the corresponding magnetic field H is

- (a) – ˆy e 109/3 cos (109 t + 30z) Amp/m

- (b) – ˆx e109/3 cos(109t + 30z) Amp/m

- (c) – ˆz e 109/3 cos (109t + 30 z) Amp/m

- (d) + ˆx e 109/3 cos (109t+30z) Amp/m

- Q.20 A 50 Ohm lossless line connects a signal of 200 KHz to a load of 200 Ohm. If the load power in

- 100 mW, the value for voltage minimum Vmin is

- (a) ( 20)/4 (b) ( 10)/ 4

- (c) ( 20)/ 2 (d) ( 10)/ 2

- Q.21 Choose the correct statement

- (a) Digital multimeters are built using current measuring elements, while analog multimeters are built

- using voltage measuring units

- (b) Digital multimeters are built using voltage measuring units, while analog multimeters are built

- using current measuring units

- (c) Both digital and analog multimeters are built using voltage measuring units

- (d) Both digital and analog multimeters are built using current measuring units

- Q.22 An analog voltmeter has a sensitivity of 10kO/volt. The galvanometer used in constructing the

- instrument will produce a full scale deflection when the current passed through it is

- (a) 10 mA (b) 20 mA

- (c) 50 mA (d) 100 μA

- Q.23 The input versus output characteristics of a digital-to-analog converter is given in the table below:

- The converter is exhibiting

- (a) offset error (b) statistical error

- (c) linearity error (d) hysteresis error

- Q.24 An optical fiber cable laid underground has developed a discontinuity at a distance d from the

- source end. The fault can be located using the instrument

- (a) Optical spectrum Analyzer (OSA)

- (b) Optical Time Domain Reflectometer (OTDR)

- (c) Optical Power Meter (OPM)

- (d) Laser Diffractometer (LD)

- Q.25 Two sinusoidal signals of the same frequency are displayed on a dual-trace oscilloscope. One

- complete cycle of each signal covers 6 cm of the horizontal scale and the starting point of the

- horizontal scale and the starting point of the two signals are separated by 0.5 cm. The phase difference

- between the two signals in degrees is

- (a) 30 (b) 45

- (c) 60 (d) 90

- Q.26 Transient signals can be observed using

- (a) storage oscilloscope (b) sampling oscilloscope

- (c) wave analyzer (d) spectrum analyzer

- Q.27 The trace on an oscilloscope continually moves to the right of the screen when

- (a) the sweep is triggered.

- (b) the sweep period is larger than the signal period.

- (c) the sweep period is smaller than the signal period.

- (d) there is no weep.

- Q.28 In a dual trace oscilloscope, the display appears segmented when

- (a) low frequency signals are observed in Alternate mode

- (b) low frequency signals are observed in Chop mode

- (c) high frequency signals are observed in Alternate mode

- (d) high frequency signals are observed in Chop mode

- Q.29 To distinguish between signals having very close values, we need an instrument with

- (a) high accuracy (b) high resolution

- (c) high sensitivity (d) high linearity

- Q.30 Match List-I (Instruments) with List-II (Measurement in which the instrument is used) and select

- the correct answer using the codes

- given below the lists:

- List-I List-II

- A. Lock-in amplifier 1. Patient monitoring

- B. Sampling oscillocope 2. Overcoming ground loop problem

- C. Isolation amplifier 3. Phase difference between two signals

- D. Strip-chart recorder 4. Signal recovery from noise 5. Observing very high frequency signals

- Codes:

- A B C D

- (a) 1 3 5 4

- (b) 2 1 3 4

- (c) 4 5 2 1

- (d) 3 4 1 2

- Q.31 A power diode has lightly doped n type substrate sandwiched between heavily doped p and n

- regions

- (a) to increase reverse breakdown voltage

- (b) to reduce ohmic loss under forward bias

- (c) to decrease switching time of the power diode

- (d) to improve transient behaviour of the diode

- Q.32 An ideal thyristor is driving an R-L load of impedance Z. Input AC voltage is Vs = Vm sin ?mt. If

- thyristor is fired at an input phase angle of 90°. What will be the output voltage and output current

- across R-L load at the instant of firing?

- (a) Output voltage is Vm and output current is Vm/Z

- (b) Output voltage and output current are both zero

- (c) Output voltage is zero and output current is delayed by an angle 90°

- (d) Output voltage is Vm and output current is zero

- Q.33 In a regenerative braking, which of the following is generally true?

- (a) Back e.m.f. in the motor exceeds the applied voltage

- (b) Back e.m.f. is less than the applied voltage

- (c) Kinetic energy of the motor is dissipated in a resistance

- (d) Kinetic energy of the motor is dissipated through free wheeling

- diode across the motor

- Q.34 A step-down chopper, fed from a 120 volt DC source, operates a DC motor whose armature e.m.f.

- and armature resistance are 100 volt and 0.5O respectively. With the magnitude control ratio 0.6, the

- quadrant of operation DC motor is

- (a) First (b) Second

- (c) Third (d) Fourth

- Q.35 For IGBT, which of the following statement is true?

- (a) Switching speed of IGBT is more than bipolar transistor

- (b) IGBT is a current-controlled device

- (c) On-state collector-emitter voltage is less than that of bipolar junction transistor

- (d) It combines voltage control features of MOSFET gate and high power capability of bipolar

- transistor

- Q.36 The semiconductor used for LEDs emitting in the visible range is

- (a) GaAs (b) GaAlAs

- (c) GalnAs (d) GaAsP

- Q.37 The polar bonds existing in III-V compound semicondutor, may be considered as equivalent to

- (a) 1 ionic bond and 3 covalent bonds

- (b) 1 ionic bond and 4 covalent bonds

- (c) 2 ionic bonds and 2 covalent bonds

- (d) 2 ionic bonds and 4 covalent bonds

- Q.38 Which of the following pairs of crystal structure possesses the same atomic packing density?

- (a) simple cubic & body centered cubic

- (b) body centered cubic & face centered cubic

- (c) face centered cubic & hexagonal close packed

- (d) body centered cubic & hexagonal close packed

- Q.39 The color bands on a carbon composition resistor occur in the sequence: yellow, violet, yellow

- and silver. Its resistance is

- (a) 470 KO ± 47 KO (b) 470 KO ± 23.5 %

- (c) 47 KO ± 10 % (d) 47 KO ± 5 %

- Q.40 The real & imaginary dielectric constants er' and er'' of 3 insulators at 1 KHz and 50° C are listed

- below:

- Material er' er''

- Polycarbonate 2.47 0.003

- PET 2.58 0.003

- PEEK 2.24 0.003

- At a given voltage, the lower power dissipation per unit capacitance at 1 KHz can be obtained from

- (a) Polycarbonate

- (b) PET

- (c) PEEK

- (d) Insufficient information to answer

- Q.41 A battery of 40 V and three capacitors of 1000 μF, 500 μF and 100 μF are all connected in (I)

- parallel and (II) series. The ratio of total charge stored in case I to that in case II is approximately

- (a) 3 : 64 (b) 64 : 3

- (c) 160 : 3 (d) 104 : 5

- Q.42 The magnetic flux f ( in Weber) linked with a coil at an instant of time t(in second) is given by f(t)

- = 2t2 – 20t + 40. The induced e.m.f. in the coil at the instant t =2 second is

- (a) 22 V (b) 20 V

- (c) 12 V (d) 10 V

- Q.43 The speed of an audio cassette tape in a cassette player is 5cm/ sec. If the maximum frequency

- that needs to be recorded is 20 KHz, the minimum spatial wavelength on the tape is

- (a) 40 μm (b) 25 μm

- (c) 4 μm (d) 2.5 μm

- Q.44 In a power transformer, the fundamental frequency of the hum arising due to magnetostriction is

- (a) equal to the line frequency

- (b) double the line frequency

- (c) 4 times the line frequency

- (d) not related to the line frequency

- Q.45 At a particular temperature and current density, the critical magnetic field for a Type I

- superconductor is Bc and that for a type II superconductor ranges from Bc1 to Bc2 keeping other

- parameters unloaded. both superconductors are now subjected to a magnetic field B that satisfies the

- conditions B>Bc and

- Bc1 < q ="4q9," x ="d3" q ="–4q9," x ="dQ4="" q ="–4q9," x ="d3" q ="–4q9," x ="d4" gap =" 1.1"

- charge =" 1.6" a =" 0.99," ib =" lB" icbo =" 200" ic =" 2.5" ic =" 1.5mA" ic =" 3.5" ic =" 4.5" 1 ="

- A.Qn" 1 =" A.Qn+" 1 =" A.Qn" 1 =" A.Qn" k =" 0," n =" (no." mp =" 1.15." x =" P.Q" y="P.R" i0 ="

- X," i1=" 0," i2 =" X," i3 =" X" i0 =" 0," i1="1," i2 =" Y," i3 =" X" i0 =" X," i1 =" 1," i2 =" 0," i3 =" X"

- i0 =" X," i1 =" 0," i2 =" X," i3 =" Z" k =" 0." dt =" Ax(t)" a ="–2" b ="10" a ="0" b ="0" fc =" 0.16" d

- =" 0.5456" n =" 31.6"

- (a) > 100 (b) > 500

- (c) > 1000 (d) > 20000

- Q.67 In an IF amplifier, the IF transformer is provided with tapping to

- (a) increase the voltage gain

- (b) increase the bandwidth of the resonance circuit

- (c) increse the impedance offered by the resonance circuit to the following cascaded amplifier

- (d) increase the quality factor of the resonance circuit

- Q.68 Consider the wave form V(t) = (1+m cos?mt) cos?ct. Show that, if the demodulated wave is to

- follow the envelope of V(t), it is required that at any time t0

- (a) RC <=?m(msin?mt0 ) / (1+mcos?m t0)

- (b) (I/RC) >= ?m(msin?mt0 ) / (1+mcos?m t0 )

- (c) 1/RC <= 1/(m?m)

- (d) RC is very large

- Q.69 QPSK system is superior to BPSK system because

- (a) its bandwidth is higher than that of BPSK System

- (b) inter channel interference in QPSK system is less then that in BPSK system

- (c) bandwidth of QPSK system is half of the bandwidth of BPSK system

- (d) in QPSK system inter-symbol interference is improved

- Q.70 A radio receiver is placed at one corner of a table and again placed at some other corner of the

- same table. Loudspeaker output is changed because

- (a) image interference is reduced

- (b) adjacent channel interference is increased

- (c) of fading

- (d) line of sight propagation is not utilized

- Q.71 In time-division multiple access system, a traffic system on the receiver side must receive the

- traffic burst addressed to it. For this

- (a) transmitting frame acquisition is required

- (b) proper synchronization of the timing of transmit frame is required

- (c) frame efficiency should be higher

- (d) synchronization is necessary to overcome the perturbations of the satellite

- Q.72 Earth coverage dish antenna is used in satellite system. It is characterized by the fact that

- (a) It is a narrow beam antenna

- (b) It is Parabolic antenna

- (c) it is sharply focused within a small area of the surface of the earth

- (d) it is a wide angle antenna which covers a large area of the surface of the earth

- Q.73 In a Klystron amplifier, the RF voltage produces

- (a) amplitude modulation (b) frequency modulation

- (c) phase modulation (d) velocity modulation

- Q.74 For the proper operation of MASER at a frequency of 10 GHz, the material used is

- (a) Al2 O3 with slight doping of chromium

- (b) Ti O2 with slight doping of iron

- (c) Ti O2 with slight doping of chromium

- (d) Al2 O3 with slight doping of iron

- Q.75 A rectangular waveguide is 4.2 cm by 1.85 cm. The cut-off frequency of the dominant mode

- through this waveguide is

- (a) 3.57 GHz (b) 3.75 GHz

- (c) 3.70 MHz (d) 3.57 MHz

- Q.76 For an antenna to be frequency-independent, it should expand or contract in proportion to the

- (a) gain (b) directivity

- (c) wavelength (d) impedance

- Q.77 Suppose that data items, numbered 1, 2, 3, 4, 5 and 6 come in the input stream in this order. By

- using a queue, which of the following rearrangement can be obtained in the output order ?

- (a) 1 2 6 4 5 3 (b) 2 4 3 6 5 1

- (c) 4 2 1 3 5 6 (d) 1 2 3 4 5 6

- Q.78 We are told that the integers between 1 and 1000 are arranged in a binary search trace with ‘<‘ as

- the ordering relation. Below are four lists of vertices encountered as we search for the number 363.

- Which list cannot be produced by this search through a binary search tree?

- (a) 924, 220, 911, 244, 898, 248, 363

- (b) 2, 252, 401, 398, 330, 344, 397, 363

- (c) 925, 202, 911, 240, 912, 245, 363

- (d) 2, 399, 387, 219, 266, 382, 381, 278, 363

- Q.79 The number of 1’s in the binary representation of 13* 163 +11*16 + 2 is

- (a) 8 (b) 7

- (c) 9 (d) 12

- Q.80 A disk has 500 bytes/sector,100 sectors / track, 20 heads and 1000 cylinders. Total capacity of

- disk is

- (a) 1 GB (b) 100 MB

- (c) 10 MB (d) 10 GB

- Q.81 Suppose that the same clock signal is used to increment the microprogram counter and to load the

- control register. Which of the following assertion(s) is/are true?

- 1. Microinstruction execution time is at least two clock periods.

- 2. Microinstruction execution time can be overlapped with fetching the next microinstruction.

- 3. Unconditional branch microinstructions must necessarily take longer than other types.

- (a) 1 only (b) 2 only

- (c) 1 and 3 (d) 2 and 3

- Q.82 Some system architects do not find RISC instruction repertoire to be cost-effective because it

- (a) result in large increase in pro gramme size

- (b) result in complex structure of microcode

- (c) has been observed that an average compiler does not employ more than a limited subset of available

- instructions

- (d) results in complex decoding of opcode filed resulting in longer execution time

- Q.83 A stake machine pushes operands on a stack and evaluates binary operators by a pcs (i.e

- pop/compute/store) where the top two operands are popped computation is performed and the result is

- pushed onto stack. Evaluation of an expression (x* y) + (u* ?) by Reverse Polish notation in a stack

- machine needs

- (a) 4 push and 3 pcs instructions

- (b) 6 push and 1 pcs instructions

- (c) 4 push and 1 pcs instructions

- (d) 5 push and 2 pcs instructions

- Q.84 A dot matrix printer takes 3 msec to point a character, and 1 msec for a space between two

- consecutive characters. If it prints 100 characters per line, its printing speed specifications in characters

- per second (cps) and time to print a line of characters are respectively

- (a) 100 cps and 400 msec

- (b) 2500 cps and 0.04 sec

- (c) 250 cps and 40 msec

- (d) 250 cps and 0.4 sec

- Q.85 Consider the following program segment with 8085 microprocessor

- LXI H 3600H

- MOV A, M

- HLT

- The MOV instruction involves

- (a) indirect addressing

- (b) immediate addressing

- (c) implicit addressing

- (d) direct addressing

- Q.86 To establish a communication between 8085 microprocessor and 8255 Programmable Peripheral

- Interface chip. the status of the chip select input would be

- (a) TRISTATE (b) HIGH

- (b) LOW (d) DON’T CARE

- Q.87 Which flag does not change by the execution of the instruction DCR B in 8085 microprocessor ?

- (a) Parity (b) Carry

- (c) Zero (d) Sign

- Q.88 Let the content of the memory location 3501H be 72H. Now consider the following program with

- 8085 microprocessor

- LDA 3501H

- CMA

- STA 3502H

- HLT

- The content of the memory location 3502H after execution of

- programme will be

- (a) 27H (b) D8H

- (c) 8DH (d) 72H

- Q.89 Given the program segment below, how many times will the instruction LP: JNZ REP be

- executed?

- MVI H, 02H

- MVI L, 05H

- REP : DCR L

- LP : JNZ REP

- : DCR H

- : JNZ REP

- ...................

- (a) 10 (b) 260

- (c) 510 (d) 7

- Q.90 What addressing mode is used in the instruction RET?

- (a) Direct (b) Immediate

- (c) Implicit (d) Register-indirect

- Q.91 A one-byte instruction is executed in the 8085 microprocessor by the following steps:

- ((SP) –1) ? (PCH)

- ((SP) –2) ? (PCL)

- ((SP) ? ((SP)-2)

- (PC) ? 0008H

- The corresponding instruction is

- (a) JMP 0008H (b) PUSH PSW

- (c) CALL 0008 H (d) RST 1

- Q.92 The zero flag of 8085 microprocessor is to be set keeping the content of the accumulator

- unchanged. Which instruction is to be used?

- (a) MOV A,A (b) ANI 00H

- (c) XRA A (d) CMP A

- Q.93 In class - A power amplifier, the collector dissipation is maximum when

- (a) no signal is present

- (b) signal swing is maximum

- (c) signal swing is (1/1.414) of its maximum

- (d) none of the above

- Q.94 If cut-in voltage and forward resistance of each diode (in the adjoining figure) are 0.7 V and 1

- ohm respectively, the current through the 48 ohm resistor is

- 8 V

- + 48 O

- –

- (a) 132 mA (b) 160 mA

- (c) 0 mA (a) (1/6) A

- Q.95 The lower cut-off frequency of the transistor stage in the adjoining figure is

- Vcc

- 3 K

- 10 μF

- V0

- 2 K

- (a) 7.95 Hz (b) 13.25 Hz

- (c) 5.30 Hz (d) 3.18 Hz

- Q.96 In the adjoining current mirror circuit if Q1 and Q2 are identical and base currents are not

- neglected, then which of the following is true ?

- I1 I2

- Q1 Q2

- (a) I2 = I1 (b) I2 = ßl1

- (c) I2 = [(ß/(ß +2)]I1 (d) I2 = [(ß + 2)/ß]I1

- Q.97 If rectangular input is applied to the adjoining circuit, it produces

- RC

- I/P O/P –

- +

- (a) square output (b) spike output

- (c) sinusoidal output (d) none of the above

- Q.98 In an oscillator, if amplifier gain (A) without feedback is (1 + RF/ R1) and gain (ß) of the

- feedback network is 1/{3 + j(?Rc–1/?Rc)}, then which of the following is true ?

- (a) RF = 2R1 (b) RF = R1

- (a) |ß| = 3 (b) |ß| = 1/3