- Junior Telecom Officers (JTO) Model Question Paper

- 1. One of the following statement which is true for relative dielectric constant is -

- a) It is dimensionless

- b) It is not equal to unity for vacuum

- c) It’s value for all substances is less than one

- d) None

- 2. Pure metals generally havea)

- high conductivity and low temperature coefficient

- b) high conductivity and large temperature coefficient

- c) low conductivity and zero temperature coefficient

- d) low conductivity and high temperature coefficient

- 3. For small size, high frequency coils, the most common core material is

- a) Air

- b) Ferrite

- c) Powdered ion

- d) Steel

- 4. For an abrupt junction Varactor diode, the dependence of device capacitance (C) on applied reverse

- bias (V) is given bya)

- C a V1/3

- b) C a V-1/3

- c) C a V1/2

- d) C a V-1/2

- 5. A superconductor is a

- a)A material showing perfect conductivity and Meissner effect below a critical temperature

- b) A conductor having zero resistance

- c) A perfect conductor with highest dimagnetic susceptibility

- d) A perfect conductor which becomes resistive when the current density through it exceeds a critical

- value

- 6. When a semiconductor based temperature transducer has a temperature coefficient of –2500 mV/0C

- then this transducer is indeed a

- a)Thermistor

- b) Forward biased pn junction diode

- c) Reverse biased pn junction diode

- d) FET

- 7. The location of lightning arrestor is -

- a) Near the transformer

- b) Near the circuit breaker

- c) Away from the transformer

- d) None

- 8. Time constant of an RC circuit increases if the value of the resistance is -

- a) Increased

- b) Decreased

- c) Neither a nor b

- d) Both a and b

- 9. Intrinsic semiconductors are those which -

- a) Are available locally

- b) Are made of the semiconductor material in its purest from

- c) Have more electrons than holes

- d) Have zero energy gaps

- 10. The primary control on drain current in a JFET is exerted by -

- a) Channel resistance

- b) Size of depletion regions

- c) Voltage drop across channel

- d) Gate reverse bias

- 11. The electrical conductivity of metals which is expressed in ohm-1 m-1 is of the order of -

- a) 1010

- b) 105

- c) 10-4

- d) 10-6

- 12. When biased correctly, a zener diode –

- a) acts as a fixed resistance

- b) has a constant voltage across it

- c) has a constant current passing through it

- d) never overheats

- 13. The current amplification factor adc is given by –

- a) IC/IE

- b) IC/IB

- c) IB/IC

- d) IB/IC

- 14. Compared to bipolars, FETs havea)

- high input impedance

- b) low input impedance

- c) same input impedance

- d) none

- 15. The source-drain channel of JFET is -

- a) ohmic

- b) bilateral

- c) unilateral

- d) both a and b

- 16. diac is equivalent to a -

- a) Pair of SCRs

- b) Pair of four layer SCRs

- c) Diode and two resistors

- d) Triac width

- 17. When a sample of N type semiconductor has electron density of 6.25 ´ 1011 /cm3 at 300K and if

- the intrinsic concentration of carriers in this sample is 2.5 ´ 1013/cm3 then the hole density will be –

- a) 106/cm3

- b) 103/ cm3

- c) 1010/ cm3

- d) 1012/ cm3

- 18. The statement ‘In any network of linear impedances, the current flowing at any point is equal to the

- algebraic sum of the currents caused to flow at that point by each of the sources of emf taken separately

- with all other emf’s reduced to zero’ represents -

- a) Kirchhoff’s law

- b) Norton’s theorem

- c) Thevenin’s theorem

- d) Superposition theorem

- 19. One of the following modes which has the characteristics of attenuation becoming less as the

- frequency is increased and is attractive at icrowave frequencies of circular cylindrical wave guides is –

- a) TE1 mode

- b) TM01 mode

- c) TE01 mode

- d) Higher order mode

- 20. A two-port network is symmetrical if –

- a) z11z22 – z12z21 = 1

- b) h11h22 – h12h21 = 1

- c) AD – BC = 1

- d) y11y22 – y12y21 = 1

- 21. For transmission line load matching over a range of frequencies, it is best to use a

- a) balun

- b) broad band directional coupler

- c) double stub

- d) single stub of adjustable position

- 22. The poles and zeros of a driving point function of a network are simple and interlace on the

- negative real axis with a pole closest to the origin. It can be realised -

- a) by an LC network

- b) as an RC driving point impedance

- c) as an RC driving point admittance

- d) only by an RLC network

- 23. Poles and zeros of a driving point function of a network are simple and interlace on the jw axis. The

- network consists of elements –

- a) R and C

- b) L and C

- c) R and L

- d) R, L and C

- 24. For a two port reciprocal network, the output open circuit voltage divided by the input current is

- equal to –

- a) B

- b) Z12

- c) —

- d) h12

- 25. In a short electric doublet the radiation properties are so thata)

- The induction field diminishes as the square root of the distance and is only appreciable in the

- vicinity of the conductor.

- b) In the radiation, magnetic field is minimum when the current is maximum.

- c) The radiation resistance of a short doublet antenna is extremely high.

- d) Mean rate of power through a unit area of spherical sphere surrounding this doublet is proportional

- to the square of the elemental length, other factors remaining constant.

- 26. The frequency modulated (FM) radio frequency range is nearly -

- a) 250 –300 MHz

- b) 150 – 200 MHz

- c) 90 – 105 MHz

- d) 30-70 MHz

- 27. In an underground cable the distortion in the transmission of carrier frequency can be eliminated by

- using -

- a) Inductive loading

- b) Resistive loading

- c) Capacitive loading

- d) Shielding

- 28. The characteristic impedance of a transmission line with inductance 0.

- 29. 4 mH /m and capacitance

- 60 pF/m is -

- a) 49 W

- b) 60 W

- c) 70 W

- d) 140 W

- 30. For a quarter wavelength ideal transmission line of characteristic impedance 50 ohms and load

- impedance 100 ohms, the input impedance will be –

- a) 25W

- b) 50W

- c) 100W

- d) 150W

- 31. The depth of penetration or skin depth for an electromagnetic field of frequency ‘f’ in a conductor

- of resistivity r and permeability m is

- a)inversely proportional to r and f and directly proportional to m

- b) directly proportional to r and inversely proportional to f and m

- c) directly proportional to f and inversely proportional to r and m

- d) inversely proportional to r and m and directly proportional to f

- 32. When an antenna has a gain of 44dB then assuming that the main beam of the antenna is circular in

- cross-section the beam width will be -

- a) 0.4456 0

- b) 1.44560

- c) 2.44560

- d) 3.44560

- 33. Lens antennas used for microwaves are usually made of -

- a) Polystyrene

- b) Glass of low refractive index

- c) Paraboloid surfaces

- d) Dielectric media having large refractive index

- 34. One of the following types of instrument which is an electrometer is -

- a) Electrodynamometer

- b) PMMC

- c) Electrostatic

- d) Moving iron

- 35. When an ac current of 5A and dc current of 5A flow simultaneously through a circuit then which of

- the following statement is true ?

- a) An ac ammeter will read less than 10A but more than 5A

- b) An ac ammeter will read only 5A

- c) A dc ammeter will read 10A

- d) A dc ammeter will read zero

- 36. When Q factor of a circuit is high, then -

- a) power factor of the circuit is high

- b) impedance of the circuit is high

- c) bandwidth is large

- d) none of these

- 37. The resolution of a logic analyser is -

- a) the maximum number of input channels

- b) the minimum duration of the glitch it can capture

- c) it’s internal clock period

- d) the minimum amplitude of input signal it can display

- 38. A memoryless system is –

- a) causal

- b) not causal

- c) nothing can be said

- d) none

- 39. An air capacitor is a –

- a) time variant

- b) active device

- c) time invariant

- d) time invariant and passive device

- 40. Thermistors are made of -

- a) pure metals

- b) pure insulators

- c) sintered mixtures of metallic oxides

- d) pure semiconductor

- 41. Pirani gauge is used to measure –

- a) very low pressures

- b) high pressures

- c) pressures in the region of 1 atm

- d) fluid flow

- 42. These circuits converts input power at one frequency to output power at a different frequency

- through one stage conversion –

- a) AC voltage controllers

- b) Cyclo converters

- c) Phase controlled rectifiers

- d) Inverters

- 43. In a forward voltage Triggering thyristor changes from –

- a) off state to on state

- b) on state to off state

- c) on state to on state

- d) off state to off state

- 44. A thyristor, when triggered, will change from forward blocking state to conduction state if its anode

- to cathode voltage is equal to -

- a) peak repetitive off state forward voltage

- b) peak working off state forward voltage

- c) peak working off state reverse voltage

- d) peak non-repetitive off state forward voltage

- 45. Gate characteristic of a thyristora)

- is a straight line passing through origin

- b) is of the type Vg = a + bIg

- c) is a curve between Vg and Ig

- d) has a spread between two curves of Vg – Ig

- 46. A four quadrant operation requires)

- two full converters in series

- b) two full converters connected back to back

- c) two full converters connected in parallel

- d) two semi converters connected back to back

- 47. If for a single phase half bridge inverter, the amplitude of output voltage is Vs and the output power

- is P, then their corresponding values for a single phase full bridge inverter are –

- a) Vs, P

- b) Vs/2, P

- c) 2Vs, 2P

- d) 2Vs, P

- 48. In an enhancement type MOSFET the output V-I characteristics has –

- a) only an ohmic region

- b) only a saturation region

- c) only ohmic region at 10 W voltage value followed by a saturation region at higher voltages

- d)an ohmic region at large voltage values preceded by a saturation region at lower voltages

- 49. The energy gap in a semiconductor -

- a) increases with temperature

- b) remains constant

- c) slightly increase with temperature

- d) decrease with temperature

- 50. In an electronic circuit matching means -

- a) connecting a high impedance directly to low impedance

- b) selection of components which are compatible

- c) transferring maximum amount of signal between different kinds of circuits.

- d) RC coupled stages

- 51. P channel FETs are less superior than N channel FETs because

- a) They have higher input impedance

- b) They have high switching time

- c) They consume less power

- d) Mobility of electrons is greater than that of holes

- 52. Small increase in temperature in the CE connected transistor is the -

- a) Increase in ICEO

- b) Increase in ac current gain

- c) Decrease in ac current gain

- d) Increase in output resistance

- 53. An amplifier has a band width of 20 KHz and a midband gain of 50 without feedback. If a negative

- feedback of 1% is applied then bandwidth with feedback is -

- a) 13. 3 KHz

- b) 30KHz

- c) 10KHz

- d) 40KHz

- 54. The output of a class B amplifier -

- a) is distortion free

- b) consists of positive half cycles only

- c) is like the output of a full wave rectifier

- d) comprises short duration current pulses

- 55. An amplifier with negative feedback -

- a) lowers its lower 3 dB frequency

- b) raises its upper 3 dB frequency

- c) increases its bandwidth

- d) all of the above

- 56. What changes would be necessary in block C if FM signals are to be received -

- a) Block becomes redundant

- b) A FM detector would be required

- c) A high frequency signal generator

- d) An additional local oscillator will be needed

- 57. The main disadvantage of Diode-Transistor logic (DTL) is its

- a)greater speed

- b) slower speed

- c) average speed

- d) none of the above

- 58. Time delay Dt in digital signals in an SIS O shift register is given by –

- a) Dt = N ´ Fc

- b) Dt = N ´ 1/Fc

- c) Dt = 1/N ´ Fc

- d) Dt = N ¸ 1/Fc

- 59. The output Qn is 1 in a JK flip flop and it does not change when clock pulse is applied) The

- possible combination of Jn and Kn can be –

- (y denotes don’t care)

- a) y and 0

- b) y and 1

- c) 0 and y

- d) 1 and y

- 60. Basic memory cell of dynamic RAM consists of –

- a) a flip flop

- b) a transistor acting as a capacitor

- c) a transistor

- d) a capacitance

- 61. The 2’s complement of 10002 is –

- a) 0111

- b) 0101

- c) 1000

- d) 0001

- 62. Master slave flip-flop is made up of –

- a) two flip flops connected in series

- b) two flip flops connected in parallel

- c) a debouncer circuit

- d) a-D- latch

- 63. Number of nybbles making one byte is –

- a) 2

- b) 4

- c) 8

- d) 16

- 64. The intrinsic impedance of free spacea)

- is independent of frequency

- b) decreases with increase of frequency

- c) increases with increase of frequency

- d) varies as square root of frequency

- 65. A system consists of 12 poles and 2 zeroes. Its high frequency asymptote in its magnitude plot has a

- slope of -

- a) –200 dB/decade

- b) –240 dB/decade

- c) –230 dB/decade

- d) –320 dB/decade

- 66. Considering the conditions-

- 1. High loop gain 2. Less ringing 3. Greater damping 4 Negative dB gain margin

- System stability requirements would include?

- a) 1 and 3

- b) 1, 2 and 3

- c) 1 and 4

- d) 2, 3 and 4

- 67. In the equatorial plane only Geosychronous satellite are launched because it is the only plane which

- provides –

- a) 24 hour orbit

- b) stationary satellite

- c) global communication

- d) zero-gravity environs

- 68. Radio Broadcasting is an example of –

- a) space multiplexing

- b) time multiplexing

- c) frequency multiplexing

- d) none of the above

- 69. PAM signals can be demodulation by using a –

- a) Low pass filters (LPE) alone

- b) A Schmitt trigger followed by a LPF

- c) A differentiator followed by a LPF

- d) A clipper circuit by a LPF

- 70. In an FDM receiver channels can be separated by using –

- a) AND gates

- b) Band pass

- c) differentiation

- d) Integration

- 71. The most common modulation system used for telegraphy isa)

- frequency shift keying

- b) two – tone modulation

- c) pulse code modulation

- d) single tone modulation

- 72. Use of varoctor diode in generation of modulated segial be

- a) FM generation only

- b) 100AM generation only

- c) PM generation only

- d) both PM and AM generation

- 73. In colour picture tube shadow mask is used to

- a)reduce x-ray emission

- b) ensure that each beam strikes only its own dots

- c) increase screen brightness

- d) provide degaussing for the screen

- 74. The circuit that separates composite video warefore from the sync pulses is

- a)the keyed AGC amplifer

- b) a clipper

- c) an integrator

- d) a sawtooth current

- 75. Band width of microwaves is

- a)1GHz -103 GHz

- b) 1GHz –100 GHz

- c) 1 GHz –10 GHz

- d) 1 GHz – 106 GHz

- 76. In transverse Magnetic mode

- a)no electric line is in direction of propagation

- b) no magnetic line is in direction of propagation

- c) bath magnetic & electric lines are is direction of propagation

- d) neither magnetic nor electric lines in direction of propagation

- 77. Signal transmission in sky wave propagation is due to –

- a) Refraction of wave

- b) Reflection of wave

- c) Pierus through Inosphere

- d) None

- 78. According to Barkhausen Criterion Phase shift of signal should be –

- a) 600

- b) 900

- c) 1800

- d) 3600

- 79. The transmission does not have -

- a) Partition noise

- b) Flicker noise

- c) resistance

- d) Short noise

- 80. Varoctor diode has non linearity of -

- a) capacitance

- b) Inductance

- c) Resistance

- d) Is a linear device

- 81. Noise figure is calculated as –

- a) i/p signal to noise ratio X o/p signal to noise ratio

- b) i/p S/N Ratio / O/P S/N Ratio

- c) i/p S/N Ratio / O/P S/N Ratio X 100

- d) i/p S/N Ratio + O/P S/N Ratio

- 82. You can determine quickly the effect of adding poles and zeros by –

- a) Nicholas chart

- b) Nyquist plot

- c) Bode plot

- d) Root locus.

- 83. The polar plot of G(S) = intercepts real axis at w = wo. Then, the real part and wo are given by

- a)–5, 1

- b) –2.5, 1

- c) –5,0-5

- d) –5, 2

- 84. Laplace transform F (s) of a function f (E) is given by

- The initial and final values of F (t) will be respectively

- a)zero and 1

- b) zero and 10

- c) 10 and zero

- d) 70 and 80

- 85. A satellite link uses different frequencies for receiving and transmitting in order to –

- a) avoid interference from terrestrial microwave links

- b) avoid interference between its powerful transmitted signals and weak in coming signal

- c) minimize free-space losses

- d) maximize antenna gain

- 86. The first determining factor in selecting a satellite system is its

- a)EIRP

- b) Antenna size

- c) Coverage area

- Antenna gain

- 87. Equalizing pulses in TV are sent during

- a)horizontal blanking

- b) vertical blanking

- c) the serrations

- d) the horizontal retrace

- 88. The son seems to have ——– from his father a somewhat gloomy and moody manner

- a)washed

- b) inherited

- c) admired

- d) attempt

- 89. Essayist works with words as sculptor with

- a)water

- b) stone

- c) air

- d) hills

- 90. What is a collection of sheep called ?

- a) bunch

- b) flock

- c) herd

- d) comet

- 91. Join these sentences meaningfully by choosing the correct alternative from the following :

- You can buy a book. You can read it.

- a) and

- b) nor

- c) either

- d) neither

- 92. What is the opposite of Asperity –

- a) gentility

- b) superiority

- c) kindness

- d) clarity

- 93. The Election Commission functions under

- a)Ministry of Home Affairs

- b) Ministry of Law

- c) Prime Minister’s Secretariat

- d) None of these

- 94. Article 352 of Indian Constitution needs to be revoked in case

- a) President’s Rule is to be imposed

- b) Emergency is declared

- c) Services of a Government servant are to be terminated without any enquiry

- d) A political party of national level is to be banned

- 95. Radio-activity was first discovered by

- a)Becquerel

- b) Madam Curie

- c) Rutherford

- d) Jenner

- 96. Ninth Plan in India ranges from

- a)1995-2000

- b) 1996-2001

- c) 1997-2002

- d) 1998-2003

- 97. How much electricity does India propose to generate through nuclear power by the year 2000 AD?

- a) 5,000 MW

- b) 10,000 MW

- c) 15,000 MW

- d) 20,000 MW

- 98. In which year did the fall of Bastille take place?

- a) 1769

- b) 1789

- c) 1889

- d) 1869

- 99. To form a quorum how many members of the Lok Sabha or Rajya Sabha should be present?

- a) 1/10th of total membership

- b) 1/6th of total membership

- c) 1/4th of total membership

- d) 1/5th of total membership

- 100. How may countries are non-permanent members of the Security Council?

- a) 6

- b) 7

- c) 9

- d) 10

- 101. The International Date Line is represented by

- a)1000 meridian

- b) 00 meridian

- c) 1800 meridian

- d) 900 meridian

- 102. India’s first satellite was launched from

- a) Sriharikota

- b) Cape Kennedy

- c) Bangalore

- d) A Soviet cosmodrome

- 103. Name the author of the famous book “Politics”-

- a) Aristotle

- b) Socrates

- c) Plato

- d) None of them

- 104. “Guernica” is Picasso’s painting on

- a)The Spanish Civil War

- b) The American Civil War

- c) The French Revolution

- d) The Russian Revolution

- 105. The object of the Supreme Court’s Keshvanand Bharati ruling is -

- a) To put a limit on Parliament’s amendatory powers

- b) To give unlimited powers to Parliament to amend the Constitution

- c) To give precedence to Directive Principles over Fundamental Rights

- d) None of these

- 106. Which country in July ‘99 officially announced mastering of indigenously developed neutron

- bomb technology?

- a) N. Korea

- b) France

- c) India

- d) China

- 107. Shifting cultivation is commonly used in which of the following states?

- a) Tamil Nadu

- b) Maharashtra

- c) Jammu and Kashmir

- d) Nagaland