- EIL Aptitude Paper 1

- 1. Non-tech. (50 ques.)

- 2. Technical (100 ques.) Duration: 2hrs

- Here we are submitting you the ques we remembered....

- Part-1

- Out of 50 ques hardly 3-4 were of general English, no ques.is from aptitude the rest all were GK ques.

- ques. Like...

- 1. Which country is rich in thorium..

- Ans. India

- 2. "Eco Mark" is used on products which are:

- Ans. Eco friendly.

- 3. Animax channel belong to which of the following:

- a) sony pictures, b)zee, c)star, d)disney

- 4. First nuclear plant in India was set up at:

- Ans. Tarapur

- 5. Recently India has done "Science Express Deal" with which of the following countries:

- a) Germany, b) Japan, c) China, d) Russia

- 6. Pandit Jwaharlal Nehru award was given first time to the president of which of the following

- countries:

- a) Brazil, b) Mexico, c) Austria, d) Sweden.

- 7. Kaziranga National Park is Famous for:

- a) Rhino, b) Zebra, etc....

- 8. APJ Abdul klam is related to which of the following missiles:

- a) ASLV, b) GSLV, c) SLV,etc..

- 9. Progen is the BPO of which company:

- a) Satyam, b) Wipro, c) Infosys, etc..

- 10. Bharat Parvasi Divas is celebrated on which day:

- Ans. 9th Nov.

- 11. Pongal is the festival belongs to which state:

- a) Kerela, b) Karnatka, c) Orissa, etc..

- 12. Which dance is known as the "Ballet of East":

- a) kathak, b) bharatnatyam, c) kuchipudi, d) manipuri

- 13. Sunderbans are famous for:

- a) Mangroves, b) Tropical forests, etc..

- 14. The famous Golfer Vijay Singh belongs to which country:

- a) Fiiji, b) keniya, c) Maldives, etc..

- 15. LTTE is related to which country:

- Ans. Shrilanka

- 16. Which of the following is not included in new 7 wonders:

- a) Taj Mahal, b) The Grand Canyon, c) machhi Puchhi, etc..

- 17. Who is known as the father of modren biology?

- a) Charles Darwin, etc...

- 18. P.Sainath is related to which of the following:

- a) TV Anchoring, b) Drama, c) Mountaineering, etc...

- 19. The elephants, the tiger, the cell is written by:

- a) Sashi Throor, etc....

- 20. Anandmath is written by which author:

- a) Rabindernath Tagore, b)BC Chatterji, c) Premchand, etc...

- 21. Bessel 2 norms are related to:

- a) Cyber crimes, etc...

- 22. Steve Fosset is related to:

- a) Crocodile Hunting, etc...

- 23. Dr. MS Swaminathan is related to :

- a) planning comission, b) sixth pay comission, c) national farmers comission, etc..

- 24. First soundless movie of Bolyywood:

- Ans. Alam Ara

- 25. What is the meaning of Dime a Dozen...?

- a) easy to get, b) easily available,etc...

- 26. Nightmare: Dream

- a) Dread: Expectation, b) Frustration: Confusion,etc..

- 27. Meaning of Concur:

- a) to deny, b) Unusual, etc...

- 28. Golden Arches is related to which of the following:

- a) KFC, b) Mcdonals, etc...

- 29. Meaning of: "The way the wind blows" :

- a) way the things were , b) way the things are, etc...

- 30. I would hope that you ____ take tea with me 2mrw.

- a) shall, b) will, c) would, d) could

- 31. In Ambulance which of the aural postural balance is difficult to maintain:

- a) nose, b) eyes, c) ears, d) hands

- 32. the next olympics wiil be held in which country:

- a) london, b) New York, c) Paris, d) Switzerland

- 33. Where is the OPEC head office located?

- Ans Viena

- upto 50 (but i remembered this much..)

- Part-2 (Technical-100 ques.)

- ques like...

- 1. Dimensions of Power:

- a) M1L2T-3, b) M1L2T-2, c) M1L1T-3, etc...

- 2. Torque in Induction Motor is directly proportional to:

- a) V, b) V(pwr)2, c) V(pwr)3, d) V(pwr)1/2

- 3. Power in Transmission lines is proportional to:

- a) V, b) V(pwr)2, c) V(pwr)3, d) V(pwr)1/2

- 4. Flashover at the surface of condenser bushings is due to:

- a) moisture in air, etc..

- 5. When Surge impedence equal characterstic impedance then :

- a) Vs = Vr, b) Vs> Vr, c) Vs< Vr, etc..

- 6. Stability limit is higher in:

- a) Steady state, b) Transient state, c) Sub transient state, etc..

- 7. Those instruments which give constants and direction only are called:

- a) Secondary type, b) Absolute type, c) Deflection type, d) Integrating type.

- 8. The internal resistance of a meter is 4 ohm and current drawn is 30mA. If we want to increase its

- range as voltmeter upto 150V then find out the series resistance added in the circuit?

- Ans. 4996 ohm

- 9. 1 ques like... If relay setting is at 125% and the CT ratio is 400/5, then the fault current in hv winding

- is?

- Ans. 6.25 amp.

- 10. If capacitve reactance of a 200 km line is 2000 ohm then what will be the charging reactance of a

- 100 km line?

- a) 2000 ohm, b) 4000ohm, c) 500 ohm, d) 8000ohm.

- 11. Which is wrong:

- a) Transient response is frequency domain.

- b) Routh criteria is stability based.

- c) Nyquist is time domain.

- etc....

- 12. With the increase in feedback of the system which will increase:

- a) stability, b) gain, etc...

- 13. Mechenical stress is max. at which part of the cable:

- a) core, b) sheath, etc...

- 14. Underground cable laying is done by:

- ans. conduits and piping.

- 15. Operation and maintenece cost is min. in which of the dollowing plants:

- a) Thermal, b)Nuclear, c) Hydel, d) Diesel.

- 16. 1 ques like.... earliest power plant is:

- a) Thermal, b) Nuclear, c) Unconventionanal, etc....

- 17. Circuit Breaker arcing contacts are made of which material:

- a) copper tungsten alloy, b) copper, c) silver, d)graphite.

- 18. The reflection cofficient for voltage of a short circuited line is:

- a) -1, b) 1, c) 0, d) 2

- 19. Double cage rotor is used in Induction motor is to improve:

- Ans. starting torque.

- 20. If resistance of the secondary winding of the transformer is given as .02ohm and reactance .06ohm

- then find out the primary resistance and reactance respectively:

- Ans. .08 ohm and .24ohm

- 21. Skewing in the squirrel cage rotor of Induction motor is purposely done to :

- Ans. to avoid interlocking and reduce noise.

- 22. 1 ques like.......capacitance of 3phase line is given 4microF. if we consider single line to

- groundthen its capacitance will be:

- a) 4microF, b) 2microF, c) 1microF, d) 8microF

- 23. The normal frequency RMS voltage that appears across the breaker poles after final arc extinction

- is known as:

- ans. Recovery voltage

- 24. 1 ques like...... In power factor meter , which element is used as shunt in instruments:

- a) silver, b) copper, c) manganin, etc...

- 25. 1 ques like...If anode of the SCR is made positive and cathode is made negative thenit will be:

- ans. forward biased and conducting.

- 26. 1 ques like.... Natural commutation is done in...

- a) cycloconverters, b) choppers, c) AC voltage controllers, etc.

- 27. 1 ques was based on....... Class B insulation is done in :

- a) DC choppers , etc.....

- 28. the highest range of temperature of insulations is...

- Ans. 180

- 29. temperature of the boiler furnace is measured by:

- Ans. optical pyrometer

- 30. Resistivity of insulators with temperature

- a) decreases linearly, b) decreases exponentially, c) increases, d) remains same.

- 31. On increasing the frequency of the transformer , hysterisis loss and eddy current loss respectively:

- a) decreses and remains same, b) decreases and increases, c) both increases , d) both decreases.

- 32. Rotor copper losses are small in Induction Motor as compared to transformer becoz:

- Ans. Rotor core is laminated.

- 33. 1 ques like.... arrange the following in ascending order:

- resistance split, capacitor start capacitor run etc motors were given...

- 34. Which type of turbine is used in tydel power generation:

- a) Kaplan, b) Fransis, c) Pelton Wheel type, d) Reverse type.

- 35. Which of the following is the distribution 3 phase voltage:

- a) 220 V, b) 415 V, c) 680 V, d) 800 V.

- 36. Which of the following bridges is used to measure dielectric losses in capacitor:

- a) Schearing Bridge, b) Wein's Bridge, c) De-Sauty Bridge, d) Maxwell Bridge.

- 37. 1 ques is based on..... Cros have which of the following...

- a) Vertical and Horizontal plates, b) sweep, etc...

- 38. 1 ques like..... A full wave centre tap thyristor is given.. find out the PIV..? dnt remember exactly

- some values were given...

- a) 141.3 V, b) 100 V, c) 248.2 V, etc...

- 39. Speed of the Induction Motor at full load is 1400 rpm , what will be the speed at half the load...

- a)1400rpm, b) 1420rpm, c) 1440rpm, d) 1460rpm.

- 40. The line feeding from distribution system to the consumers is called:

- a) feeder, b) service mains, c) distributor, etc...

- 41. Which of the following generation plant is most reliable.

- Ans. Interconnected system.

- 42. 1 ques like.... load current in thyristor depends upon.....

- a) delay angle and load type, b) delay angle but not on load type, c) only delay angle, etc...

- 43. Buchholz relay is:

- a) a gas actuated relay, b) current sensitive relay, c) overcurrent relay, d) over voltage relay.

- 44. Diac is a:

- a) 4 terminal 2junction device, b) 3 terminal 2 junction device, c) 5 terminal 3 junction device,

- d) 6 terminal 3 junction device.

- 45. Dual converter consists of:

- Ans. 1 converter and 1 inverter.

- 46. Slip is min. for which of the following machine:

- a) 25HP,4 Pole , b) 1HP, 4 pole, c) 2 HP , 6 pole, d) 25 HP, 6 Pole.

- 47. In Air Blast circuit breakers the air pressure inside is:

- a) 1kg/cm2, b) 100 mm of Hg , c) 20-30 kg/cm2, d) 200-300 kg/cm2.

- 48. 1Barrel contains how much litres:

- a) 216, b) 159, etc....

- 49. DC Choppers are used to convert:

- a) DC to DC, b) AC to DC, c) AC to AC, etc....

- 50. 1 ques like.... how the corona effect varies with size and frequency, it increases with..

- a) increasing size and reducing supply frequency, b) reducing size and incresing supply frequency,

- etc...

- 51. 1 ques was based on..... in frequency variable method which of the following factor is kept

- constant:

- a) V/f = constant, b) Vf = constant, c) V=constant, d) none of these..

- 52. 1 ques was based on ..... relation of Voltage with load angle delta..... options were...

- a) V/1-& , b) V/1+&, c) 1-&/&V, d) &/(1-&)V

- 53. 1 ques like.... Universal motor is:

- a) frequency dependent, b) frequency independent, etc...

- 54. If CT winding connections are made in star/delta on the primary side of the T/F then CT

- connections on the secondary side winding of T/F will be:

- Ans. delta/star.

- 55. slip rings in DC machines are made of:

- a) carbon, b) graphite, c) copper alloy, d) copper.

- 56. 1 question like.... bending radius of the cable should be:

- a) D, b)2D, c) 3D, d) 4D

- 57. 1 ques like..... In cycloconverters the O/P frequency power factor will bw as compared to I/P

- frequency power factor:

- a) higher, b) lower, c) same..

- 58. In higher rating transformers the tank is of the following type:

- a) radiator, b) oil tank, c) seperator, etc.

- 59. Merz Price protection is used in :

- a) Alternator, b) Transmission lines, c) Transformer, etc.

- 60. Meggar is:

- a) moving coil type, b) moving iron type, etc.

- 61. Differential relay is:

- a) over current relay, b) over voltage relay, etc.

- 62. 1 ques like....Two transformers are connected in parallel R/X is smthing..... then the currents in the

- two T/F will be:

- a) Ia lags Ib , b) Ia leads Ib, c) both will be in phase, etc....

- 63. skin effect does not depend upon:

- Ans. ambient temperature.

- 64. 1 ques like.... AC resistance is more due to....

- Ans. skin effect

- 65. Human body gets shock due to:

- a) static charge discharging of body, b) voltage induced in the body, c) current induced in the

- body, d) none of these.

- 66. Heavy water in nuclear reactor is used as:

- a) accelerating agent, b) decclerating agent, c) lubricating agent, d) cooling agent.

- 67. specifications of earth plate are:

- a) 60* 60*12.5 , b) 90*90*12.5 , etc.

- 68. Distance of ground electrode from building is:

- a) 1m, b) 2m, c) 3m , d)4m

- 69. Why earth pit is filled with alternate layers of salt and charcoal:

- Ans. to reduce earth's resistance.

- 70. If cos& is a power factor, then MVAR ia proportional to:

- a) cos&, b) sin&, etc...

- 71. 1 ques like.....why central phase is reversed.....(smthing dnt remember exactly)

- 72. capacitors are connected where at the transmission line:

- a) at sending end, b) at receiving end, c) in the middle of the line, etc.

- 73. Which gas is used in the gas fired prime movers at gas power plants:

- a) LPG, b) CNG, etc.

- EIL Aptitude Paper 2

- 1. How many litre are in one barrel? RO: 159

- 2. Where is the OPEC head office located? RO: Viyana

- 3. Which is the fiest Silence move? RO: Harischanra

- 4. Animax channel belongs to which channel? RO: Diseny

- 5. Whose wrote a famous book "AnandMatha" ? RO: B.C Chetergee

- 6. Which Institute A.P.J.Abdul Kalam associated? RO: PSLC

- 7. Which Country was the fisrt impose Corban Test?

- 8. Purgeon BPO unit associated with which Company?

- 9. Who is the first president of INC?

- 10.Which Prize is alternative of Nobel Prize? RO: Child livelihood

- 11.Nilgiri is famous for which crops?

- 12.At which state Ganga & Alakhnanda meet?

- 13.Which country impose euthenesia first law?

- 14."Lithe" is belongs to which country?

- 15.P. Sainath belongs to which proffession?

- 16.M.S Swaminathan is belong to which commission?

- 17."The Cell, the elephant & the snak" who wrote this book?

- 18."pongal" festival belongs to which state?

- 19.Which dance is called "East of Bellet"?

- 20.Kangiranga park is famous for which animal? RO: Rihno

- 21.The famous golfer Vijay Singh is belong to which country? RO: Fiji

- 22.At which date "Pravasi Diwas" celebrated? RO: 9 jan

- 23.Jawaharlal Nehru award is given to which country presidence in 2006?

- 24.2012 Olympic will be held on which country? RO: London

- 25.Golden arches product belong which country?

- Some more questions are given on current affairs, 5-6 questions are from English crammer (easy) , if u

- have the habit of reading newspaper then that questions will be easy. These are some questions from

- general awareness. (50 questions in this section).

- EIL Aptitude Paper 3

- General Awareness section

- 1) Name the first country to impose CARBON TAX

- 2) The author of the book “The Elephant, the Tiger and the Cellphone” - Shashi Tharoor

- 3) Which dance form is called “Ballet of East”

- 4) Where the 2012 Olympics are going to be held - London

- 5) “Progeon” is the BPO of which company

- (a) Infosys (b) Satyam (c) Wipro (d) HCL

- Ans: Infosys

- 6) Which prize is known as the Alternative Nobel Prize

- Ans: Right Livelihood Award (not sure)

- 7) How many litres are equal to One barrel - 159 litres

- 8) Where is the OPEC headquarter located - Vienna, Austria

- 9) Name the first country to legalize EUTHANASIA

- 10) ANIMAX is of which channel

- (a) Sony (b) Disney (c) Zee (d)

- Ans: Sony

- 11) P. Sainath is associated with - Journalism

- 12) Golfer Vijay Singh is from which country - Fiji

- 13) Dr. M.S.Swaminathan is associated with

- (a)Planning Commission (b) Sixth Pay Commission

- (c) National Commission on Farmers (d) Administrative Affairs Association

- Ans: National Commission on Farmers

- 14) Name the first silent Indian movie - Raja Harishchandra

- 15) Who was the first women president of Indian National Congress - Sarojini Naidu

- 16) Which of the following is not in the SEVEN WONDERS OF THE WORLD

- Ans: don’t remember the options

- 17) Who is the father of modern biology

- (a)Aristotle (b) Mendel (c) (d)

- 18) “Science Express” is launched with the help of which country

- (a)Germany (b) Japan (c) (d)

- Ans: Germany

- 19) “Concur” means – to agree

- 20) Steve Fosset is associated with - Adventure

- 21) LTTE is associated with – Sri Lanka

- 22) “Golden Arches” are associated with –

- (a)KFC (b) Mc Donalds (c) Nirulas (d)

- Ans: Mc Donalds

- 23) “Taj Mahal” was originally set to be built in which Indian state-

- (a)Rajasthan (b) MP (c) Orissa (d) Maharashtra

- Ans: Maharashtra (Burhanpur)

- 24) “dime a dozen” means – extremely common and cheap

- 25) “way wind blows” means – how things are going to be

- 26) “e-choupal” initiative was taken by – ITC

- 27) When is PRAVASI BHARATIYA DIWAS celebrated - 9 January