- EIL Technical Paper 1

- Technical section:

- 1) Bucholz relay is..................

- a. Gas actuated relay

- b. Current sensing device

- c. Voltage sensing device

- Ans: a

- 2)Shunt is made up of .........

- a. Copper

- b. Tungsten

- c. Maganin

- Ans: c

- 3)In a 4kva ,400/200 v m/c the value of impedance & reactance are given as 0.002 & 0.006 .what will

- be the new values of them when referred to the h.v side.

- a. 0.2,0.6

- b. 0.08,0.036

- c. 0.4,0.12

- 4)Metallic sheath is used to .........

- a. prevent the cable from moisture

- b. provide mechanical strength

- Ans: b

- 5) Of the following bridges the one which can be used for the measurement of dielectric loss of a

- capacitor is –

- a. Schering bridge ..........

- b.Heaviside campbell equal ratio voltage

- c. Owen bridge

- d. Anderson bridge

- Ans. a. Schering bridge

- 6)Diac is a...............

- a. 3-layer ,2-junction

- b. 2-layer,3-junction

- c. 4-layer,3-junction

- 7)Load voltage equation of step up chopper.........

- a. v(1-a)

- b. v/(1-a)

- Ans: b

- 8)Which is not realated to ckt breaker.........

- a. conservator

- b. explosion pot

- c. operating mechanism

- Ans: a

- 9) Coefficient of reflection of short transmission line is........

- a. 1

- b. -1

- c. 0

- d. infinite

- Ans: b

- 10)1st nuclear power plant located at .......

- a. trombay

- b. kalpakkam

- c. tarapur

- Ans: tarapur

- 11)If a current setting is given as 25%, ct ratio is 400/5 then find pick up value......

- a. 6.25

- b. 0.25

- c. 4

- Ans: a

- 12. d.c chopper converts .......

- a. ac to ac

- b. ac to dc

- c. dc to dc

- d. dc to ac

- Ans:c

- 13)In a controlled rectifier , the nature of load current i.e, whether load current is continous or

- discontinous is

- a. does not depend on type of load & firing angle delay

- b. depends on load & firing angle delay

- c. depends only on type of load

- d. depends only on firing angle delay

- Ans:b

- 14)For thyrister pulse triggering is preferred on dc trigerring because..........

- a. gate dissipation is low

- b. pulse system is simpler

- c. triggering system is required for very short duration

- d. all of these

- Ans:d

- 15)Thyristers are basically ..........

- a. scr’s

- b. triac’s

- c. both scr’s and triac’s

- d. all pnpn devices

- Ans : d

- Non tech section

- The nontech section comprises of only g.k questions

- 1)Vijay singh, who is golf player belongs to .................

- a. maritus

- b. Maldives

- c. Kenya

- 2)LTTE associated with...........

- a. Bangladesh

- b. Bhutan

- c. shrilanka

- d. Nepal

- Ans:a

- 3)S.Sainath related to.........

- a. journalism

- b. t.v hosting

- 4)OPEC ‘s headquarter is located at ...............

- a. Vienna

- b. gienna

- c. maxico

- Ans:a

- 5)Jawaharlal Nehru award given to which country...........

- a. mexico

- b. u.s.a

- 6)Which contry first started carbon tax...........

- 7)Nobel prize is equivalent to

- a. raman magsasse

- b. abel prize

- 8)The book “the elephant ,tiger and……………..” is written by

- a. artya sen

- 9)First women leader of INC was

- a. Anne becent

- b.Sarojini naidu

- Ans:a

- 10)The book Anand math is written by.............

- a.B.C Chatterji

- b. Rabindranath tagorre

- 11)APJ abdul kalam associated with..........

- a. gslv

- b. pslv

- c. slv

- 12)Nilgiri hills is famous for ..........

- a. tea

- b. coffee

- c. rubber

- d. spices

- 13)Olympic 2012 are going to be held at

- a. London

- b. Bejieng

- Ans: London

- 14)The sentence ‘the way the wind flows ‘means........

- a. how the things are

- b. how the things are made

- c. how we make the things

- 15)I hope that u _______ come for a cup of tea tomarrow

- a. shall be

- b. would be

- c. will be

- 16)Pongal is a festival of

- a. Karnatka

- b. Andra Pradesh

- c. West Bengal

- d. Tamil nadu

- 17)Tajmahal was to be built originally at.........

- a. M.P

- b. Orissa

- c. West Bengal

- c. Rajasthan

- 18)Pravasi bhartiya divas was celebrated on........

- a. 9 jan

- b. 16 april

- Ans:a

- 19)Kajiranga national park is famous for........

- a. ryhno

- b. lion

- c. deer

- Ans: a