- GATE Architecture Engineering Sample Papers

- Q-1 Unlike the Greeks, the Romans often used more than one order of columns in tiered arcades. Which order was almost invariably the base?

- a) Ionic

- b) Doric

- c) Corinthian

- d) Composite

- Q-2 Which Roman author wrote what became the standard text on classical architecture?

- a) Marcus Agrippa

- b) Isidore of Miletus

- c) Anthemius of Tralles

- d) Vitruvius

- Q-3 What would be most effective in reducing the deflection of a uniformly-loaded, simplysupported beam?

- a) Reducing the load by 10%

- b) reducing the span by 10%

- c) increasing the moment of inertia, I, by 10%

- d) all of the above are equally effective

- Q-4 What is the internal axial tension force in the tension tie-rod for the truss shown below?

- a) 55KN b) 27.5KN c)0 KN d) None

- Q-5 . Which if the following spray painting methods listed below has the highest transfer

- efficiency?

- a). Conventional air spray

- b). Airless spray

- c). High-volume low-pressure spray

- d). Electrostatic spray

- Q-6 . Which of the following admixtures should be added to accelerate concrete setting?

- a.) Sodium chloride

- b.) Sodium carbonate

- c.) Potassium chloride

- d.) Calcium chloride

- Q-7 Which of the following is NOT a necessary condition for condensation?

- a) Saturation

- b) high altitude

- c) dew-point temperature reached

- d) surfaces

- Q-8 What is the dewpoint when the dry-bulb temperature is 24°C and the wet-bulb temperature is 15°C?

- a) 8°C

- b) -18°C

- c) 36°C

- d) 4°C

- Q-9. Which is the incorrect colour coding as per Vaastu for the specified element

- a) Water = Blue

- b) Fire = Red

- c) Ether = White

- d) Earth = Yellow

- Q-10 which is not a correct match for structure with a likely coating or treatment:

- a) Steel piping (atmospheric service) = Organic zinc-rich primer; epoxy-polyamide midcoat; aliphatic

- polyurethane topcoat

- b) Lining for a chemical storage tank = FRP, vinyl ester; or epoxy resin, depending on specific product

- c) Ships, to prevent fouling = Copper oxide polymer

- d) New marine pilings = Three coats epoxy-polyamide; two coats coal tar epoxy

- Q-11 In what architectural style was the Temple of Athena constructed?

- a) Composite

- b) Doric

- c) Ionic

- d) Corinthian

- Q-12 The first known Corinthian column capital was seen in the construction of which temple?

- a) The Temple of Apollo

- b) The Temple of Artemis

- c) The Temple of Athena

- d) The Temple of Hephaestus

- Q-13 ‘True Style’ later came to be known as _________________.

- a) Mannerism

- b) Carolingian

- c) Neoclassicism

- d) Gothic

- Q-14 Which of the following, one of the Seven Wonders of the Ancient World, was burnt to the ground?

- a) The Temple of Apollo

- b) The Temple of Athena

- c) The Temple of Artemis

- d) The Temple of Hera

- Q-15 Converted into a church during the Byzantine period was which of the following temples, built in the Doric style?

- a) The Temple of Paestum

- b) The Temple of Hera

- c) The Temple of Attalus

- d) The Temple of Hephaestus

- Q-16 What type of building was the Stoa of Attalus?

- a) A shopping mall

- b) An ampitheatre

- c) A temple

- d) A storehouse

- Q-17 Of what materials was the statue of Athena that stood in the Parthenon made?

- a) Gold and Limestone

- b) Marble and Gold

- c) Ivory and Gold

- d) Marble and Ivory

- Q-18 The critical path for the given network is….

- a) 1-2-5-7-9

- b) 1-2-5-8-9

- c) 1-2-4-6-9

- d) 1-3-8-9

- Q-19. Which of the following is not correct for Stone masonry…

- a) In RUBBLE stone masonry, the stones are left in their natural state, without any kind of shaping

- b) In ASHLAR masonry, the faces of stones that are to be placed in surface positions are squared so

- that the surfaces of the finished structure will be more or less continuous plane surfaces.

- c) Both rubble and ashlar work may be either RANDOM or COURSED.

- d) Rubble stone masonry work is random but ashlar waork is always coursed

- Q-20. IF T( l ) = latest allowable occurrence time, T ( e) earliest expected time then which of the statement is not correct?

- a) if T( I ) is greater than T( e) then it is Positive slack

- b) if T( I ) is less than T( e) then it is negative slack

- c) if T( I ) is equal to T( e) then it is zero slack

- d) if T( I ) is not equal to T( e) then it is zero slack