- IFS Chemical Engineering 2007

- PAPER - I

- SECTION A

- 1. Answer any four of the following :

- (10 × 4 = 40)

- (a) Prove that the velocity profile in a laminar flow of a Newtonian fluid through a circular pipe

- is a parabola.

- (b) (i) Differentiate between differential and cumulative analysis to find the average size of

- the sample.

- (ii) Derive the expression for the critical speed of a ball mill. Mention the meanings of

- symbols and their units clearly.

- (c) Define humidification and dehumidification, Explain the working of a water cooling tower

- with a neat diagram.

- (d) Derive the relation between rate of heat transfer and resistances in series in a composite wall.

- 2. (a) Starting from the first principles, obtain Bernoulli’s equation from momentum balance in

- potential flow and extend the equation for practical applications.

- (10)

- (b) Define capacity and effectiveness of a screen. Derive an expression for the overall

- effectiveness of a screen.

- (10)

- (c) Explain different types of fluidisation and their applications.

- (10)

- (d) Give a critical account of Rittinger’s and Bond’s laws.

- (10)

- 3. (a) Explain the mechanism of drying during the constant rate and failing rate periods. Discuss the

- effects of humidity and velocity of the gas on drying rate.

- (10)

- (b) Mention the various types of crystallisation equipment used in the process industry. Explain

- the working of any one of the crystallisers with a neat diagram.

- (10)

- (c) Write the suitable conditions for liquid- liquid extraction. Discuss the importance of

- equilibrium distribution coefficient and selectivity in the choice of a solvent.

- (10)

- (d) Write the procedure in detail for the design of a distillation column by Mc Cabe-Thiele

- method.

- (10)

- 4. (a) Discuss briefly the various regimes in boiling heat transfer.

- (10)

- (b) Describe the construction of shell and tube heat exchanger with a neat sketch.

- (10)

- (c) Define the capacity and steam economy of an evaporator. Discuss the effect of boiling point

- elevation on evaporator performance.

- (10)

- (d) Air is heated in air heather from 30° C to 110°C and the hot gases are cooled from 210° C to

- 145°C. calculate the LMTD in parallel and counter glows. Give critical comments of the

- values obtained.

- (10)

- SECTION B

- 5. Answer any four of the following:

- (10 × 4 = 40)

- (a) Discuss various types of membranes.

- (b) Write the design procedure of either torispherical head or elliptical head.

- (c) Mention the principal stresses induced in a vessel. How they act over the vessel.

- (d) Differentiate between pneumatic and electronic controllers.

- (e) List the various liquid level measuring instruments. Give their range and applications.

- 6. (a) Compare and contrast the terms— osmosis” and “reverge osmosis”.

- (10)

- (b) Discuss the principles of ultrafiltration. Mention its applications.

- (10)

- (c) Describe the processes of dialysis and also of electrodialysis. Write their applications.

- (10)

- (d) Briefly discuss about “Super critical fluid extraction”.

- (10)

- 7. (a) Discuss the selection of material of construction for pressure vessels.

- (10)

- (b) Explain the design procedure for vertical type storage vessels.

- (c) Mention various types of heads commonly used in cylindrical vessels and list their

- applications.

- (10)

- (d) Define joint efficiency. Mention the factors influencing the choice of supports.

- (10)

- 8. (a) Explain the working principles involved in various pressure measuring instruments.

- (10)

- (b) (i) Explain the negative and positive feedback control system.

- (5)

- (ii) Explain the servo and regulatory control problems.

- (5)

- (c) What are the elements of computer based process control? Explain their functions.

- (10)

- (d) What are phase and gain margins? Discuss the bode stability criterion.

- (10)

- PAPER - II

- SECTION A

- 1. Answer any four of the following (Maximum 150 words):

- (a) Discuss reversible thermodynamic processes.

- (10)

- (b) Write short note on fugacity.

- (10)

- (c) Write short note on reactors in series.

- (10)

- (d) A solution of naphthalene, C10H8, in benzene, C6 H6, contains 25 mole per cent of

- naphthalene. Express the composition of the solution in weight percent.

- (10)

- (e) A solution of Sodium Chloride in water contains 230 grams of NaCl per liter at 20°C. The

- density of solution at this temperature is 1.148 grams per cubic centimeter, calculate :

- (i) Volumetric percent of water

- (ii) Composition in mole percent

- (iii) Modality.

- Note : density of water at 20° C = 0.998 g/cc.

- 2. (a) The liquid phase reaction,

- (where K1=8 liter/mol.min

- K2=3 liter/mol.min)

- is carried out in 150 litre stirred tank reactor under steady state conditions. Reactants A and B

- are fed to the reactor at equal volumetric flow rates. The concentrations of A and B streams

- are 2.8 mole A/ litre and 1.6 mole B/litre. What should be the flow rate of each stream to get

- 75% conversion of the limiting reactant?

- (25)

- (b) Prove that the heat transferred equals the enthalpy change of the system for a mechanically

- reversible, constant-volume, constant-pressure, non- flow processes.

- (15)

- 3 (a) A steel casting [Cp = 0.5 kJ kg-1 K-1] weighing 40 kg and at a temperature of 450°C is

- quenched in 150 kg of oil [Cp = 2.5 kJ kg-1 K-l at 25°C. If there are no heat losses, what is the

- change in entropy of :

- (i) the casting,

- (ii) the oil and

- (iii) both considered together? (25)

- (b) Derive the expression for the design equation of tubular reactor. (15)

- 4. (a) One litre/mm of liquid containing A and B [CA0 = 0.10 mol/litre, CBO =0.01 mol/litre] flow

- into a mixed reactor of volume of one litre. The materials react in a complex manner for

- which the stoichiometry is unknown. The outlet streak from the reactor contains A, B and

- C(CAf = 0.02 mol/litre, Cbt= 0.03 mol/litre, Ccf = 0.04 mol/litre). Find the rate of reaction of

- A, B and C for the conditions within the reactor. (20)

- (b) Chlorobenzene is nitrated using a mixture of nitric acid and sulphuric acid. During the pilot

- plant studies, a charge consisted of 100 kg chlorobenzene (CB), 106.5 kg 65.5% (by weight)

- nitric acid and 108 kg 93.6% (by weight) sulphuric acid. After two hours of operation, the

- final mixture was analysed. It was found that the final product contained 2% unreacted

- chlorobenzene. Also the product distribution was found to be 66% p-nitrochlorobenzene and

- 34% o-nitrochloro benzene, calculate:

- (i) The analysis of charge

- (ii) The percentage conversion of chlorobenzene.

- (20)

- SECTION B

- 5. Answer any four of the following in brief (Maximum 150 words):

- (a) PERT

- (10)

- (b) Break-even analysis

- (10)

- (c) Biogas

- (10)

- (d) Essential oils

- (10)

- (e) Norms for air emission.

- (10)

- 6. Describe manufacture of ammonia giving neat flow diagram and discuss its major pollution

- problems.

- (40)

- 7. (a) Suggest and justi1’ a location for a soda ash plant.

- (20)

- (b) Discuss greenhouse effect.

- (20)

- 8. (a) Discuss cracking in brief.

- (20)

- (b) The original value of a piece of equipment is Rs. 22,000. Its salvage value is Rs. 2,000 at the

- end of service life of 10 years. Determine the book value of the equipment at the end of 5

- years using:

- (i) Straight-line method

- (ii) Textbook declining-balance method

- (iii) Double declining-balance method.

- (20)

- IFS Chemical Engineering 2006

- PAPER - I

- SECTION A

- 1. Answer any four of the following

- (10 × 4 = 40)

- (a) (i) How is friction factor defined and what are the parameters on which it is dependent?

- (5)

- (ii) What are the different types of conveyers available for bulk solid transportation?

- (5)

- (b) Distinguish between ‘Adiabatic Saturation Temperature’ and ‘Wet Bulb temperature’.

- Explain why both these are same for air-water system.

- (10)

- (c) Discuss caking of crystals and its prevention.

- (10)

- (d) Distinguish between resistances in series and resistances in parallel through which heat is

- being conducted under steady-state condition. Explain using electrical analogy.

- (10)

- (e) Differentiate among nucleate boiling, pool boiling and subcooled boiling.

- (10)

- 2. (a) How do you use stress-strain relationship to explain the rheological classification of fluids?

- Explain with examples.

- (10)

- (b) Derive the continuity equation. State the assumptions.

- (10)

- (c) (1) List the advantages and disadvantages of plate and frame filter press.

- (5)

- (ii) State the two laws of crushing and write suitable mathematical relationships for the

- two laws.

- (5)

- (d) (i) Explain the basis for the selection of mixing impellers.

- (5)

- (ii) Briefly explain the mechanism of fluidization.

- (5)

- 3. (a) (i) Bring out clearly the concept of film transfer coefficient and overall transfer

- coefficient in mass transfer.

- (5)

- (ii) Explain briefly the diffusivity of gases and liquids.

- (5)

- (b) (i) Discuss the mass transfer resistances in a wetted column.

- (5)

- (ii) Write briefly on Shank system of leaching.

- (5)

- (c) With a neat drying rate curve, explain different zones of drying.

- (10)

- (d) What a1e the limitations of McCa1e-Thiele method? How are these overcome in Ponchon-

- Savarit method?

- (10)

- 4. (a) (i) What is LMTD, and why and where is it used?

- (5)

- (ii) Obtain the relationship between individual and overall heat transfer coefficients.

- (5)

- (b) (i) State and explain the Stefan-Boltzmann law of radiation.

- (5)

- (ii) Write a note on absorption of radiation by opaque solids.

- (5)

- (c) (i) What is the influence of boiling point elevation on evaporator performance?

- (5)

- (ii) Briefly describe a vapour compression evaporator.

- (5)

- (d) Write the design procedure for a double pipe heat exchanger.

- (10)

- SECTION B

- 5. Answer any four of the following

- (10 × 4 = 40)

- (a) Explain the principle, of reverse osmosis. What are its limitations and applications?

- (10)

- (b) List out the factors to be considered in the design of vessels.

- (10)

- (c) Name the different supports used for process vessels mentioning their advantages and

- disadvantages.

- (10)

- (d) Explain with sketches the working principles, installation and applications of thermocouples

- for temperature measurement.

- (10)

- (e) Explain the working of a PID control system. Draw a pneumatic control circuit for this

- system and explain the characteristics.

- (10)

- 6. (a) Explain the principle of ultra filtration and list out its industrial applications.

- (10)

- (b) Discuss the industrial applications of ion exchange process.

- (10)

- (c) Explain any two methods for separating a binary liquid mixture stating the underlying

- principles.

- (10)

- (d) Explain the working of an instrument based on electro dialysis.

- (10)

- 7. (a) Explain the design equations for calculating the thickness of cylindrical and spherical

- shells. How will you select a standard plate for fabricating a shell?

- (10)

- (b) Write down the stepwise procedure for the design of an elliptical head.

- (10)

- (c) Name the different stresses acting on a tall vessel which is to be installed in a seismic zone.

- (10)

- (d) Mention any five industrially important alloys of Nickel and Copper. Give their approximate

- composition and application.

- (10)

- 8. (a) With a neat diagram, describe the radioactive vacuum gauge method for measuring pressrue

- in a process industry.

- (10)

- (b) Sketch the following function

- and obtain the Laplace transform.

- (10)

- (c) (i) Explain in practical terms, how one tunes a feedback controller for an existing process

- in a chemical plant.

- (5)

- (ii) Define phase margin and gain margin, and show how you can compute them from

- Bode Plot.

- (5)

- (d) Develop a transfer function between the pressure drop and the manometer reading h for a

- mercury manometer. List the assumptions made.

- (10)

- PAPER - II

- SECTION A

- 1. Answer any four of the following (in 150 words, maximum)

- (a) Discuss about proximate analysis of coal. (10)

- (b) Discuss about Mollier diagrams. (10)

- (c) An aqueous solution contains 40% Na2CO3 by weight. Express the composition in mole per

- cent. (10)

- (d) A natural gas has the following composition, in volumetric per cent:

- Calculate

- (i) Composition in mole per cent.

- (ii) Composition in weight per cent. (10)

- (e) Discuss the mathematical statement of second law of thermodynamics. (10)

- 2. (a) Mono-chloroacetic acid (MCA) is manufactured in a semi batch reactor by the action of

- glacial acetic acid with chlorine gas at 373 K in the presence of PC13 catalyst. MCA thus

- formed will further react with chlorine to form dichloroacetic acid (DCA). To prevent the

- formation of DCA, excess acetic acid is used. A small-scale unit, producing 5000 kg/d MCA,

- requires 4536 kg/d of chlorine gas. Also, 263 kg/d of DCA is separated in the crystallizer to

- get almost pure MCA product. Find the % conversion, % yield of MCA and selectivity. (25)

- (b) Discuss about the heat of reaction. (15)

- 3. (a) For the reaction A → R, K = 0.02 min. It is desired to produce 4752 g moles of R per 10

- hours a day and 99% of A entering the reactor is to be converted in a batch reactor. To charge

- the reactor and heat it to reaction temperature requires 0.26 hours. To discharge the reactor

- and to prepare it for the next run takes 0.9 hours. Calculate the volume of the reactor

- required. Pure A with molar density of 8 g mole per liter is charged to the reactor. (25)

- (b) Discuss about phase rule. (15)

- 4. (a) Discuss ideal solution and its properties. (20)

- (b) Derive an expression for design equation for an ideal batch reactor. (20)

- SECTION B

- 5. Write short notes on any four of the following (in 150 words, maximum)

- (a) PVC (10)

- (b) Principles of piping layout (10)

- (c) Coal chemicals (10)

- (d) Alternative investments (10)

- (e) Effects of air pollution (10)

- 6. (a) Describe the manufacture of penicillin using fermentation route. (20)

- (b) List the mathematical methods for profitability evaluation and describe any one. (20)

- 7. List various environmental protection laws and give the main features/provisions of any four in brief.

- (40)

- 8. (a) Discuss the estimation of capital investment. (20)

- (b) Discuss petroleum refining briefly. (20)

- IFS Chemical Engineering 2005

- PAPER - I

- SECTION A

- 1. Answer any four of the following

- (4 x 10 = 40)

- (a) What is Bernoulli equation? Determine it for the flow of a compressible fluid in a pipeline.

- (b) Discuss, with the help of a sketch, the principle of steam jet ejector for creating vacuum.

- (c) Explain the mechanism of heat transfer in nucleate pool boiling of liquids.

- (d) Discuss penetration theory of mass transfer. How does it differ from surface renewal theory?

- (e) Describe the construction and working of a Swenson-Walker crystalliser.

- 2. (a) Discuss differential and cumulative methods of screen analysis.

- (10)

- (b) Explain the construction and operation of a plate and frame filter press.

- (10)

- (c) What is area meter? Explain its operation.

- (10)

- (d) Determine an expression for terminal velocity of solid particles settling freely in a fluid

- stream. Modify the above expression for the case of hindered settling.

- (10)

- 3. (a) Differentiate between free and forced convection. Describe the procedure to calculate heat

- transfer coefficient under free convection.

- (10)

- (b) Explain effectiveness of a heat exchanger. Obtain its expression for a parallel flow heat

- exchanger in terms of NTU and capacity ratio.

- (10)

- (c) Discuss the effect of boiling point elevation on the capacity of an evaporator.

- (10)

- (d) Define emissivity of a body. How is it related to absorptivity?

- (10)

- 4. (a) Derive Rayleigh equation for differential distillation of a binary system.

- (10)

- (b) Discuss pressure drop characteristics of packed columns and explain the significance of

- flooding point.

- (10)

- (c) Explain the mechanism of moisture movement during drying of a solid.

- (10)

- (d) Briefly explain the construction and operation of a packed extraction tower.

- (10)

- SECTION B

- 5. Answer any four of the following

- (4 x 10 = 40)

- (a) Explain the concept of cross flow micro filtration along with its advantages over conventional

- filtration.

- (b) Name various types of heads commonly used in cylindrical vessels. Draw their sketches and

- list areas of applications.

- (c) Mention stepwise procedure for the design of a torispherical head for a cylindrical vessel.

- (d) What is final value theorem? How is it used to determine response of a system? Explain it

- with suitable examples.

- (e) Discuss cascade control and its application in process industries.

- 6. (a) Describe procedure of immersion precipitation process for the production of membranes.

- (10)

- (b) Explain, with the aid of a sketch, the working of an electro dialysis.

- (10)

- (c) What is pervaporation? Explain it with the help of a neat sketch and also indicate areas of its

- application.

- (10)

- (d) Discuss ion-exchange separation process for the demineralization of water.

- (10)

- 7. An autoclave of 7.9 m3 volume capacity has ellipsoidal heads (major to minor axis ratio = 2:1) at its

- top and bottom ends. The autoclave is subjected to a maximum pressure of 8 MN/m2. The allowable

- stress of the material (carbon steel) is 1040 MN/m2. Weld joint efficiency is 85 per cent.

- Determine optimum dimensions of the autoclave if the following cost ratios are applicable:

- (a) The cost of fabricated shell is estimated to vary inversely with D”4 where D is the inside

- diameter of the autoclave.

- (b) The fabricated cost of top and bottom heads per unit weight is 1.5 times the fabricated cost of

- shell per unit weight. A 12 mm plate is available in store. Check if it can be used for the

- fabrication of shell of the autoclave. Alternatively, suggest the suitable thickness of plate

- required for the fabrication of shell of the autoclave.

- (40)

- 8. (a) What are phase margin and gain margin? Discuss their utility in control tunnings.

- (10)

- (b) Discuss procedure for plotting of open loop transfer function by root locus method.

- (10)

- (c) How do you measure liquid level if the liquid contains suspended solid particles? Suggest

- measuring instrument along with the principle of working.

- (10)

- (d) Explain elements of a computer control system.

- (10)

- PAPER - II

- SECTION A

- 1. Answer any four of the following:

- (a) Find the first-order rate constant for the disappearance of A in the gaseous reaction 2A → R,

- holding the pressure constant, the volume of the reaction mixture starting with 80 per cent A

- decreases by 20 per cent in 3 mm.

- (10)

- (b) A solution contains 50 per cent benzene, 30 per cent toluene and 20% xylene by weight at

- 100° C. The vapours are in contact with the solutions. Calculate total pressure and mole per

- cent composition of liquid and vapour:

- (10)

- (c) Show that the internal energy of an ideal gas is a function of temperature only.

- (10)

- (d) For adiabatic batch reactor, derive an expression to determine the time required to achieve a

- desired conversion.

- (10)

- (e) Define Bubble point and Dew point. Explain in detail how to calculate bubble point or dew

- point of given binary mixture.

- (10)

- 2. The elementary reaction. A + B → R + S is effected in a set up consisting of a mixed reactor into

- which two reactant solutions are introduced followed by a PFR. A large enough of excess of B is

- used so that the reaction is first order w.r.t. A. It is proposed to reverse the order of two units. How

- does this change affect the conversion?

- (40)

- 3. A high purity limestone is burnt in a lime kiln which is fired externally with a coal containing 75.5

- per cent C, 5.5 per cent H, 1.6 per cent N, 1.1 per cent S, 7.6 per cent and rest ash. The stack gas

- analysis is 20.2% CO2, 7.1% O2 and rest N2,

- Calculate-

- (a) kg limestone burnt/kg of coal;

- (b) excess air used for combustion.

- (40)

- 4. One mole of an ideal gas initially at P1 and T1 is compressed reversibly and adiabatically till the

- pressure is P2 and then it is cooled at constant volume to the initial pressure, finally the gas is

- restored to the initial state through an isobaric process. Calculate the work done by the process.

- (40)

- SECTION – B

- 5. Answer any four of the following:

- (a) Define depreciation. What are the different methods for determining depriciation? Explain

- the method, sum of the years digit method.

- (10)

- (b) Explain briefly the factors to be considered for the location of a chemical plant.

- (10)

- (c) Describe the manufacture of caustic soda by either diaphragm or mercury cell process.

- Discuss the pollution aspects of this industry.

- (10)

- (d) Explain briefly the pollutants emitted from a tanning industry.

- (10)

- (e) Explain the cracking and polymerisation of petroleum fractions.

- (10)

- 6. (a) Explain the break-even analysis and its equation.

- (10)

- (b) A Project can produce 12000 units per year at 100 per cent capacity. The variable cost per

- unit is Rs. 3 at 100 per cent capacity. Fixed costs are Rs. 10,000 per year. Find the break-even

- point if the selling price is Rs. 5/ unit. Now the manufacturer finds that he can sell only 80%

- at Rs 5/ unit. How much should he charge for additional units if he brings production upto

- 100 per cent capacity and increases profits after taxes by an additional amount of Rs. 1,000?

- Use tax rate for the above problem 52 per cent. .

- (30)

- 7. (a) Describe the process with neat flow sheet of the manufacture of ethyl alcohol.

- (30)

- (b) Discuss the applications of following products:

- (10)

- (i) Enamel paint

- (ii) Rancidity.

- 8. (a) Discuss the methods possible for preventing various industrial hazards.

- (20)

- (b) Explain primary treatment method of waste-water treatment.

- (20)

- IFS 2004 CHEMICAL ENGINEERING

- PAPER - I

- SECTION A

- 1. Answer any four of the following

- (10 x 4 = 40)

- (a) (i) What is time dependent flow ? With the help of this phenomenon, explain the

- meaning of Thixotropic liquid and Rheopeptic substances.

- (ii) Write the power law equation for dilatant and pseudo plastic fluids, and explain the

- terms ‘flow consistency index’ and ‘flow behaviour index’.

- (b) Derive an equation for steady conduction in the radial direction of a hollow cylinder. From

- the equation, explain the term log-mean radius. Under what condition, can the log-mean

- radius be replaced by arithmetic mean radius?

- (c) In a double pipe heat exchanger, a hot fluid is cooled from 150 to 100°C by using cold water,

- which is heated from 30 to 50°C. Derive a suitable expression to prove that there is a distinct

- advantage of using the exchanger in counter-current flow over that of using it in parallel flow.

- (d) Explain how thermal condition of feed influences the liquid or vapour flow rate around the

- feed plate of a rectification column. What common factor can be used for characterizing five

- different feed conditions? From the value of this common factor, explain the meaning of

- horizontal and vertical feed lines.

- (e) Explain the phenomenon of primary nucleation in the process of crystallization, and illustrate

- the difference between homogeneous and heterogeneous nucleation.

- 2. (a) For an incompressible fluid, write the Bernoulli equation for fractionizes flow. What

- correction would you apply for incorporating fluid friction? Explain the difference between

- skin friction and form friction.

- (10)

- (b) What is terminal velocity? Write the equation for terminal velocity for the flow of a single

- particle through a fluid under the action of

- (i) gravity and

- (ii) centrifugal force.

- How is the equation for terminal velocity under free settling condition modified for hindered

- settling conditions?

- (10)

- (c) What is a ball mill? Derive an equation for the critical speed of a ball mill.

- (10)

- (d) What is a venture meter? Describe the principle of working of a venture meter and write an

- expression for the venture coefficient.

- (10)

- 3. (a) An air water system has a dry bulb temperature of 65.5°C and a wet bulb temperature of

- 54.5°C.

- (i) If the air is cooled to 37.8°C, what per cent of the moisture will condense?

- (ii) If the air is again heated to 65.5°C, how much heat will be required per kg of dry air?

- Specific heat of air = 0.23 kcal/ (kg 0C).

- (10)

- (b) A spherical particle of ethanol of 10 mm diameter is evaporating in an air stream at 26°C.

- The thickness of the stagnant air surrounding the particle is 1.5 mm. Vapour pressure of

- ethanol at 26°C is 60 mm Hg and the mole fraction of ethanol in the main stream of the air is

- 0.005. The diffusion coefficient of ethanol in air is 1.04 × 10-5 m2/sec and the value of R is

- 82.06 × 10-3 m3 atm/kg mole.

- Determine,

- (i) The mole fraction of ethanol at different positions across the stagnant layer.

- (ii) Rate of evaporation of the particle per second.

- (10)

- (c) What are different types of packing that are used in an absorption tower? Explain a simple

- procedure for calculating the height of a packed tower for absorption of a single component

- from a gas mixture.

- (10)

- (d) What is critical moisture content? Describe the method for drying of a wet solid before it

- reaches the critical moisture content.

- (10)

- 4. (a) Develop a relation between heat transfer and fluid friction by using the Reynolds’s Analogy.

- (10)

- (b) (i) What is a radiation shield? How does it help in reducing radiant heat loss?

- (ii) A double walled spherical container is used for storing liquid oxygen. The diameter of

- the inner and outer walls of the container is 30 cm and 36 cm, respectively. The

- surfaces of both the spheres are plated, so that their emissivity is 0.05. Determine the

- rate at which oxygen will evaporate at -183°C, when the outer wall of the sphere is

- maintained at 20°C. Latent heat of evaporation of oxygen is 57.2 kcal/kg.

- (5 + 5)

- (c) What are drop wise and film wise condensation? Mention the conditions that favour drop

- wise condensation. Describe how the heat transfer coefficient in a vertical condenser is

- influenced by the height of the condenser.

- (10)

- (d) What is effectiveness of a heat exchanger? Derive a relation between the effectiveness and

- number of transfer units for a countercurrent heat exchanger.

- (10)

- SECTION B

- 5. Answer any four of the following

- (10 × 4 = 40)

- (a) What are the relative advantages and disadvantages of the proportional, integral and

- derivative control actions? What are their characteristic effects on the closed loop response

- for a process?

- (b) Explain briefly the essential elements of a computer controlled process system.

- (c) What is reverse osmosis? How can it be used for the separation of a solute from an aqueous

- solution?

- (d) What is molecular distillation? With a neat sketch describe the working of a molecular

- distillation unit.

- (e) Discuss how wind load is taken care of in the design of a tall vertical vessel.

- 6. (a) Illustrate the principle of separation of gases by using non-porous membrane.

- (10)

- 3 of 5

- (b) Discuss about electrolytic regeneration of mixed bed ion exchange resins.

- (10)

- (c) Explain the process of diffusion dialysis and mention its applications.

- (10)

- (d) What are absorption and extraction type separation processes? Illustrate them with examples.

- (10)

- 7. (a) Discuss how to find the frequency response of a proportional + integral controller,

- (10)

- (b) Explain how a strain gauge is used for pressure measurement.

- (10)

- (c) Illustrate the process of feed forward control.

- (10)

- (d) Write a relationship that will give the molar or specific enthalpy of a multi-component liquid

- at temperature T, with known composition for N components. What are the principal control

- considerations that affect the scope of mathematical modeling of a chemical process?

- (10)

- 8. (a) Illustrate the method for the design of a conical head of a cylindrical pressure vessel.

- (10)

- (b) Discuss about major loads and subsidiary loads that must be considered for the design of a

- pressure vessel.

- (10)

- (c) What are skirt supports? Discuss the method for calculating the thickness of a skirt support.

- (10)

- (d) Discuss the method for calculating the thickness of the shell of a thin wall pressure vessel.

- (10)

- PAPER - II

- SECTION A

- 1. Answer any four of the following

- (a) Explain the graphical procedure for finding the concentrations in a series of mixed reactor.

- (10)

- (b) A solution of FeCl3 in water contains 64.1% FeC13 by weight. Calculate the yield of

- FeC13.6H2O crystals obtained from 1000 kgs solution, if cooled to 27°C. Solubility at 27°C =

- 68.3% by weight.

- (10)

- (c) Derive the following relation from the fundamentals.

- (10)

- (d) Explain the procedure to calculate the bubble pressure 8 dew pressure for a given ideal

- Binary mixture.

- (10)

- (e) Derive the performance equation for a given 1st order Ideal CSTR operating at steady-state

- conditions.

- (10)

- 2. A Producer gas with composition by volume 27.3% CO, 5.4% CO2, 0.6% O2, 66.7% N2 is burnt

- with 20% excess air. If the combustion is 98% complete, calculate the composition, by volume, of the flue gas.

- (40)

- 3. Substance A in a liquid reacts to produce R & S as follows:

- A feed (CA0 = 1, CR0 CS0 = 0) enters two mixed reactors in series (τ1 = 2.5 min, τ2 = 5 min).

- Knowing the composition in the final reactor ( ) 1 1 1 CA = 0.4, CR = 0.4, CS = 0.2 , find the compositions leaving the 2nd reactor.

- (40)

- 4. The system (1) n-pentane (2) n-hexane (3) n-heptane obeys Raoults law, for a system at 105°C. In a

- flash drum has overall composition of Z1 = 0.25, and Z2 = 0.45.

- Determine,

- (a) Bubble pressure, Dew pressure and its composition.

- (b) L (Liquid rate), V (Vapour rate) and xiyi when

- The data of Antoine’s constants are: (40)

- SECTION B

- 5. Answer any four of the following

- (a) What are the various steps involved for calculating Total Product Cost?

- (10)

- (b) What is profitability? Discuss different methods for evaluating the profitability.

- (10)

- (c) Briefly explain the concept of straight and mixed fertilizers. List out various sources for the

- manufacture of Ammnonium sulphate.

- (10)

- (d) Describe in detail the activated sludge process.

- (10)

- (e) What is polymerization? Explain different methods of polymerization.

- (10)

- 6. The total capital investment for a chemical plant is Rs. 15, 00,000 and the plant produces 30 laths kg

- of product annually. The selling price of the product is Re. 0.41/kg. Working capital amounts to 15%

- of total capital investment. Raw material costs for the products are Re. 0.045/kg, labour cost Re.

- 0.04/kg. Distribution costs are 5% of the total product cost.

- Estimate:

- (a) Manufacturing cost/kg of product

- (b) Total product cost/year

- (c) Profit/kg of product before taxes.

- (40)

- 7. (a) Describe with neat flow sheet the manufacture of soap by continuous process.

- (20)

- (b) With a neat flow sheet explain the manufacture of LDFE.

- (20)

- 8. (a) Explain the thermal processes for the treatment of solid wastes.

- (20)

- (b) Discuss the various sources of air pollutants and their effect on man and his environment.

- (20)

- IFS 2003 CHEMICAL ENGINEERING

- PAPER-I

- SECTION A

- 1. Answer any four of the following:

- (10 x 4 = 40)

- (a) What is an incompressible fluid? Determine an expression for pressure drop in an

- incompressible fluid flowing in a pipe under the condition of laminar flow.

- (b) Discuss the working of a centrifugal pump along with its operational problems and remedial

- measures.

- (c) Describe the procedure to calculate number of equilibrium stages in the fractionation of a

- binary system by Ponchon-Savant method.

- (d) Explain LMTD correction factor. How is it calculated in the use of multipass shell and tube

- heat exchangers?

- (e) Develop a relation between heat transfer coefficient and friction factor by using colburn

- analogy.

- 2. (a) Discuss flow pattern caused by a turbine type impeller in an agitated tank and suggest ways

- to prevent swirling and vortex formation.

- (10)

- (b) Explain the principle of pilot tube and its application. Also discuss how average velocity can

- be calculated?

- (10)

- (c) Discuss the construction and operation of a notary drum filter. How is rate of filtration related

- to speed of rotation?

- (10)

- (d) What is fluid energy mill ? Explain its principle.

- (10)

- 3. (a) Explain the term critical thickness of insulation. Determine an expression for it for a spherical

- vessel.

- (10)

- (b) Saturated steam of atmospheric pressure condenses on a cold tube of 25 mm diameter and 1

- m long. Determine per cent change in heat transfer rate if the orientation of tube is changed

- from horizontal to vertical.

- (10)

- (c) Compare forward feed double effect evaporator with backward feed double effect feed

- evaporator.

- (10)

- (d) Briefly discuss the procedure to calculate radiant heat load of an oil fired furnace.

- (10)

- 4. (a) Define diffusivity term, Discuss the method of measurement of diffusivity of a binary system

- in a laboratory.

- (10)

- (b) Describe the procedure to calculate minimum liquid-gas ratio for absorbers.

- (20)

- (c) Discuss the working principle of a forced draft cooling tower.

- (10)

- (d) What are constant rate and falling rate periods in the drying of solids? Develop an expression

- for time of drying of a solid under falling rate period.

- (10)

- SECTION B

- 5. Answer any four of the following:

- (10 x 4 = 40)

- (a) Explain the phenomena of reverse osmosis. Discuss the characteristics of membrane required

- for this process.

- (b) Discuss the failure of vessels based on theory of plasticity.

- (c) Name various types of heads commonly used in cylindrical vessels, alongwith their sketches

- and areas of application.

- (d) Determine an expression for frequency response of a transportation lag. Show it graphically

- also.

- (e) Discuss ratio control and its application in process industries. Give two examples.

- 6. (a) What is ultrafiltration? Discuss the dependence of membrane permeation rate on applied

- pressure difference, feed solute concentration and cross flow velocity for ultrafiltration.

- (10)

- (b) Discuss low pressure distillation along with its application in industries.

- (10)

- (c) Explain (i) membrane fouling, and (ii) membrane selectivity in separation processes using

- membrane.

- (10)

- (d) Explain the working principle of dialysis.

- (10)

- 7. (a) Explain Coate’s theoretical analysis for bending stress at the head-shell junction of a vessel,

- (10)

- (b) Discuss the design procedure of skirt supports for vertical vessels.

- (10)

- (c) A process requires a shell to be designed to contain a working pressure of 200 MN/m2. The

- shell temperature is 400°C, Inside diameter and length of the shell have been estimated to be

- 500 mm and 4000 mm, respectively. If carbon steel having an allowable stress of 10.40 x 102

- MN/m2 is used as material of construction of the shell, determine suitable]e thickness of the

- shell.

- (20)

- 8. (a) What are various devices which can be used for absolute pressure measurement ? Discuss any

- one of them which can be used with continuous recording system.

- (10)

- (b) Discuss two important characteristics of a control valve,

- (10)

- (c) Describe the procedure for the design of sampled data controller in a system.

- (10)

- (d) How do you find the frequency response of a PID controller? Explain it,

- PAPER - II

- SECTION A

- 1. Answer any four of the following:

- (a) The normal boiling point of benzene (C6H6) is 80.1°C and its latent heat of vaporization is

- 33.655 kJ/mol. Estimate the vapour pressure of benzene at 45°C.

- (10)

- (b) Prove that two reversible, adiabatic lines cannot intersect.

- (10)

- (c) Prove that

- (10)

- (d) For an ideal batch reactor, derive an expression to determine the time required to achieve a

- desired conversion.

- (10)

- (e) Compare the size of plug flow and mixed flow reactors for first and second order reactions.

- (10)

- 2. Ethane (C2H6) is initially mixed with oxygen to obtain a mixture of 80% ethane by volume. His

- mixture is burned in an engine with 200% excess air. During combustion 80% of ethane burns to

- CO2 10% of ethane burns to CO and 10% of ethane remains unburned. Determine the composition of

- the exhaust gas on dry basis.

- 3. (a) For a particular binary system the activity coefficients are adequately represented by ln γ =

- 0.5 X22

- and ln γ2 = 0.5 X2

- 1 . The saturation pressure of the components at 800C are P1

- S = 900 toor and

- P 2

- S = 600 torr (1 torr = 1 mm Hg column). Is it possible for the system to exhibit azeotropy at 800C?

- If so, what is the azotropic composition?

- (20)

- 4. Consider the reaction

- Equal volumetric flow rates of A and B streams are fed to the reactor and each stream has a

- concentration of 20 mol/liter of reactant. or 90% conversion of A, calculate the concentration of R in

- the product stream or-

- (a) a plug flow reactor;

- (b) a mixed flow reactor.

- (40)

- SECTION B

- 5. Answer any four of the following:

- (a) Name the principal varnish resins and describe, in brief, the method of production of varnish.

- (10)

- (b) Describe the production of penicillin by fermentation.

- (10)

- (c) Explain the terms : Deflagration and Detonation, and distinguish between them.

- (10)

- (d) With the help of neat sketches, describe adiabatic lapse rate and its effect on different types

- of plume behaviour.

- (10)

- (e) Discuss the factors which must be considered in the design of activated sludge system.

- (10)

- 6. Present a neat flow diagram and describe the method of production of sucrose from sugarcane.

- Discuss the major engineering problems associated with the production of sucrose from sugarcane.

- (40)

- 7. (a) A proposed manufacturing plant requires an initial fixed capital investment of Rs. 5 crores

- and a working capital of Rs. 1 crore. It is expected that the annual income will be Rs. 4 crores

- and the annual expenses including depreciation will be Rs. 1.2 crores, before taxes. A

- minimum annual return of 25 per cent before income taxes is required. Income tax amount to

- 40 per cent of all pretax profits. Determine (a) the annual per cent return on the total initial

- investment after income taxes, and (b) the annual per cent return on the average investment

- before income taxes, assuming straight line depreciation and zero salvage value.

- (40)

- 8. (a) Develop mass balance equations for the food and biomass in a typical activated sludge

- system.

- (20)

- (b) Define BOD and COD, and explain the methods of controlling BOD and COD in tannery

- effluents.

- (20)

- IFS 2002 CHEMICAL ENGINEERING

- PAPER - I

- SECTION A

- 1. Answer any four of the following: (10 × 4 = 40)

- (a) What is the physical significance of Bernoulli’s equation? How can this equation be modified

- to account for friction losses?

- (b) What are the laws of size reduction? Explain them, mentioning the ranges of their.

- applicability.

- (c) Discuss the procedure for evaluating mass transfer coefficient using Wetted Wall Column.

- (d) Discuss briefly the diffusion and capillary mechanisms involved in moisture movement in

- drying of solids.

- (e) “By using the analogy suggested by Osborne Reynolds, develop a relation between heat

- transfer coefficient and friction factor.”

- 2. (a) Considering fluid flow through a circular pipe, derive the Hagen-Poiseuille equation.

- (b) Explain the principle of Pitot tube.

- (c) List three industrial filters, specially indicating their applications.

- (d) Discuss the degree of mixing and determination of power of mixing. (10 × 4 = 40)

- 3. (a) Explain “Point efficiency”. Murphree tray efficiency and overall tray efficiency.

- (b) Explain how to calculate minimum liquid—gas ratio for absorber.

- (c) Explain how you would determine number of stages for double solvent extraction.

- (d) Discuss Mier’s super saturation theory. (10 × 4 = 40)

- 4. (a) What are the effects of various operating conditions on overall heat transfer coefficient in an

- evaporator unit?

- (b) What do you understand by geometric shape factor in thermal radiation 7 Determine the

- geometric shape factor for a very small disc of area dA and large parallel di4c of area A

- located above the smaller one.

- (c) Differentiate between equivalent diameters for heat transfer and pressure drop for flow of a

- fluid through the annulus of a double pipe heat exchanger.

- (d) With a neat sketch, describe briefly the scheme used for solar water heating. (10 × 4 = 40)

- SECTION B

- 5. Answer any four of the following: (10 × 4 = 40)

- (a) Discuss absorption and extraction type separation.

- (b) Write the design procedure for flat heads.

- (c) Explain cathodic protection to prevent corrosion.

- (d) How do you find the frequency response of a PI controller? Explain.

- (e) With a neat sketch, describe a suitable device for controlling the process outlet temperature of

- a heat exchanger that uses steam as the heating medium.

- 6. (a) Explain the theory of electrical separations.

- (b) Discuss the electro-dialysis and its applications.

- (c) Explain the parallel leaf-type ultra filtration module.

- (d) Discuss on very low pressure distillation conducted industrially for heat-sensitive substances.

- (10 × 4 = 40)

- 7. (a) Discuss the criteria for shell failure based on theory of elasticity.

- (b) Distinguish between External bolting chair and Centered anchor-bolt chair.

- (c) Name the various types of heads used for closing ends of cylindrical shells, indicating their

- applications.

- (d) Write the stepwise procedure for determining the standard plate thickness for fabricating a

- cylindrical shell operating at internal pressure.

- (10 × 4 = 40)

- 8. (a) Explain how strain gauge is used in pressure measurement.

- (b) Discuss the working of an optical pyrometer.

- (c) Derive the transfer function of an RC circuit. Show that the time constant of an RC circuit is

- the product of R and C.

- (d) Explain the graphical rules for Bode plots.

- (10 × 4 = 40)

- PAPER - II

- SECTION A

- 1. Answer any four of the following:

- (a) What is proximate analysis of Coal? How do you classify Coals? Give examples. (10)

- (b) Discuss the Second Law of Thermodynamics and its applications. (10)

- (c) What are the ideal reactors? Emphasize the basic assumptions. Write the performance

- equations for each of these reactors (No need for derivation). (10)

- (d) Discuss the concepts of Effective Diffusivity, Effective thermal conductivity and

- Effectiveness factor. (10)

- (e) Discuss the concepts of fugacity, activity and chemical potential. (10)

- 2. A natural gas having a composition of CH4: 80% (by vol), C2H6 : 8.0, C3H8 : 4.5, C4H10: 2.0,

- C5H12:1.0, CO2 :2.5 and N2: 2.0 is burnt with excess air in a burner. The burner gases contain (on

- dry-basis) CO2 : 10.3, N2 : 85.9 and O2 : 3.8. Calculate the per cent excess air supplied and also the

- composition of the burner gases on wet basis. (40)

- 3. Calculate the equilibrium constant for the vapour-phase dehydration of ethanol to ethylene at 350°C.

- The reaction is :

- Data:

- Values of the constants in the heat capacity equation:

- (40)

- 4. What is the importance of pore-diffusion in gas-solid (porous catalyst) reactions? Derive an

- expression for the effectiveness factor for I order kinetics and single cylindrical pore. (40)

- SECTION B

- 5. Answer any four of the following:

- (a) What are the various raw materials! Feed stocks used to produce hydrogen in the manufacture

- of ammonia?

- (10)

- (b) What are the various routes to produce VCM? Discuss their relevance to India.

- (10)

- (c) How the industrial solid wastes are disposed?

- (10)

- (d) What is the Environment (Protection) Act 1986? Discuss the provisions under it.

- (10)

- (e) What are the various factors taken into consideration in laying out of a chemical plant?

- (10)

- 6. (a) What are the by-products/co-products of sugar manufacture and their uses?

- (10)

- (b) Sugar plant is an example of total energy recovery concept. Elaborate.

- (10)

- (c) Describe with a neat flow-sheet a modern sugar production plant.

- (20)

- 7. (a) What are the major industrial liquid pollutants (in the effluents) ?

- (10)

- (b) List the techniques used for removal of dissolved organic solids.

- (10)

- (c) Discuss the principle and operation of a Tricklebed filter (with a neat sketch) in liquid

- effluent treatment.

- (20)

- 8. (a) What is Project Scheduling?

- (10)

- (b) What is the difference between PERT and CPM?

- (10)

- (c) Draw up a typical PERT/CPM Charts for setting up a medium scale chemical plant.

- (20)

- IFS 2001 CHEMICAL ENGINEERING

- PAPER-I

- SECTION A

- 1. Answer any four of the following

- (10 x 4 = 40)

- (a) Explain free settling and hindered settling. What is the functional relationship between

- terminal velocity and particle size?

- (b) What are enthalpy-concentration diagrams? How do you construct it?

- (c) What do you understand by primary nucleation and secondary nucleation during

- crystallization?

- (d) Explain critical thickness of insulation. How do you find it for cylindrical shapes? Assume

- the thermal conductivity of the insulation as k. Heat transfer coefficient is h.

- (e) Define the following:

- (i) Mixing index for granular solids

- (ii) Sphericity

- (iii) Relative volatility

- (iv) Emissivity

- 2. Answer the following:

- (10 × 4 = 40)

- (a) Explain briefly about random packings used in packed towers. What are the characteristics

- considered during their selection?

- (b) Differentiate between skin drag and form drag. Where do they exist and under what

- conditions?

- (c) Describe the differential and cumulative screen analyses.

- (d) Compare the venturi meter and orifice meter.

- 3. (a) Derive the Fenske equation to calculate the minimum number of plates in distillation.

- (20)

- (b) Explain a mixer-settler extraction system.

- (10)

- (c) Discuss the experimental measurement of diffusivity in a binary system.

- (10)

- 4. (a) Estimate the heat transfer coefficient using available empirical correlation for the following

- case:

- A lubricating oil is flowing through a long smooth tube of 25 mm inside diameter. The inlet

- and outlet oil temperatures are 21°C and 55°C. The average wall temperature is assumed to be 65°C.

- The Reynolds number for the prevailing flow conditions has been found to be 10020.

- Data : At the average temperature, oil has Viscosity = 31 cp

- Specific heat = 1.88 kJ/kg K

- Specific gravity = 0.91

- Thermal conductivity = 0.123 W/m K

- Viscosity at 65°C = 12 cp

- Any missing data may be suitably assumed with justification.

- (20)

- (b) Discuss the method of estimation of the radiant heat duty of an oil fired furnace.

- (10)

- (c) Describe the different methods of feeding a multiple effect evaporator.

- (10)

- SECTION B

- 5. Answer any four of the following: (10 × 4 = 40)

- (a) Explain about ultra filtration.

- (b) Write briefly on the dielectric method of level measurement.

- (c) Write the stepwise procedure for the design of a 2: 1 elliptical head.

- (d) Describe the flapper-nozzle assembly used in process control.

- (e) Describe the procedure for the design of sampled data controllers.

- 6. (a) For membrane processes, explain about (15)

- (i) Dependence of permeability on temperature and its prediction

- (ii) Membrane selectivity

- (b) Explain about the following: (15)

- (i) Commercial dialyzers

- (ii) Membranes for dialysis

- (c) What is Pervaporation? Elaborate. (10)

- 7. A flat-bottomed vertical cylindrical tank having a conical roof is to be designed for storing a

- petroleum product having a specific gravity of 0.8. The capacity of the storage tank should be 250

- m3. Assume height to diameter ratio as unity. The width of the plate to be used for each course is 1.8

- m. The allowable stress value of the material of construction is 14 kg/mm2. Assume a weld joint

- efficiency factor of 0.85. Determine the thickness required for each course of the cylindrical shell of

- the storage tank. (40)

- 8. Answer the following: (10 × 4 = 40)

- (a) Describe the electromagnetic flow meter. What are their advantages and limitations?

- (b) Find the frequency response (AR and phase angle) of a proportional + integral controller.

- Give a sketch of the Bode diagram of the same.

- (c) Write briefly on Feed forward control.

- (d) What axe the elements of a computer control system? Explain briefly

- PAPER-II

- SECTION A

- 1. Answer any four of the following

- (a) Distinguish between adiabatic flame temperature and actual flame temperature.

- (10)

- (b) What is Joule-Thomson coefficient and what is inversion point? Sketch the isenthalpic curves

- for a gas and show the inversion curve and explain the significance of Joule-Thomson

- coefficient.

- (10)

- (c) Define effectiveness factor. On what parameters does the effectiveness parameter of a first

- order reaction depend? What is meant by the regime of strong pore resistance?

- (10)

- (d) What is an azeotrope? Draw the P-X-Y and T-X-Y diagrams for maximum boiling and for

- minimum boiling azeotropes. What kind of azeotrope a system is likely to form if it exhibits

- large positive deviations from Raoults law?

- (10)

- (e) What are performance equations of different reactors? Derive the performance equation for a

- plug flow reactor operating at steady state conditions? How would you graphically represent

- these performance equations?

- (10)

- 2. A hydrocarbon fuel of unknown composition is burned with air and the volumetric composition of

- the resulting combustion products on dry basis is CO2 = 7.26% CO = 2.42%, O2 = 7.5% and N2 =

- 82.82%, Determine the composition of the fuel on mass basis and the per cent excess air used for

- combustion.

- (40)

- 3. Industrial grade methanol is produced according to the reaction

- CO(g) + 2H2 (g) → 4 H3 OH(g).

- Assuming that the reaction mixture attains a state of equilibrium. Estimate the degree of conversion

- at 500 K and 5 bar pressure. The enthalpy of vaporization of CH3 OH at 298.15 K is 37.988 kJ/mol.

- The saturation pressure of CH3OH at 298.l5 K is 0.l67l6 bar. The isobaric molar heat capacities of

- the reactants and products are expressed as Cp= a + bT = cT2 where, C is in kJ/mol K and T is in K.

- For CH3 OH (1) it is known that ΔH°f298 = -238.648 kJ/mol and ΔG°f296 = -166.215 kJ/mol.

- (40)

- Data:

- Substance a b × l03 c × 106 ΔH0 f298 (kJ/mol) ΔG°f298 (kJ/mol)

- CO(g) 28.07 4.63 — -110.532 -137.327

- H2(g) 27.01 3.51 — — —

- CH3OH(g) 18.38 101.56 -28.68 — —

- 4. What is autocatalytic reaction? How do you calculate the conversion X of a reactant at a given time

- t? Derive the required expression assuming the initial concentration of reaction A to be CA0. Which

- reactor arrangement would you suggest for obtaining the highest achievable conversion? Explain

- using a rate-concentration curve.

- (40)

- SECTION B

- 5. Answer any four of the following :—

- (a) What is coking? What are the different coking processes employed in the industry? Discuss

- in brief, the basic process variables, which contribute to the quality and yields of Delayed

- Coking products.

- (10)

- (b) What are the major engineering problems faced in fermentation processes? Enumerate the

- different methods for the production of ethyl alcohol. Describe in brief the process of

- production of ethyl alcohol by fermentation.

- (10)

- (c) What are the various factors which should be considered while making a feasibility survey of

- a proposed design project?

- (10)

- (d) What is acid rain and what are the possible causes for it? Elaborate on the adverse effects of

- acid rain.

- (10)

- (e) What is meant by Ozone depletion? Describe the various Ozone destruction cycles in brief.

- How is the presence of Ozone in the stratosphere beneficial?

- (10)

- 6. How would you define cement? List the various types of port land cement along with their

- characteristic properties and composition. Describe in detail, with the help of a neat flow sheet, the

- process of production of Portland cement.

- (40)

- 7. A plant designer has to choose between equipment 1 and equipment 2, the details of which are given

- below:-

- Equipment 1 Equipment 2

- Cost Rs. 500,000 Rs. 200,000

- End of the year

- Maintenance cost — Rs. 10,000

- Salvage value Rs.50,000 —

- Service life 8 Years 4 Years

- If the interest rate is 10% compounded annually, calculate the present worth of investment be made

- for each equipment and choose the equipment which has the lower present worth. (40)

- 8. (a) Describe the different meteorological variables which influence severity of pollution. Define

- lapse rate and temperature inversion. (20)

- (b) Explain the different techniques employed to use forest biomass solid waste as domestic and

- industrial fuel supplement. (20)

- IFS 2000 CHEMICAL ENGINEERING

- PAPER-I

- SECTION A

- 1. Answer any four of the following: (10 x 4 = 40)

- (a) Develop an expression for predicting the minimum fluidisation velocity.

- (b) Explain about the penetration theory of mass transfer.

- (c) With a neat sketch, explain the working principle of a spray drier.

- (d) Discuss the merits and demerits of different feed conditions in a multiple effect evaporator.

- (e) Briefly explain about the effectiveness NTU method for the design of double pipe and shell,

- and tube heat exchangers.

- 2. Briefly explain the working principles of the following equipments : (10 x 4 = 40)

- (a) Ball mill

- (b) Plate-and-frame filter press

- (c) Pneumatic conveyor

- (d) Rotary pump.

- 3. Answer the following:

- (a) Write on equilibrium and differential distillation. (10)

- (b) What do you understand by bound moisture and unbound moisture of a solid material? (10)

- (c) Briefly explain about the design of cooling towers. (10)

- (d) Define the following:

- (i) Absorption factor

- (ii) Theoretical number of plates

- (iii) Optimum reflux

- (iv) Plait point

- 4. (a) Develop an equation for the radiative heat transfer between two concentric cylinders with

- diameters d1 and d2 and emissivities e1 and e2 radiating heat at temperatures T1 and T2 K

- respectively.

- (20)

- (b) Draw a neat sketch of the floating head heat exchanger with one pass on shell side and two

- passes on tube side and explain the function of its various components.

- (20)

- SECTION B

- 5. Answer any four of the following: (10 x 4 = 40)

- (a) Explain clearly about ion-exchange separation process.

- (b) What are the materials generally used in process vessel construction? Briefly explain about

- the basic characteristics of (i) carbon steels and (ii) high alloy steels.

- (c) Mention about various types of supports for (i) vertical vessels and (ii) horizontal vessels.

- With neat sketches, explain about saddle support and skirt support.

- (d) Enlist the different types of pyrometers. Describe the working principle of any one type of

- pyrometer.

- (e) Explain the importance of frequency response study in control system. Define corner

- frequency and mention its importance.

- 6. (a) Explain clearly about (i) dialysis, (ii) osmosis and (iii) reverse osmosis. (15)

- (b) With neatsketches, explain the working principles of batch dialysis and continuous dialysis

- and types of equipments used for this purpose. (25)

- 7. Calculate the cylindrical shell thickness, the conical roof thickness and the thickness of bottom plates

- for carbon steel (structural) conical roof storage tank to store 250,000 litres of liquid of density 940

- kg/m3. Slope of the roof-one in five.

- Superimposed load on the roof= 125 kg/m2. Permissible stress in steel = 1450 kg/cm2. Specific

- gravity of steel = 7.7.

- Modulus of elasticity = 2 x 106 kg/cm2

- Assume diameter of the tank equal to its height. Draw a free-hand sketch of the tank indicating the

- essential fittings on it.

- (40)

- 8. (a) How do you measure liquid level if (i) the liquid is corrosive and (ii) the liquid contains

- suspended solid particles? Suggest the instruments to be used in these two cases. Briefly

- describe their principles of functioning.

- (20)

- (b) Write about the following:

- (i) Control valve characteristics (8)

- (ii) Block diagram (5)

- (iii) PID controllers (7)

- PAPER-II

- SECTION A

- 1. Answer any four of the following:

- (a) Define theoretical amount of air and excess air supplied for combustion of coal and sulphur.

- (10)

- (b) Define heat of reaction. How do you calculate heat of reaction at a given temperature and

- pressure?

- (10)

- (c) What are the ideal reactors? Emphasise the assumptions involved in defining the same.

- (10)

- (d) ‘The entropy of the universe is ever increasing’. Discuss.

- (10)

- (e) How do you analyse kinetic data collected in ad experimental batch reactor finally to arrive at

- the rate equation for the given reaction? (10)

- 2. A fuel gas containing CO2: 2.1%, C25H42: 7.0%, CO: 33.8%, H2: 40.6%, C12H44: 11.2% and N2:

- 5.3%

- is burnt with 30% excess air. Calculate the fuel gas (burner outlet gas) analysis on dry basis. Assume

- complete combustion in the burner. (40)

- 3. For the catalytic dehydrogenation of 1-butene (C4H8) to 1,3-butadiene (C4H6)

- C4H8(g) → C4H6(g) + H2(g)

- carried out at 900 K and 1 atm, and with a ratio of 10 mol of steam per mol of butene, determine the

- extent of reaction at equilibrium. Determine the same in absence of the steam too. (40)

- 4. In an experimental catalytic packed- bed reactor, using a fixed feed rate of FA0 = 10kg- mol/hr, for

- the reaction A → R, the following kinetic data were collected

- Find the rate equation.

- (40)

- SECTION B

- 5. Answer any four of the following:

- (a) What are the feed stocks used to produce polystyrene commercially?

- (10)

- (b) What are the major differences in structures and physical properties of low density

- polyethylene (LDPE), high density polyethylene (HDPE) and linear low density polyethylene

- (LLDPE)?

- (10)

- (c) What are the permissible limits of the constituents in the ambient air for residential and

- protected areas in India?

- (10)

- (d) Write short notes on :

- HAZOP/HAZAN studies

- (10)

- (e) What factors you take into consideration in determining a chemical plant layout?

- (10)

- 6. How is paper produced commercially? Describe paper production from the raw materials, with the

- help of a neat flow sheet.

- (40)

- 7. (a) What is ‘greenhouse’ effect? Discuss how various greenhouse gases contribute to the effect.

- What are your suggestions to minimize the ‘greenhouse’ effect?

- (24)

- (b) How do ground level concentration of pollutant emitted from a chimney vary with

- meteorological conditions?

- (16)

- 8. (a) Define fixed capital and working capital. How do you arrive at them?

- (20)

- (b) Discuss briefly the utility of PERT/CPM charts.

- (10)

- (c) Explain the different components in Profit and Loss Account.

- (10)