- QUESTION BANK FOR JE-I(SIGNAL) TO SE(SIGNAL) EXAMINATION (Subjective Questions)

- Q1: With the help of a sketch, define station limits for “B” class stations equipped with

- Two Aspect signalling on single line and double line sections.

- Q2: What are the classifications of stations in Indian Railways?

- Q3: What do you mean by “Read Back Technique” in Railway signalling? Please cite few

- examples of this technique being used in your Railway.

- Q4: What is the difference between “A” Marker and “A” Board in Railway

- signalling?

- Q6: What do you mean by the “S” Marker and where it is being used in Railway signalling?

- Q5: What is the normal aspect of a signal in Automatic signalling territory?

- Q6: What do you mean by Slip siding and Catch siding? Write down cases

- when Slip siding and Catch siding are required to be provided in the Railway system.

- Q7: In Indian Railways, what are the systems of train working adopted?

- Please also mention the name of the most widely used system on Indian Railways.

- Q8: Please prepare the locking table for Lever numbers 1, 3, 4, 6 & 7 of the

- following diagram: -

- Q9: What are the essential requirements of the Absolute Block System?

- Q10: What are the standards of interlocking prescribed in Railway Signalling?

- Q11: What are the different classes of LC gates? Please explain.

- Q12: What do you mean by the term “Route Holding” in Railway signalling? Please explain the various means to achieve the same.

- Q13: What do you mean by “Loose” and “Tight” locking in Railway signalling? Please

- explain with examples.

- Q14: What do you mean by Ballast Resistance? What are the minimum permissible

- values for the ballast resistance in the station yard and in the Block section?

- Q15: What do you mean by the staggering of adjacent track circuit rail polarities and why

- is this required?

- Q16: List the names of various types of traction bonds used in RE area using DC track

- circuits.

- Q17: Write down the adjustment of DC track circuit for failsafe and reliable working.

- Q18: What do you mean by Fouling Protection of Track? Please explain.

- Q19: What is Dead Section in track circuit area and how can it be eliminated?

- Q20: What do you mean by cut section track circuit and/or fed over track circuits?

- Q21: Can an AFTC be used in a track section with steel sleepers? Why?

- Q22: What do you mean by “End Fed” and “Centre Fed” types of AFTCs along-with

- corresponding maximum permissible lengths of track circuits?

- Q23: What are the various types of Bonds being used in AFTCs?

- Q24: Please list the names of the constituent parts of a Electrical Lever Lock and its

- functioning.

- Q25:: What do you mean by Indication Locking and Track Locking and on which levers

- these are being used?

- Q26: Explain the working of Snubbing Circuit in Point machine operation using 3-wire

- control.

- Q27: What is the concept of Cross protection in signalling circuits? Please explain.

- Q28: Explain the working of 4-wire point detection circuit using circuit diagram.

- Q29: What do you mean by super imposed detection of points? As per the latest

- directives of Railway Board, is it suggested to use this arrangement?

- Q30: Write down the general features of IRS Rotary Type point machines with 143 mm

- stroke.

- Q31: Explain the working of Point operation circuit using Siemens Point Contactor unit in

- an electromechanical installation.

- Q32: What do you mean by RHS and LHS turnouts? Can we use the same type of Point

- machine (IRS Rotary type) for both the turnouts? Please explain.

- Q33: Compare different types of conventional signal lamps.

- Q34:: What are the different types of ECRs being used with conventional signal lamps?

- Q35: What are Triple Pole lamps? Describe their merits over Double pole lamps.

- Q36: What do you mean by H-Type, I-Type and L-Type signal transformers along-with

- their uses?3

- Q37: What is Aspect Control and Aspect Control circuits? Draw an aspect control circuit

- for an inner distant signal in Double Distant signalling territory.

- Q38: What is cascading of signal aspects? Explain with suitable circuit diagram.

- Q39: What do you mean by Red Lamp Protection? Expalin with suitable diagrams.

- Q40: What do you mean by Cutting in Arrangement in signalling circuits and reason for

- its use?

- Q41: What do you mean by “Direct Feeding”, “Local Feeding” and “Remote

- Feeding” of signals? Explain.

- Q42: Describe the advantages of LED signal Lamps over conventional signal

- lamps.

- Q43: What are the various PVC signal cables used in Railway signalling in

- SECR? Are they screened one or not?

- Q44: What do you mean by cable meggering? Please explain.

- Q45: What is Electric Signal Reverser? List different types of Electric Signal Reversers

- being used over Indian Railways.

- Q46: What do you mean by EKT? Describe the working of EKT.

- Q47: What do you mean by TS, TSS, FP, SP & SSP? Please explain.

- Q48: What are the effects of RE in signalling?

- Q49: List the names of various types of Block Instruments being used in stations on

- Single/Double line and RE/Non-RE area. What special feature is incorporated in the

- block instruments meant for use in RE area?

- Q50: What do you mean by maximum length of parallelism in Railway Signalling over

- RE area? Explain.

- Q51: Why the repeater relays of track Relays are made Slow To Pick-up and with what

- time delay and why? Explain.

- Q52: What are the reasons behind using various traction bonds in Track Circuits over

- RE area?

- Q53: What arrangements/changes are required to be carried out in the DC

- track circuits being used in RE area w.r.t. Non-RE area?

- Q54: What are the effects of RE in signalling circuits and how are they minimised?

- Q55: List the various types of Route indicators used in Railway Signalling and which

- type is being commonly used in Indian Railways?

- Q56: What are the items which are being used to increase the length of a DC track

- circuit in RE area? What are the maximum permissible length of track circuits in Single

- and Double line section on RE area?

- Q57: What are the maximum permissible distances for direct feeding of signals on

- Single and Double line section in RE area? 4

- Q58: What are the various means of suppressing the value of induced voltages at the

- source of 25 kV traction? Explain.

- Q59: Draw a sketch showing the arrangement of power supply, neutral section feeding

- post, sectioning & sub sectioning posts.

- Q60: Draw and explain the State Transition diagram for the working of Axle Counter.

- Q61: What are the different constituent parts of an axle counter?

- Q62: What is Trolley Suppression Track circuit? Why is it necessary in Axle counter?

- Q63: What are approach locking, back locking and dead approach locking in electrical

- signalling? Explain with suitable examples using sketches.

- Q64: What do you mean by Sectional route release in electrical signalling? Explain with

- suitable example using sketches.

- Q65: What is Crank Handle Interlocking in Railway signalling? Explain with suitable

- example using sketch.

- Q66: What is LC Gate Interlocking in Railway signalling? Explain with suitable examples

- using sketches.

- Q67: What is the working principle of 24V battery charger used in Railway signalling?

- Explain with sketch.

- Q68: What do you mean by Filters in electrical rectifier circuits? What is the

- difference between the chargers being used in signalling and telecommunication applications?

- Q69: What is IPS? Describe in detail. How many battery banks are used in IPS

- in Railway signalling?

- Q70: What is electrical transformer? Explain its working in detail.

- Q71: Mention the major advantages of the SMPS over Thyristor controlled

- Power supplies.

- Q72: What is an Inverter and where is this being used in Railway Signalling?

- Q73: Calculate the total load in a typical 4 line way side station with one

- siding line and a common loop in a Double line section. Draw necessary sketches.

- Q74: In a PI installation, which circuit ensures the “One Signal-One Train”

- feature? Please mentioned the name of this circuit and explain with suitable example.

- Q75: Explain the signal control circuit for a home signal with one route with

- suitable sketch and circuits.

- Q76: List the name of activities pertaining to signal department which require

- prior sanction of CRS for their execution in the yards.

- Q77: What is the validity period of CRS sanction? Explain the process for obtaining CRS’s sanction. 5

- Q78: What do you mean by ground connections?

- Q79: What is the role of Drawing Office in S&T department?

- Q80 What are the equipments to be provided with a Trolley/Lorry/Motor Trolley?

- Q81 What is IB signal? Why this signal is provided? Is it being provided on single line

- section? Explain the working of an IB signal.

- Q82 What is the procedure to pass an IBS signal at its ON aspect by the driver of any

- train?

- Q83: What is half notch? Why is this provided and in which equipment/instrument?

- Q84 Draw the Block Clearance circuit for SGE Double line instrument using appropriate

- circuits.

- Q85 What are the principles of Lock and Block working? How these are achieved in

- SGE Double Line Block Instrument?

- Q86 In Siemens signal group relays, the GLSR relay is made slow to release. Why?

- Please explain.

- Q87 What are the items to be checked during scrutiny of Engineering Scale Plan?

- Q88 What items will you check during inspection of a motor operated point?

- Q89 What items will you check during inspection of SSDAC?

- Q90 What item are being noted during day/night footplate?

- Q91 What items are being noted during joint point and crossing inspection with SE/SSE (P-Way)?

- Q92 Draw the ALSR CKT for signal number S- ?

- Q93 How will you councell your staff to prevent short cut method?

- Q94 Why Disconnection Memo should be issued before attending to maintenance of

- certain signal gears & for which activities/works, it should it be issued?

- Q95 (a) What is the propose of using the choke in single rail DC track circuits in RE

- area?

- Q95 (b) What is half notch? Why is this provided and in which equipment/instrument?

- Q96 Draw the complete diagram of Single Rail DC track circuit in RE area including

- value of cack component?

- Q97 What parameters are being checked during inspection of Analog Axle Counter and

- what are their standard values?

- Q98 (a) What are the conditions for granting line clear in double line B class station on

- MACLS territory?

- Q98 (b) What are the conditions for granting line clear in single line B class station on

- Two Aspect CLS territory?

- 99. Draw the dog chart of following: - 2 X 3 W 5 R

- 6

- 100. For the given yard draw the RCC for the following signals: - – Down Home Signal (S-1), Shunt Signal (Sh- 26), Starter Signal (S-4) and Calling

- On signal (Co-2). 78

- FOR SELECTION FROM JE/I/Tele to SE/Tele MW

- 1. What is Radio Patch? Describe it with the help of a Suitable diagram to patch control

- circuit, in case of need.

- 2. Explain phase lock loop system with suitable diagram.

- 3. How rainfall attenuation is measured in MW communication.

- 4. What are the advantages of digital MW communication in comparison with the analog

- MW?

- 5. Write one method of finding the modulation index.

- 6. Discuss different methods for avoiding the fading in MW communication.

- 7. Discuss the salient features of DTL-MUX equipment.

- 8. Draw a block diagram of Transreceiver radio equipment of Digital radio MW.

- 9. What is the testing carried out before taking over the charge of digital MW system?

- 10. Draw the block diagram for Radio Equipment? 7D6 & 7D15

- 11. Explain the function of 7D6 & 7D15 Radio Equipment?

- 12. What is fading? What are the counter measures Railways adopt to present?

- 13. What is DTL MUX? Explain it with diagram?

- 14. What is the meaning of video in & video-out signal-in Radio equipment?

- 15. How many types of measuring instrument are used in MW circuit? Write short notes on each?

- 16. Write the short notes to explain the following?

- · Channel Modem Card.

- · VFT card.

- · Design & set up the MW Communication in between two MW station and two telephone exchanges in Railways?

- CONTROL

- 1. Draw a neat sketch of a six pin socket wiring practiced in RE area and indicates the

- advantage in it.

- 2. Explain in detail DTMF Signaling and its advantages?

- 3. Why two tones are transmitted at a time in DTMF signaling? What are it’s advantages?

- 4. What are the types of traffic train control being used in Railways? Explain each of them?

- 5. What is total interruption of control? What is the work procedure at stations in such

- situations?

- RE & 6 Quad CABLE

- 1. What is meant by Loading in RE Cable?

- 2. What is meant by BON? Why and where it is used in RE Cable?

- 3. What is meant by Half Loading Section?

- 4. What is meant by Screening Factor in an underground telecommunications cable?

- 5. Write down types of Telecom Cable used in Railway for different communication system.9

- 6. Discuss benefits of replacement of O/H alignment by 6Q cable.

- 7. Write jointing procedure of 50 pair PIJF cable.

- EXCHANGE

- 1. What is meant by Pulse Code modulation?

- 2. Give the working principle of Digital Exchange & its features.

- 3. Describe the properties of earthing used in Telecom with specific values used for

- resistances, wire diameter, in each case.

- 4. Explain the working of SPC Exchange in brief with block diagram.

- 5. Describe PCM-TDM with appropriate diagram where it is used.

- 6. What are the functions of Line cards?

- 7. Explain Call processing in Electronic Exchange.

- 8. Write different types of tones with frequency and ‘ON’ and ‘OFF’ time duration for each

- case.

- 9. What is “hunt group”? Explain the requirement of hunt groups.

- 10. What is DID and DOD. Explain advantages of these.

- 11. Write down the maintenance & protection of electronics exchange? How it is better than strongers?

- 12. Advantage of Digital Exchange over analog Exchange?

- 13. Write 10 of features of electronic exchange. Which you would like to have in your

- exchange?

- 14. Draw the basic structure of ISDN Exchange? What are the advantages & procedure’s of it’s working.

- 15. Explain the structure of 2mbps bit stream with frame & multiframe?

- 16. Explain Bit-interleaving and Byte-interleaving?

- 17. Write down procedure to augmentation of electronic exchange up to 5000 Lines along

- with justification. Prepare the brief estimate also.

- 18. How we can add two exchanges with 2MB connectivity? What will be its advantages.

- 19. Discuss and explain difference between IP exchange and SPC exchange.

- 20. Discussed feature of ISDN with block diagram?

- OFC

- 1. Explain the function of an OLTE in Optic-Fiber-System?

- 2. What are the advantages of Optic fiber cable over the conventional cable system?

- 3. How STM is more useful in SDH system over PDH system.

- 4. How monomode fiber is advantageous for communication network rather than

- multichrome.

- 5. How dispersion is reduced in graded index fiber.

- 6. Discuss the merits & demerits of SDH?

- 7. What may be the reasons of alarm in OFC communication system?

- 8. Discuss the difference between PDH & SDH system? Also advantages of SDH over PDH

- system.

- 9. Explain different network Topologies & basic, requirement of topologies?10

- 10. Write down in what ways transmission & reception occurs in OFC. How

- OFC can be optimum utilized in Indian Railways. How we can expand uses of OFC in

- Commercial ways.

- 11. What are test conducted in OFC?

- 12. How we can patch from OFC to conventional cable?

- 13. What is RDSO specification of OFC Cable.

- 14. Detail the works of Railtel? How this body is advantageous for Railways.

- RAILNET

- 1. What is LAN? What is the difference between internet & intranet, and which can be

- categorized under LAN?

- 2. What do you meaning of LAN, WAN, MAN & VLAN?

- 3. What is the function of Router?

- 4. How networking in our Railway has been done? Discuss its benefits & procedures for

- Internet Connectivity.

- 5. Discuss different network topologies? Give explanation diagram for each?

- 6. How to connect 10 nodes (PC) in network. Explain with diagrams, detailing equipments

- needed.

- 7. Explain different network Topologies. Also explain requirement of individual topologies

- with diagrams?

- 8. What is Router? What types of ports are available in Router?

- 9. Explain the functions of switch? Also detail the advantages of switch over HUB?

- 10. What is VPN? How it will be useful in a corporate network?

- 11. What is Layer 3 switch?

- 12. What is synchronous & asynchronous communication?

- 13. What is the use of NMS to monitor and control the problems in Railnet?

- 14. Detail the differences between OSI & ISO layer?

- 15. Differentiate between L-3 switch and Router?

- 16. Difference between Hub and switch with diagrams?

- 17. What do you mean by NMS?

- 18. What is the permissible length between switch to switch via CAT-5/6 cable?

- 19. What is TCP/IP and UDP?

- 20. Why OFC cable is used in LAN? What are it’s advantages?

- 21. Give details of AFRES, MMIS, and PRIME?

- 22. Why VLAN used? What are the advantages of VLAN configuration over the normal configuration?

- 23. Give full form of SNMP. Detail its basics and why it is used?

- 24. What is the meaning of MAC ?

- 25. What is IP address? Also give details of four sections of it.

- 26. Explain OSI seven layer model?

- 27. What is synchronous and asynchronous communication?

- 28. What type of mail server is being used in our Railway?

- 29. What are the minimum number of ports in a Router?

- 30. What theory used in LAN, is it Multicast, Unicast or Broadcast?

- 31. What is RAS and what are it’s functions??

- 32. What is proxy server? Why Proxy server is being used in SEC Railway? 11

- 33. What do you mean by MPLS? Where it is used?

- 34. For testing one Hop distance which command are used?

- 35. How many types of modem are used for network? Specify for permanent & dialup types?

- 36. What do you mean about Firewall?

- 37. Why NetBIOS protocols are used?

- 38. In Railway which type of architecture are used for MIS?

- 39. List the differences between ROM & RAM?

- 40. What Operating and application software are used in client and server side in

- Railways for MIS application?

- 41. What do you mean by UTP CAT5/6 cables?

- 42. What is Hop distance?

- 43. For route checking which command are used in command mode?

- 44. What do you mean about IDS, IPS in security?

- 45. Which technology used for wireless data communication between two PCs. or

- Mobile to PC?

- 46. Bandwidth for STM-1 & E-2 steam?

- 47. MAC address is which Layer address in OSI Layer?

- 48. What are the terms indicated by subnet mask ?

- 49. Give an example of IP address, subnet mask and gateway of router with simple

- example?

- 50. Draw a block diagram of SECR Railnet with all network devices and servers and

- explain each network devices in brief. (Including R & NGP)

- 51. Give short notes on:

- · VOIP

- · URL

- · NIC

- · CSMA/CD

- · Frame

- · Packet

- FOIS

- 1. How do we connect FOIS network between two stations. Explain with block diagram.

- 2. What are the advantages we are getting from FOIS? Explain fourth point of public

- & Railways.

- 3. Explain with block diagram the connectivity of FOIS between Divisions to HQ in SECR.

- PASSENGER AMENETIES

- 1. Discuss how Touch & Feel system works in Railway. What are items included in it?

- 2. Discuss unification of PRS & UTS. What are the advantages of it?

- 3. Discuss how IVRS, video conferencing, Call Centre, Train Charting/Voice Recorder

- works.12

- 4. Draw a block diagram of PRS at a way side station?

- 5. Write down the working principle of Coach Guidance System and Train indication board?

- Interaction with other public agencies

- Give short notes on:

- (i) SACFA

- (ii) PTCC

- (iii) WPC

- Write short notes: -

- a. Wave Guide

- b. Space Diversity

- c. Fade Margins

- d. Frequency Diversity

- e. Railnet

- f. FOIS

- g. OTDR

- h. NTES

- i. IVRS

- j. DTMF Signaling

- k. ISDN

- l. LAN

- m. WAN

- n. FOIS

- o. MODEM

- p. LAN Extender

- q. Router

- r. Voice Data Logger

- s. NMS

- t. PRC

- u. CCB Telephone

- v. Q signaling

- w. Jitter & Wander

- x. PDH

- y. SDH

- z. Leased line

- aa. BRI

- bb. PRI

- cc. TDM

- dd. PCM

- ee. CUG scheme

- Objective type questions 13

- (i) Write full forms of the following

- i. CEPT card

- ii. ISDN

- iii. SMPS

- iv. MTBF

- v. OTDR

- vi. SCADA

- vii. ACSR

- viii. OLTE

- ix. MODEM

- x. NMS

- xi. UTS

- xii. PIJP

- xiii. DTMF

- xiv. BON

- xv. GSM

- xvi. CRIS

- xvii. FOIS

- xviii. SPC

- xix. DTMS

- xx. COIS

- xxi. AMI

- xxii. HDB-3

- xxiii. TDM

- xxiv. RMS

- xxv. STM

- xxvi. LAN

- xxvii. WAN

- xxviii. PCM

- xxix. GRIM

- xxx. CUG

- xxxi. TDM

- xxxii. NMS

- (ii) Write True/False of the following:-

- 1. In OFC Cable Repeater, Index of Core is kept higher than clad. (For Total Internal

- Reflection) (T/F)

- 2. OTDR can be used to measure the Splice loss also in OFC-(T/F)

- 3. Inmarset works via satellite and any access to Private Network is not required- (T/F)

- 4. In (PRS+UTS) unified system UTS counter can issue a Reserved Ticket- (T/F).

- 5. For Radio Patching working of CNL cuts. It is necessary to convert 4W.CKT into 2W

- before patching.- (T/F).

- 6. With MLLN of CNL cuts, it is necessary to convert 4W CKT into 2W before patching-

- (T/F).

- 7. In DTMF dialing always two different frequencies are transmitted (T/F).

- 8. 1 Watt is equal to 30 dbm. (T/F)

- 9. 672 MB are provided in STM-I (T/F)

- 10. OTDR is used in Digital MW. (T/F).

- 11. WAN can be provided in closed premises. (T/F). 14

- 12. In TDM, the transmission of signal is on time sharing basis. (T/F)

- 13. In SPC Exchange, the switching network set up temporary connection between two are more exchanges. (T/F)

- 14. LAN Extender can be used for WAN connectivity (T/F).

- 15. Specification of OFC Cable is IRSTC-55(T/F).

- (iii) Fill in the blanks.

- 1. The duration of a multiframe is ………..

- 2. In a 30 channel system, the sampling time period is………….

- 3. In 30 channel PCM system, a time slot duration is …………. microsecond.

- 4. STM 4 is having -----------2Mb

- 5. In data circuit, Data transfer limit in CAT-6 cable is ---------

- 6. Channel transmission capacity of STM-1 is --------------

- 7. Bit rate of 2Mb with --------------------- PPM tolerance.

- 8. Bit rate of 2nd order mux ----------------- Kbps ---------------- PPM tolerance.

- 9. Duration of frame -------------------

- 10. Duration of Time slot -------------------

- 11. What does EPABX stand for ----------------------------------

- 12. Analog Subscriber loop resistance ------------------------ Ohm.

- 13. Working voltage of electronic exchange ----------------- V ----------------- tolerance.

- 14. Ideal working Temperature of exchange ---------------------

- 15. Dial tone ------------------ Hz modulated with ----------------- Hz frequency.

- 16. Ring back tone ------------------ Hz modulated with ------------------ Hz frequency.

- 17. Busy tone ------------------- Hz modulated with --------------- Hz frequency.

- 18. Switch is a ------------------ device (Broadcasting/Unicasting).

- 19. Switch function on the principle of ------------------ (CSMA/CDMA).

- 20. Insertion loss of Telephone equipment in the case of: -

- · Handset is on the -----/switch/Nil

- · Handset is off the -----/switch/0.2db

- · Press to -------“button pressed/3.0db

- 15

- QUESTION BANK FOR THE SELECTION OF JE-I(SIGNAL) TO SE(SIGNAL) (Objective/True-False and short Questions)

- 1 Operating Current of coil of indicator in DLBI is about:-

- i) 40 – 50 mA ii) 95-100mA

- iii) 17-25 mA iv) 37-42 mA

- 2 Rated current and coil resistance of a polarized relay is:-

- i) 100 mA & 100 Ohm ii) 33 mA & 25 Ohm

- iii) 25 mA & 77 Ohm iv) 125 mA & 77 Ohm

- 3 L-Type of transformers are used:-

- i) Where Signals lamps are directly fed up to 605 Mtr.

- ii) Where LED Signals lamps are used.

- iii) Where Signals lamps are directly fed up to 240 Mtr

- iv) Where Signals lamps are directly fed beyond 1000 Mtr

- 4 Rating of SL-35B triple pole lamps is :-

- i) 12 V/24W,24W ii) 12V/24W,18W

- iii) 24 V/24W,24W iv) 24V/12W,12W

- 5 The No. of front and back contacts in QTA2 relay is:-

- i) 1F/1B ii) 2F/1B

- iii) 1F/2B iv) 2F/2B

- 6 Minimum permissible TSR with concrete sleeper track circuits in RE area is:-

- i) 2.5Ω/KM ii) 0.6 Ω /KM

- iii) 0.25Ω/KM iv) 0.5 Ω /K

- 7 QS3 DC neutral Relays are used in :-

- i) All circuits of non RE section

- ii) Only internal circuits of RE area

- iii) In axle counter in place of self type relay

- iv) Point machine controlling RE area

- 8 QJ1 DC neutral Relays are used in :-

- i) All circuits of non RE section

- ii) For timing control

- iii) In axle counter in place of self type relay

- iv) Point machine controlling RE area

- 9 Lamp proving relays are :-

- 16

- i) Current sensing DC line relays

- ii) Voltage sensing DC line relays

- iii) Current sensing DC track relays

- iv) Voltage sensing DC track relays

- 10 Rated Voltage and Power of an AC LED signal is :-

- i) 230V, 15W ii) 110V, 50W

- iii) 110 V, 15W iv) 230V, 50W

- 11 In DLBI Commutator handle gets locked when it is turned :-

- i) From Line Close to Line Clear

- ii) From Line Clear to Line Closed

- iii) From Line Closed to train on Line

- iv) Line Clear to Train on Line

- 12 ALSR Relay is used for :-

- i) Sequential route release ii) Sectional Route release

- iii) Route Locking iv) Dead approach locking

- 13 EUYNR is: -

- i) Emergency route cancellation relay

- ii) Emergency group signal cancellation Relay

- iii) Emergency route cancellation button relay

- iv) 120 second time delay relay

- 14 Commissioner of Railway Safety (CRS) belongs to the ministry of:-

- i) Railways ii) Civil Aviation

- iii) Surface Transportation iv) Labour

- 15 Refresher course of signal supervisors is required once in:-

- i) 3 Year s ii) 5 Years

- iii) 4 Years iv) 6 Years

- 16 Number of aspects in the Inner Distant signal of an IBS signal in Double Distant signalling territory is:-

- i) 3 ii) 2

- iii) 4 iv) Same as those of its Distant Signal

- 17 Longitudinal bonding is provided by:

- i) Engineering Department

- ii) Electrical department

- 17

- iii) S&T department

- iv) Combined with ii &iii

- 18 Induced voltage in unscreened cable for double line is

- i) 116 V/Km

- ii) 95 V/Km

- iii) 87.5 V/Km.

- iv) 78.5V/Km

- 19 Maximum length of direct feeding for screened and unscreened cables respectively is

- i) 890 m and 980 m

- ii) 345m and 543m

- iii) 600m and 240m

- iv) 640m and 200m

- 20 IRS type high thrust point machine can operate safely with maximum contact test load of…..kg on the throw bar

- i) 670

- ii) 700

- iii) 730

- iv) 760

- 21 Stroke of IRS type point machine is

- i) 143 mm

- ii) 220 mm

- iii) Both i and ii

- iv) Stroke can be adjusted between 110 mm to 220 mm

- 22 No. of throw rod, detection rod and lock rod in IRS type point machine are

- i) 1, 2 and 2

- ii) 2,1 and 2

- iii) 2, 2 and 1

- iv) 2,2 and 2

- 23 Study the following statement about testing of Glued Joints

- A) In dry condition resistance of glued joint shall be less than 25 Mega ohm when meggering voltage of 100 V DC is applied across the joint

- B) In wet condition resistance of glued joint shall not be less than3 Kilo ohm when meggering voltage of 100 V DC is applied across the joint. Which of these statements are true?

- 18

- i) Both A & B

- ii) Only A

- iii) Only B

- iv) None of A & B

- 24 Study the following statement

- A) For track Circuits lengths up to 700 m, Maximum permissible rail resistance is 2 Ohm/Km

- B) For track Circuits lengths more 700 m, Maximum permissible rail resistance is 0.5 Ohm/Km. Which of these statements are true

- i) Both A & B

- ii) Only A

- iii) Only B

- iv) None of A & B

- 25 Study the following

- A) Rail resistance = (Voltage Drop in Rails)/ Average track circuit current

- B) Ballast Resistance = (Average rail Voltage)/Leakage Current Which of these statements are correct?

- i) Both A & B

- ii) Only A

- iii) Only B

- iv) None of A & B

- 26 Study the following statement

- A) For safe working of track Circuits, without a shunt across the track, track relay excitation shall not exceed 250% of its pick up valve.

- B) With normal feed source voltage and minimum permissible ballast resistance of the track, Track relay voltage shall be less than 125 % of its pick up valve.

- Which of these statements are true?

- i) Both A & B

- ii) Only A

- iii) Only B

- iv) None of A & B

- 27 Study the following statement

- A) In DLBI block bell has separate line wire.19

- B) In DLBI Top indicator needle of station in advance is connected to bottom indicator needle of station sending train.

- i) Both A & B are correct

- ii) Only A is correct

- iii) Only B is correct

- iv) None of A & B are incorrect

- 28 Study the following statement

- A) In DLBI, Commutator handle should locked first before the “Train on line” indication appears on the indication when the handle is turned from “Line Clear” to “Train on line” position.

- B) If turning of Commutator is done quickly, a situation may arise when TOL contacts are made but the Commutator is not locked in TOL position.

- i) Both A & B are correct and B is correct explanation of A.

- ii) Both A & B are correct and B is not correct explanation of A

- iii) Only A is correct.

- iv) Only B is correct.

- 29 Study the following statement

- A) A transverse Rail Bond is provided connecting all the noninsulated rails.

- B) This provides continuous return path to traction current.

- i) Both A & B are correct and B is correct explanation of A.

- ii) Both A & B are correct and B is not correct explanation of A

- iii) Only A is correct.

- iv) Only B is correct.

- 30 Study the following statement

- Transverse Bond is provided by ……… department and identification of non-insulated rail is done by……….department.

- i) S&T, Electrical

- ii) S&T, S&T

- iii) Electrical, S&T

- iv) Electrical, Electrical

- 31 Study the following statement

- A) In double cutting arrangement, proving of control contacts is done on either limb of HR coil.

- B) This prevents false energisation of HR from multiple faults.

- i) Both A & B are correct and B is correct explanation of A.

- ii) Both A & B are correct and B is not correct explanation of A

- iii) Only A is correct.

- 20

- iv) Only B is correct.

- 32 EI & PI are provided for stations having no. of routes….. and …… respectively.

- i) 0-25, 25-200

- ii) 70-100, 20-35

- iii) 50-200, 0-50

- iv) 5-100, 100-200

- 33 Track crossings of signalling cables are done at a depth of 1meter below ……….

- i) Ballast Level

- ii) Top Level of Rail

- iii) Bottom Level of Sleeper

- iv) Bottom Level of Rail

- 34.M/s CEL is one of the manufacturers for:

- a. Analog Axle Counters only

- b. Integrated Power Supply

- c. Electronic Interlocking Equipment

- d. Both Analog and Digital Axle Counters

- 35.For Mechanical Signal Items, who does the inspection of materials?

- a. RDSO

- b. RITES

- c. Consignee

- d. None of the above.

- 36.The “Outlying Siding” is referred to the siding for which:

- a. The siding line takes off from running lines and the siding is located out side of that station.

- b. The siding line takes off from running lines and is located in side of that station.

- c. The siding line takes off from running lines and located between block stations.

- d. An additional line is provided for this purpose in any station.

- 37.CRS’s sanction is not required for the following work in the existing yard?

- a. Replacing mechanical operation of a point with motor operation

- b. Inserting a turnout on passenger running line

- c. Shifting of a signal from LHS to RHS.

- 21

- d. Opening of a new line for public

- 38.The following is wrong with respect to track repeater relay (TPR)?

- a. It has 12 Front /4 Back contacts

- b. It is of QNA1 type

- c. It has got AC immunity against induced voltages

- d. It is an example of track relay

- 39.The power supply Track Circuits are taken through which of the following components/modules of the IPS?

- a. Rectifier

- b. DC-DC converter

- c. Inverter

- d. Transformer

- 40.Choose the correct option for Digital Axle Counters presently being used in SEC Railway

- a. It has both “Preparatory” and “Hard” Resetting options

- b. It doesn’t require trolley suppression track circuit

- c. It can detect more than one section

- d. It uses only Two conductors for establishing

- communication between the pair of EJBs

- 41.Block Bell Equipment is used for

- a. Converting AC line voltage coming from the Block instrument at other end of station to required DC voltage in local Block instrument

- b. Converting high AC line voltage coming from the Block instrument at other end of station to a required low AC voltage in local Block instrument

- c. Converting high DC line voltage coming from the Block instrument at other end of station to a required low DC voltage in local Block instrument

- d. Converting DC line voltage coming from the Block instrument at other end of station to required AC voltage in local Block instrument

- 42.What is the approximate maximum distance for Direct Feeding of conventional signal lamps on Double Line RE area?

- a. 180 meters

- 22

- b. 600 meters

- c. 595 meters

- d. 220 meters

- 43.Choose the correct answer from the following

- a. The term “Block Forward” is concerned with the station lying only in a Single Line section

- b. The term “Block Back” is concerned with the station lying only in a Single Line section

- c. The term “Block Back” is concerned with the station lying either in a Single Line section or a Double Line section

- d. The term “Block Back” is concerned with the station lying only in a Double Line section

- 44.The signal overlap for IBS signal is

- a. 120 meters

- b. 180 meters

- c. 200 meters

- d. 400 meters

- 45.The Outer and Warner signals in LQ signalling territory are examples of

- a. A permissive and stop signals respectively

- b. A stop and permissive signals respectively

- c. A subsidiary and main signals respectively

- d. Both are subsidiary signals

- 46.What is the normal aspect of main signals in Automatic Signalling territory?

- a. Yellow

- b. Red

- c. Green

- d. Double Yellow

- 47.As per the latest policy of Railway Board, Level Crossing Gate can be interlocked if the TVU of the LC gate is

- a. Up to 20,000

- b. Below 25,000

- c. Between 15,000 to 20,000

- d. More than 25,000

- 48.Which is correct with respect to VCOR in signaling installations?

- a. This is a special type of relay used in Digital Axle Counter

- b. This is a special type of relay used to control rectifier output of an IPS

- 23

- c. This is a special type of relay used in Electronic Interlocking

- d. This is a special type of relay used in Siemens RRI

- 49.The working of an electrical transformer is based on

- a. Mutual Inductance

- b. Self Inductance

- c. Hall Effect

- d. Self Conductance

- 50 Circuit wiring in PI installations over SECR is being done with

- a. 0.6 mm tinned wire

- b. 1.0 mm tinned wire

- c. 16/0.2 multi strand copper wire

- d. 1.5 Sq. mm copper wire

- 51 Fill in the blanks:-

- i) For Roding, running under the track the minimum distance between top of Roding and bottom of rail should be….

- ii) Minimum length of the lock bar for a broad gauge is……..

- iii) Lock bar should have minimum no. of Clips…….

- iv) The colour of point lever is ……..

- v) Resumption of normal working after overhauling can be done by SSE / Signal if no. of lever are not more than ……..

- vi) All gates within station limit are called …gates.

- vii) A gate qualifies for interlocking if the TVU are more than……..

- viii) Clearance between road surface and LC boom should be between ……to…… Meter.

- ix) Far boom locking to be effective it shall not be possible to lift the boom by …..From close position.

- x) Minimum of. …….. % of spare conductors are to be provided in the main cable up to farthest point zone.

- xi) Station section in a “C’ class station is from ……to ………

- xii) Specification of self regulatory battery charger is S………

- xiii) A concrete sleeper is used in track circuited area if it has a minimum resistance of ….Ohms between insert to insert.

- xiv) Minimum cross section of lead wire for track circuit should be …….mm.

- xv) Excitation level of a track relay will not be less than …….of rated pick up voltage.

- xvi) Plug in type of track relays has to replace after …….years.

- 24

- xvii) The overhauling period of DLBI is ……….years.

- xviii) For obstruction test of a point test piece is kept at a ……… distance from the tow of

- xix) Fuse of correct capacity should be of about ……time the rated current in signaling circuit.

- xx) The minimum implantation distance of a colour light signal from the centre of the track should be ………..

- xxi) The colour of shunt signal button on the panel is ………..

- xxii) Route setting on the panel is indicated by…….

- xxiii) UYR relay is used for ……….

- xxiv) WCR relay is used for………….

- xxv) ALSR is used for ……….

- xxvi) The value of induced voltage in unscreened cable in double line section is ……..ohm/Km.

- xxvii) The AC immunity of a IRS point machine is…….Ohm.

- xxviii)As per SEM the minimum visibility of a distance signal in MACLS territory is……..

- xxix) The Block Section Limit Board is located at minimum …..metre from home signal.

- xxx) Clamp type direct locks are desirable for speed above……… Km/Hour.

- 52. Write full form of :-

- (i) SPAD

- (ii) CCRS

- (iii) RAMS

- (iv) ECTS

- (v) CTC

- (vi) SSDAC

- (vii) MSDAC

- (viii) EKT

- (ix) REB

- (x) TLJB

- (xi) MACLS

- (xii) OLWR

- (xiii) DMTR

- (xiv) MTTR

- (xv) MTBF

- 53. Write true or false:-

- i) Insulated rail shall be kept clear of the ballast by 25mm.

- ii) Only single rail track circuit can be provided in 25 KV Ac area.

- 25

- iii) Rail joint bonds are maintained by S&T department.

- iv) Audio frequency track circuit does not require glued joints.

- v) Diodo token less block instrument cannot be used in RE area.

- vi) 3-Phase point machine is immune to induced voltage and can be used to any length of parallelism.

- vii) Direct feeding of AC LED signals is permitted up to 2 Kms.

- viii) All external circuit requires cross protection arrangement.

- ix) When SM key is out no operation can be done from the panel.

- x) Calling- on signal button is of red colour with white dot.

- xi) Implantation distance is painted in red colour on signal post if distance is more than 2.36 metre.

- xii) Shelf type line relays are overhauled on after every 15 year.

- xiii) CRS approval is required for locating a signal on right side.

- xiv) For solid rodding the minimum unsupported length is 1.8 metres.

- xv) Green notice is issued by traffic department.

- xvi) The type of turn out is specified in Interlocking plan.

- xvii) Calling- on signal detects all the points in routes and overlap.

- xviii) After two months of commissioniong responsibility of maintaining Signalling gears in station lies on open line even if station is not handed over to open line.

- xix) Interlocking of a L. C. gates requires sanction of CRS.

- xx) QSPA1 relay is used as repeater of TR.

- xxi) Authority to proceed means authority to enter in block section.

- xxii) Station limit and station section means same thing.

- xxiii) Class “D” stations are non interlocked.

- xxiv) Double yellow aspect means caution.

- xxv) A depended shunt signal has no ON aspect.

- xxvi) In automatic signaling letter “A” in white on black circular disc

- is provided.

- xxvii) Absolute block system is most prevalent system of block working.

- xxviii)Driver can pass an Automatic signal in red without any authority.

- xxix) Caution board is provided at 400 metre away from speed indicator board.

- xxx) Grease/petroleum jelly can be provided on secondary battery terminals to prevent corrosion.

- xxxi) The LED ECR used in ON aspect can also be used for route aspect.

- xxxii) SL-21 signal lamps have two filaments of equal rating.

- xxxiii)The maximum length of track circuit in RE area using PSC sleeper is 350 metres.

- 26

- xxxiv)Force drop arrangement is provided in EKT.

- xxxv) Calling –on signal is of lunar white colour.

- 27